| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0157 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0146 |
| --- | --- |

| Title: | On Indications for Inter-layer Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes semantics change for max_tid_il_ref_pics_plus1[ i ][ j ] which allows indicating that sub-layer non-reference pictures belonging to highest temporal sub-layer in a layer are not used for inter-layer prediction. All the indications that can be currently signalled using max_tid_il_ref_pics_plus1[ i ][ j ] are maintained. The new indication is added to those existing indications. No syntax change is proposed. Additional specification text changes related to the proposed indication are provided. It is asserted that the proposed indication enables low complexity decoding for multi-loop decoding without introducing any new syntax.

# Introduction

In JCTVC-O1008 and JCT3V-F1004 max_tid_il_ref_pics_plus1[ i ][ j ] values are signalled in video parameter set (VPS) extension. max_tid_il_ref_pics_plus1[ i ][ j ] equal to 0 specifies that within the CVS non-IRAP pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] are not used as reference for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[ j ]. max_tid_il_ref_pics_plus1[ i ][ j ] greater than 0 specifies that within the CVS pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId greater than max_tid_il_ref_pics_plus1[ i ][ j ] – 1 are not used as reference for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[ j ].

In HEVC, SHVC and MV-HEVC sub-layer non-reference picture is a picture that contains samples that cannot be used for inter prediction in the decoding process of subsequent pictures of the same sub-layer in decoding order. Samples of a sub-layer non-reference picture may be used for inter prediction in the decoding process of subsequent pictures of higher sub-layers in decoding order.

This document proposes semantics change for max_tid_il_ref_pics_plus1[ i ][ j ] in VPS extension which allows indicating that sub-layer non-reference pictures belonging to highest temporal sub-layer in a layer are not used for inter-layer prediction. All the indications that can be currently signalled using max_tid_il_ref_pics_plus1[ i ][ j ] are maintained. The new indication is added to those existing indications. No syntax change is proposed. Additional specification text changes related to the proposed indication are provided. This includes: a change to the semantics of num_inter_layer_ref_pics_minus1, update of a bitstream conformance constraint and update of the marking process for sub-layer non-reference pictures not needed for inter-layer prediction is proposed.

It is asserted that one of the motivations for the proposed signalling is to enable low complexity decoding for multi-loop decoding.

- The proposal adds a further indication on top of max_tid_il_ref_pics_plus1[ i ][ j ]  related indication by dividing pictures at the highest temporal sub-layer in a layer into sub-layer reference pictures and sub-layer non-reference pictures and allows indicating if sub-layer non-reference pictures are not used for inter-layer prediction. This indication does not need any new syntax signalling. This indication is useful for the highest temporal sub-layer because a sub-layer non-reference picture contains samples that cannot be used for inter prediction in the decoding process of subsequent pictures of the same sub-layer in decoding order. Thus these pictures could be discarded using proposed indication thus helping reduce the complexity of decoding and motion compensation that needs to be performed for reference layer pictures when decoding a target layer.

- It is helpful in the case when an encoder is not using nuh_temporal_id_plus1 assignment and thus has assigned a value of nuh_temporal_id_plus1 equal to 1 for all the pictures. For example this may be the case for existing HEVC base layer bitstreams created using configuration such as HM common conditions. In this case the proposed indication can still be used to indicate support for low complexity decoding by providing an indication if sub-layer non-reference pictures belonging to TemporalId 0 are not used for inter-layer prediction for the HEVC base layer.

# Proposal

We propose semantics change for max_tid_il_ref_pics_plus1[ i ][ j ] in VPS extension which allows indicating that sub-layer non-reference pictures belonging to maximum temporal sub-layer in a layer are not used for inter-layer prediction. All the indications that can be currently signalled using max_tid_il_ref_pics_plus1[ i ][ j ] are maintained. The new indication is added to those existing indications. No syntax change is proposed.

Additional specification text changes related to the proposed indication are proposed as follows:

A change to the semantics of num_inter_layer_ref_pics_minus1 is proposed based on the new indication related to sub-layer non-reference pictures. Also a bitstream conformance constraint is proposed based on the new indication.

Additionally a change to the marking process for sub-layer non-reference pictures not needed for inter-layer prediction is proposed.

Proposed changes related to syntax, semantics and decoding process for the proposal are highlighted below compared to JCTVC-O1008 [1].

- F.7.3.2.1.1 Video parameter set extension syntax

(no change)

| vps_extension( ) { | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| … |  |
| max_tid_ref_present_flag | u(1) |
| if( max_tid_ref_present_flag ) |  |
| for( i = 0; i < MaxLayersMinus1; i++ ) |  |
| for( j = i + 1; j <= MaxLayersMinus1; j++ ) |  |
| if( direct_dependency_flag[ j ][ i ] ) |  |
| max_tid_il_ref_pics_plus1[ i ][ j ] | u(3) |
| all_ref_layers_active_flag | u(1) |
| … |  |
| } |  |

- F.7.4.3.1.1 Video parameter set extension semantics

…

max_tid_ref_present_flag equal to 1 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ][ j ] is present. max_tid_ref_present_flag equal to 0 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ][ j ] is not present.

max_tid_il_ref_pics_plus1[ i ][ j ] equal to 0 specifies that within the CVS non-IRAP pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] are not used as reference for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[ j ].

max_tid_il_ref_pics_plus1[ i ][ j ] greater than 0 specifies that :

-If max_tid_il_ref_pics_plus1[ i ][ j ] is not equal to sub_layers_vps_max_minus1[ i ]+2, within the CVS pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId greater than max_tid_il_ref_pics_plus1[ i ][ j ] – 1 are not used as reference for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[ j ].

-Otherwise (if max_tid_il_ref_pics_plus1[ i ][ j ] is equal to sub_layers_vps_max_minus1[ i ]+2), within the CVS pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId greater than max_tid_il_ref_pics_plus1[ i ][ j ] – 2 are not used as reference for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[ j ] and sub-layer non-reference pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId equal to max_tid_il_ref_pics_plus1[ i ][ j ] – 2 (equal to sub_layers_vps_max_minus1[ i ]) are not used as reference for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[ j ].

The value of max_tid_il_ref_pics_plus1[ i ][ j ]  shall be in the range of 0 to Min(sub_layers_vps_max_minus1[ i ]+2, 7). When not present, max_tid_il_ref_pics_plus1[ i ][ j ] is inferred to be equal to sub_layers_vps_max_minus1[ i ]+17.

- F.7.4.7.1 General slice segment header semantics

…

inter_layer_pred_enabled_flag equal to 1 specifies that inter-layer prediction may be used in decoding of the current picture. inter_layer_pred_enabled_flag equal to 0 specifies that inter-layer prediction is not used in decoding of the current picture.

num_inter_layer_ref_pics_minus1 plus 1 specifies the number of pictures that may be used in decoding of the current picture for inter-layer prediction. The length of the num_inter_layer_ref_pics_minus1 syntax element is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of num_inter_layer_ref_pics_minus1 shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive.

The variables numRefLayerPics and refLayerPicFlag[ i ] and refLayerPicIdc[ j ] are derived as follows:

for( i = 0, j = 0;  i < NumDirectRefLayers[ nuh_layer_id ]; i++ ) {

refLayerIdx = LayerIdxInVps[ RefLayerId[ nuh_layer_id ][ i ] ]

if( max_tid_il_ref_pics_plus1[ refLayerIdx ][ LayerIdxInVps[ nuh_layer_id ] ] != (sub_layers_vps_max_minus1[ refLayerIdx ] + 2) )

refLayerPicFlag[ i ] = ( sub_layers_vps_max_minus1[ refLayerIdx ] >= TemporalId )  &&

(max_tid_il_ref_pics_plus1[ refLayerIdx ][ LayerIdxInVps[ nuh_layer_id ] ] > TemporalId )

else

refLayerPicFlag[ i ] = ((max_tid_il_ref_pics_plus1[ refLayerIdx ][ LayerIdxInVps[ nuh_layer_id ] ] – 2)  >= TemporalId )

if( refLayerPicFlag[ i ] )

refLayerPicIdc[ j++ ] = i

}

numRefLayerPics = j

The variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id  = =  0  | |  NumDirectRefLayers[ nuh_layer_id ]  = =  0 )

NumActiveRefLayerPics = 0

else if( all_ref_layers_active_flag )

NumActiveRefLayerPics = numRefLayerPics

else if( !inter_layer_pred_enabled_flag )

NumActiveRefLayerPics = 0

else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayers[ nuh_layer_id ]  = = 1 )

NumActiveRefLayerPics = refLayerPicFlag[ 0 ] ? 1 : 0

else

NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1

All slices of a coded picture shall have the same value of NumActiveRefLayerPics.

inter_layer_pred_layer_idc[ i ] specifies the variable, RefPicLayerId[ i ], representing the nuh_layer_id of the i-th picture that may be used by the current picture for inter-layer prediction. The length of the syntax element inter_layer_pred_layer_idc[ i ] is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of inter_layer_pred_layer_idc[ i ] shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive. When not present, the value of inter_layer_pred_layer_idc[ i ] is inferred to be equal to refLayerPicIdc[ i ].

When i is greater than 0, inter_layer_pred_layer_idc[ i ] shall be greater than inter_layer_pred_layer_idc[ i − 1 ].

The variables RefPicLayerId[ i ] for all values of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, are derived as follows:

for( i = 0, j = 0; i < NumActiveRefLayerPics; i++)
	RefPicLayerId[ i ] = RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ]

All slices of a picture shall have the same value of inter_layer_pred_layer_idc[ i ] for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive.

It is a requirement of bitstream conformance that for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, either of the following two conditions shall be true:

- The value of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ][ LayerIdxInVps[ nuh_layer_id ] ] is greater than TemporalId and max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ][ LayerIdxInVps[ nuh_layer_id ] ] is not equal to sub_layers_vps_max_minus1[ i ]+2.

- The value of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ][ LayerIdxInVps[ nuh_layer_id ] ] is equal to sub_layers_vps_max_minus1[ i ]+2 and the picture in the current access unit with nuh_layer_id equal to RefPicLayerId[ i ] is a sub-layer reference picture with TemporalId equal to max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ][ LayerIdxInVps[ nuh_layer_id ] ]-2  or max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ][ LayerIdxInVps[ nuh_layer_id ] ]-2 is greater than TemporalId.

- The values of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ][ LayerIdxInVps[ nuh_layer_id ] ] and TemporalId are both equal to 0 and the picture in the current access unit with nuh_layer_id equal to RefPicLayerId[ i ] is an IRAP picture.

- F.8.1.4 Decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0

PicOutputFlag is set as follows:

–	If LayerInitializedFlag[ nuh_layer_id ] is equal to 0, PicOutputFlag is set equal to 0.

–	Otherwise, if the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

The following applies: 1. 	If discardable_flag is equal to 1, the decoded picture is marked as "unused for reference". 2. Otherwise, the decoded picture is marked as "used for short-term reference".

When TemporalId is equal to HighestTid, the marking process for sub-layer non-reference pictures not needed for inter-layer prediction specified in subclause  F.8.1.4.1 is invoked with latestDecLayerId equal to nuh_layer_id as input.

When FirstPicInLayerDecodedFlag[ nuh_layer_id ] is equal to 0, FirstPicInLayerDecodedFlag[ nuh_layer_id ] is set equal to 1.

- F.8.1.4.1 Marking process for sub-layer non-reference pictures not needed for inter-layer prediction

Input to this process is:

–	a nuh_layer_id value latestDecLayerId

Output of this process is:

–	potentially updated marking as "unused for reference" for some decoded pictures

NOTE – This process marks pictures that are not needed for inter or inter-layer prediction as "unused for reference". When TemporalId is less than HighestTid, the current picture may be used for reference in inter prediction and this process is not invoked.

- The variables numTargetDecLayers, and latestDecIdx are derived as follows:

–	numTargetDecLayers is set equal to the number of entries in TargetDecLayerIdList.

–	latestDecIdx is set equal to the value of i for which TargetDecLayerIdList[ i ] is equal to latestDecLayerId.

- For i in the range of 0 to latestDecIdx, inclusive, the following applies for marking of pictures as "unused for reference":

–	Let currPic be the picture in the current access unit with nuh_layer_id equal to TargetDecLayerIdList[ i ].

–	When currPic is marked as "used for reference" and is a sub-layer non-reference picture, the following applies:

–	The variable currTid is set equal to the value of TemporalId of currPic.

–	The variable remainingInterLayerReferencesFlag is derived as specified in the following:

remainingInterLayerReferencesFlag = 0
		iLidx = LayerIdxInVps[ TargetDecLayerIdList[ i ] ]
			for( j = latestDecIdx + 1; j < numTargetDecLayers; j++ ) {
				jLidx = LayerIdxInVps[ TargetDecLayerIdList[ j ] ]
				if( ( (currTid  <= ( max_tid_il_ref_pics_plus1[ iLidx ][ jLidx ] –1 ) )  && ( ( max_tid_il_ref_pics_plus1[ iLidx ][ jLidx ]   != (sub_layer_vps_max_minus1[ iLidx ]+2)  )  )  ) || ( (currTid  <=  ( max_tid_il_ref_pics_plus1[ iLidx ][ jLidx ] –2 ) ) && (  ( max_tid_il_ref_pics_plus1[ iLidx ][ jLidx ]   == (sub_layer_vps_max_minus1[ iLidx ]+2)  ) ) ) ) 
					for( k = 0; k < NumDirectRefLayers[ TargetDecLayerIdList[ j ] ]; k++ )
						if( TargetDecLayerIdList[ i ]  = =  RefLayerId[ TargetDecLayerIdList[ j ] ][ k ] )
							remainingInterLayerReferencesFlag = 1
			}

- When remainingInterLayerReferenceFlag is equal to 0, currPic is marked as "unused for reference".

# Conclusion

This document proposes semantics change for max_tid_il_ref_pics_plus1[ i ][ j ] which allows indicating that sub-layer non-reference pictures belonging to highest temporal sub-layer in a layer are not used for inter-layer prediction. A change to the semantics of num_inter_layer_ref_pics_minus1 and a bitstream conformance constraint is proposed based on the proposed indication. Additionally a change to the marking process for sub-layer non-reference pictures not needed for inter-layer prediction is proposed.

We propose to adopt the proposals in the draft of SHVC and MV-HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, Y.-K. Wang, “High Efficiency Video Coding (HEVC) Scalable Extension Draft 4,” JCTVC-O1008, Geneva, October 2013.

[2] G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 6,” JCT3V-F1004, Geneva, October 2013.