| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0158 |
| --- | --- |

| Title: | Informal Subjective Video Quality Comparison Between the eBrisk-UHD HEVC and x264 AVC Encoders |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Michael Horowitz Faouzi Kossentini Hassene Tmar 1055 W. Hastings Street, Vancouver, BC, Canada V6E 2E9 | Tel: Email: | [+1 512 436-3290] [michael@ebriskvideo.com] |
| Source: | eBrisk Video, Inc. |  |  |

_____________________________

# Abstract

This paper presents the results of an informal subjective video quality comparison between two production-quality encoders, the eBrisk-UHD HEVC encoder and the H.264 / MPEG-4 AVC encoder known as x264. The encoding configurations are designed to (1) reflect those that are expected to be used in Ultra High Definition (UHD) video streaming applications and (2) enable a fair comparison to the maximum extent feasible. It is noted that while both the eBrisk-UHD and x264 encoders are capable of 10-bit encoding, the experiment results reported in this paper were obtained using 8-bit YCrCb 4:2:0 4K (i.e., 3840  2160 pixels) content. When viewing UHD (4K) content encoded using the eBrisk-UHD and x264 encoders in which the eBrisk-UHD encoder was configured to use a 60% lower average bit rate compared with that of x264 (i.e., the x264 encoder’s average bit rate was 2.5 times that of the eBrisk-UHD encoder), viewers indicated that they either had no preference or preferred the eBrisk-UHD-encoded/decoded video in 69.4% of trials. In addition, in a comparison between the eBrisk-UHD and HM 12 encoders with the two encoders configured to use the same average bit rate, approximately 40% of the viewers either preferred the eBrisk-UHD encoder or had no preference. The selection of which encoding was displayed on which side for the side-by-side viewing was established in a randomized manner, and the subjective viewing sessions were conducted in a double-blind fashion. The results reported in this paper generally confirm that the HEVC design (as represented by the eBrisk-UHD encoder) yields a substantial increase in coding efficiency relative to H.264 / MPEG-4 AVC (as represented by x264), with HEVC achieving approximately 2.5 times the compression capability of H.264 / MPEG-4 AVC for 4K content in the context of video streaming applications.

# Introduction

This paper presents the results of an informal subjective quality comparison between the eBrisk-UHD encoder, which is conformant with Recommendation H.265 Version 1.0 (04/2013) / ISO/IEC 23008-2 Main Profile (MP) [1], and x264 [2], which is conformant with H.264 / MPEG-4 AVC High Profile (HP) [3]. Both the eBrisk-UHD and x264 encoders are capable of 10-bit encoding, however, due to the lack of publically-available 4K 10-bit content, the experiments reported in this paper were performed exclusively using 8-bit YCrCb 4:2:0 4K content (i.e., 3840  2160 pixels). The paper is intended to complement earlier work [4-13], which has focused primarily on lower-resolution content (e.g., 720p and 1080p). The coding efficiency gains reported for HEVC (as represented by the eBrisk-UHD encoder) relative to H.264 / MPEG-4 AVC (as represented by x264) are generally larger compared with gains reported in earlier papers where lower-resolution content had been considered.

The paper is organized as follows. Section 2 contains a description of the test conditions used to collect the subjective video quality assessment results. Section 3 contains subjective assessment results and Section 4 discusses observations associated with the results. Section 5 presents conclusions.

# Test Conditions

In this section, the test conditions, including the encoder configuration and evaluation conditions (e.g., sequence presentation details, viewing equipment, lighting conditions), and tested video sequences are described.

## x264 encoder configuration

The x264 (version core 135 r2345) and eBrisk-UHD encoders were configured for high coding efficiency operation. More specifically, the x264 encoder configuration details are listed below.  Parameters not explicitly listed, assume the x264 encoder default values (see [2]).

The following list contains all explicitly-specified x264 encoder parameter values:

- --input-res <width>x<height> 	: the resolution of the input sequence (e.g., 3840x2160)

- --qp <qp> 			: the quantization parameter

- --fps <fps>			: the native frame rate of the input sequence

- --frames <frame number>		: the number of frames to be encoded

- --output	 <bitstream filename>	: the output bitstream filename

- --keyint <intra period>	: the period at which an intra-coded frame is introduced. The intra period value was set to 48 to match the configuration used by the eBrisk-UHD encoder.

## Subjective evaluation conditions

Twenty-seven (27) volunteer viewing subjects were used for the subjective experiments presented in this paper. Fifteen (15) of the viewers had little or no previous experience in evaluating video sequences. The viewing was conducted in a somewhat-darkened room using a XBR55X900A 55'' UHD Sony Bravia LED monitor. Additional key elements of the subjective evaluation and video presentation methodology are listed below:

- Viewers participated one at a time with each viewer seated in a chair that was positioned approximately 1.5 meters from the monitor and centered.

- For each of the four test sequences listed in Table 1, the eBrisk-UHD and x264 encoded video were cropped in the horizontal dimension and spliced together side-by-side. The relative position of the compared encodings was randomized – sometimes the eBrisk-UHD-encoded/decoded sequence was shown on the left and sometimes it was on the right. The experiments were executed in a double-blind manner.

- The sequences were displayed at their native spatial resolutions; however, the display rate was set to 30 frames per second (fps).

- The video sequences were presented to each viewer between one and three times and the viewers were asked to assess the relative quality of the side-by-side encodings according to a five-degree scale: left better, left slightly better, no preference, right slightly better or right better. The viewers were provided ample time between the presentation of each of the spliced video sequences to record their preferences.

The authors plan to demonstrate the encoded/decoded video sequences used to compare the eBrisk-UHD and x264 encoders at the 16th meeting of the JCT-VC in San Jose, CA.

## Video sequences

The four video sequences described in Table 1 were used in the subjective experiments. The YCrCb 4:2:0 8-bit 4K video sequences have a variety of characteristics typical of what might be encountered in the context of a streaming application, and they were selected from those that are generally available and used for video coding test purposes. All the sequences have a native frame rate of 50 fps. The 4K sequences were displayed at 30 frames per second; the highest frame rate at which the monitor is capable of displaying 4K content. The order of presentation followed that shown in Table 1, from top to bottom.

Table 1. Video sequences used for the subjective quality comparison in order of presentation (top to bottom).

| Sequence snapshot | Sequence name | Resolution | Frame rate (fps) | Number of frames |
| --- | --- | --- | --- | --- |
|  | IntoTrees | 3840x2160 | 50 | 400 |
|  | OldTownCross | 3840x2160 | 50 | 400 |
|  | ParkJoy | 3840x2160 | 50 | 400 |
|  | DucksTakeOff | 3840x2160 | 50 | 400 |

# Subjective Quality Assessment Results

The results of the subjective tests described in Section 2 are presented in this section. Table 2 shows the results of subjective video quality assessments through which the two encoders were compared. For each of the video sequences, the average bit rates of the eBrisk-UHD and x264 encoders are shown along with the viewers’ assessments of the eBrisk-UHD and x264 encoded/decoded video sequences, according to the five-degree scale described in Section 2. Each video sequence was encoded using a fixed quantization parameter (QP). The QP values for the encodings were selected to yield good, but not especially high video quality, in order to avoid viewing scenarios where either both encodings would yield indistinguishably excellent quality or both encodings would yield substantial coding artifacts. In preparation for the subjective viewing tests, for each video sequence, several x264 encodings were performed, each with a different QP value. In each case, the encoding that yielded an average bit rate closest to 2.5 times that of the corresponding eBrisk-UHD encoding was selected (i.e., the bit rate of the eBrisk-UHD encoding was approximately 60% lower than that of x264).

In addition to the subjective tests comparing the eBrisk-UHD and x264 encoders, two supplementary tests were performed. In the first test, each viewer was shown a side-by-side sequence of OldTownCross in which the two sides were identical to one another. The viewers were not told that such a test case would be included in the experiment. The subjective results for this test were as follows: 7% of viewers reported “left slightly better”, 7% reported “right slightly better” and 86% reported “no preference”. In the second test, the eBrisk-UHD encoder was compared to the HM 12 encoder using the Main profile random access configuration of the JCT-VC common conditions [14]. Each viewer was shown the side-by-side sequences of DucksTakeOff and ParkJoy, with one side presenting the eBrisk-UHD-encoded/decoded video sequence and the other side presenting the HM 12-encoded/decoded video sequence. The two encoders were configured to use the same bit rate. Approximately 40% of viewers either preferred the eBrisk-UHD-encoder/decoded video sequence or had no preference, suggesting that the compression efficiency of the eBrisk-UHD encoder is similar to that of the HM 12 encoder, despite the fact that the eBrisk-UHD encoder is, on average, more than 600-times faster on single-core x86-based hardware platforms, and more than 15,000-times faster on multi-core dual-Xeon-E5-2697-v2-based platforms.

Table 2. Subjective viewing comparison results for sequences encoded using the eBrisk-UHD and x264 encoders.

| Sequence Name | eBrisk-UHD QP | eBrisk-UHD Bitrate | x264 QP | x264 Bitrate | eBrisk-UHD Better | eBrisk-UHD Slightly Better | No Preference | x264 Slightly Better | x264 Better |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| InToTrees | 34 | 4.5 Mbps | 35 | 11.9 Mbps | 21 | 6 | 0 | 0 | 0 |
| OldTownCross | 32 | 3.1 Mbps | 34 | 7.8 Mbps | 14 | 3 | 3 | 4 | 3 |
| ParkJoy | 38 | 11.9 Mbps | 38 | 29.0 Mbps | 0 | 2 | 7 | 10 | 8 |
| DucksTakeOff | 37 | 15.4 Mbps | 39 | 37.6 Mbps | 12 | 4 | 3 | 6 | 2 |
| Selection Count Totals |  |  |  |  | 47 | 15 | 13 | 20 | 13 |
| Percent Totals |  |  |  |  | 43.5% | 13.9% | 12.0% | 18.5% | 12.0% |

# Observations

The subjective results presented in Table 2 show that the viewers either had no preference or favored the eBrisk-UHD-encoded/decoded video at a bit rate that was approximately 60% lower than that of x264 in 69.4% of the trials. These results are consistent with the subjective results reported in [15]. In addition, the results demonstrate an increased coding efficiency gain in favor of HEVC compared with H.264 / MPEG-4 AVC when using higher-resolution content. In fact, comparing the coding efficiency gains of HEVC for certain 4K sequences in this study with those gains reported in earlier studies (e.g., [13]), in which high-quality resampled (lower-resolution) versions of the same video sequences were used, suggests that the increased coding efficiency gains observed for 4K sequences cannot be explained solely by differences in content.

We note that the eBrisk-UHD encoder is capable of encoding the 4K content discussed in this paper at an average of more than 35 fps on a dual-Xeon-E5-2697-v2-based computing platform. The x264 encoder is capable of encoding the same content at an average of 31 fps on the same platform. In order to perform the speed test for x264, the “–threads” parameter was set to ensure that all Xeon cores were fully engaged. We note that the eBrisk-UHD and x264 encoding speeds were measured at the bit rates used in the experiments (i.e., with similar subjective video quality). When the bit rate of x264 was lowered to match that used by the eBrisk-UHD encoder, the x264 encoding speed increased to 38 fps. Keeping in mind that the x264 encoder implementation has benefitted from several years of coding efficiency and subjective quality optimization, the fact that the eBrisk-UHD encoder compares so favorably so soon after the ratification of Recommendation H.265 Version 1.0 (04/2013) / ISO/IEC 23008-2, suggests that the HEVC design is well-suited for the development of production-quality encoders.

In summary, it is suggested that the results generally confirm that the HEVC design (as represented by the eBrisk-UHD encoder) yields a substantial improvement in compression capability beyond that of H.264 / MPEG-4 AVC (as represented by x264) for video streaming applications. Further, mounting evidence suggests that the coding performance gains of HEVC over H.264 / MPEG-4 AVC generally increase with increasing video resolution up to at least 4K resolutions.

# Conclusion

The results presented in this paper compare the compression performance of the eBrisk-UHD encoder, which is conformant with Recommendation H.265 Version 1.0 (04/2013) / ISO/IEC 23008-2 Main Profile (MP) with x264, which is conformant with the H.264 / MPEG-4 AVC High Profile (HP). The results suggest that HEVC generally produced better subjective video quality when compared with H.264 / MPEG-4 AVC in the context of video streaming applications, while maintaining an average bit rate that is approximately 60% lower than that of H.264 / MPEG-4 AVC. More specifically, viewers either expressed no preference or favored the eBrisk-UHD-encoded/decoded video at a bit rate that was approximately 60% lower than that of x264 in 69.4% of the trials. In addition, in a comparison between the eBrisk-UHD and HM 12 encoders with the two encoders configured to use the same average bit rate, approximately 40% of the viewers either preferred the eBrisk-UHD encoded/decoded video or had no preference. This result suggests that the compression efficiency of the eBrisk-UHD encoder is similar to that of the HM 12 encoder, despite the fact that the eBrisk-UHD encoder achieves average encoding speeds that are more than 600-times faster on single-core x86-based hardware platforms, and more than 15,000-times faster on multi-core dual-Xeon-E5-2697-v2-based platforms. Furthermore, a comparison of these 4K subjective results with results from earlier subjective studies where lower-resolution video content was considered, suggests that the coding efficiency gains of HEVC over H.264 / MPEG-4 AVC increase with the resolution of the video sequence, at least up to 4K resolutions. Finally, the fact that the eBrisk-UHD encoder compares so favorably to the x264 encoder so soon after the ratification of Recommendation H.265 Version 1.0 (04/2013) / ISO/IEC 23008-2, suggests that the HEVC design is well suited for the development of production-quality encoders.

Acknowledgement

The authors would like to thank the members and staff of Wavefront™, a not-for-profit Canadian National Centre of Excellence for accelerating the growth of wireless companies, for participating in the subjective video evaluations.

References

- ITU-T Rec. H.265 | ISO/IEC 23008-2, High Efficiency Video Coding (Main Profile, April 2013).

- VideoLAN x264 software library, http://www.videolan.org/developers/x264.html, version core 135 r2345, (July 30, 2013)

- ITU-T Rec. H.264 | ISO/IEC 14496-10, Advanced video coding (High Profile, July 2004).

- B. Li, G. J. Sullivan, J. Xu, “Comparison of Compression Performance of HEVC Draft 7 with AVC High Profile,” JCT-VC document JCTVC-J0236, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th meeting, Stockholm, SE, July 2012.

- B. Li, G. J. Sullivan, J. Xu, “Comparison of Compression Performance of HEVC Draft 6 with AVC High Profile,” JCT-VC document JCTVC-I0409, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th meeting, Geneva, CH, Apr. 2012.

- B. Li, G. J. Sullivan, J. Xu, “Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile,” JCT-VC document JCTVC-H0360, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th meeting, San José, CA, Feb. 2012.

- B. Li, G. J. Sullivan, J. Xu, “Compression Performance of High Efficiency Video Coding (HEVC) Working Draft 4,” IEEE Intl. Symp. Circuits and Systems, May 2012.

- B. Li, G. J. Sullivan, J. Xu, “Comparison of Compression Performance of HEVC Working Draft 4 with AVC High Profile,” JCT-VC document JCTVC-G399, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th meeting, Geneva, CH, Nov. 2011.

- Y. Zhao, “Coding efficiency comparison between HM5.0 and JM16.2 based on PQI, PSNR and SSIM,” JCT-VC document JCTVC-H0063, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th meeting, San José, CA, Feb. 2012.

- T. K. Tan, A. Fujibayashi, Y. Suzuki, J. Takiue, “Objective and subjective evaluation of HM5.0,” JCT-VC document JCTVC-H0116, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th meeting, San José, CA, Feb. 2012.

- V. Baroncini, J.-R. Ohm, G. J. Sullivan, “Report on preliminary subjective testing of HEVC compression capability,” JCT-VC document JCTVC-H1004, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th meeting, San José, CA, Feb. 2012.

- F. Kossentini, N. Mahdi, H. Guermazi, M. Horowitz, J. Xu, B. Li, G. J. Sullivan, J. Xu, “Informal Subjective Quality Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile,” document JCTVC-H0562, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th meeting, San José, CA, Feb. 2012.

- M. Horowitz, F. Kossentini, N. Mahdi, S. Xu, H. Guermazi, H. Tmar, B. Li, G. J. Sullivan, J. Xu, "Informal subjective quality comparison of video compression performance of the HEVC and H.264/MPEG-4 AVC standards for low-delay applications", SPIE Applications of Digital Image Processing XXXV, Proc. SPIE Vol. 8499, Aug. 2012.

- F. Bossen, “Common HM test conditions and software reference configurations”, document JCTVC-K1100, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th meeting, Shanghai, CN, Oct. 2012.

- P. Hanhart, F. De Simone, M. Rerabek, T. Ebrahimi, “Subjective quality evaluation of the upcoming HEVC video compression standard,” SPIE Applications of Digital Image Processing XXXV, Proc. SPIE, Vol. 8499, paper 8499-30, Aug. 2012.

# Patent rights declaration(s)

eBrisk Video, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The horizontal cropping removed one half of the pixels so as to enable both the eBrisk-UHD and x264 encodings to be displayed side-by-side on the 3840 pixel-wide 4K monitor.

The earlier studies also used earlier versions of the HEVC draft standard and a slightly different evaluation methodology.