The changes of the proposal on top of JCTVC-O0218 are marked in yellow.

7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if (palette_mode_flag[x0][y0]) { | ae(v) |
| if (!palette_pred_flag[x0][y0]){ | ae(v) |
| palette_size_minus1[x0][y0] | ae(v) |
| for (cIdx = 0; cIdx < 3; cIdx ++) |  |
| for( i = 0; i <=palette_size_minus1[x0][y0]; i ++ ) |  |
| palette[x0][y0][ cIdx][i] | ae(v) |
| } |  |
| else{ |  |
| leftIdx = x0-1 |  |
| while (palette_mode_flag in CU[leftIdx][y0] ==0){ |  |
| leftIdx = leftIdx -1 |  |
| } |  |
| palette_size_minus1[x0][y0]= palette_size_minus1[leftIdx][y0] |  |
| for (cIdx = 0; cIdx < 3; cIdx ++) |  |
| for( i = 0; i <=palette_size_minus1[x0][y0]; i ++ ) |  |
| palette[x0][y0][ cIdx][i]= palette[leftIdx][y0][ cIdx][i] |  |
| } |  |
| scanPos = 0 |  |
| while (scanPos < nCbS * nCbS) { |  |
| run_mode_flag | ae(v) |
| if (run_mode_flag) |  |
| ….. |  |
| } |  |

## Semantics

palette_mode_flag[x0][ y0] equal to 1 specifies that the current coding unit is encoded using palette mode. palette_mode_flag  equal to 0 specifies the current coding unti is encoded using a mode other than palette mode.

When palette_mode_flag is not present, it is inferred to be equal to 0.

In the following sematics, the array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

palette_pred_flag[x0][y0] equal to 1 specifies that the palette of current coding unit is predicted from left coding units. Note the the left coding units may not necessary adjacent to the current coding unit. palete_pred_flag equal to 0 specifies the palette of current coding unit is computed base on the pixel of current coding unit. When palette_pred_flag is not present, it is inferred to be equal to 0.

palette_size_minus1[x0][y0] plus 1 specifies the number of elements in the palette. palette_size_minus1[x0][y0] shall be the range of 0 to 23.