The changes are based on top of WD text of JCTVC-P0108. Changes are marked in yellow

7.3.8.5 Coding unit syntax

| … | Descriptor |
| --- | --- |
| major_color_num[ x0 ][ y0 ][ cIdx ] | ae(v) |
| for( n = 0; n < major_color_num[ cIdx ]; n++ ) { |  |
| major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| if( !major_color_pred[ cIdx ] ) { |  |
| if( major_color_pred[ cIdx ] ) { |  |
| major_color_pred_dir[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| } else { |  |
| major_color_value[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| } |  |
| … |  |

7.4.9.5 Coding unit semantics

…

major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] equal to 1 specifies n-th major color value for color component cIdx could be predicted from the above or left CU. major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] equal to 0 specifies current major color couldn’t be predicted. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_pred_dir[ x0 ][ y0 ][ cIdx ][ n ] specifies the prediction direction of the n-th major color for color component cIdx. major_color_pred_dir[ x0 ][ y0 ][ cIdx ][ n ] equal to 1 specifies current major color is the same as the corresponding major color in the above CU. When it equals to 0, current major color is the same as the corresponding major color in the left CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

…

Table 9-34 Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| coding_unit( ) | cu_transquant_bypass_flag | FL | cMax = 1 |
|  | cu_skip_flag | FL | cMax = 1 |
|  | intra_mc_flag | FL | cMax = 1 |
|  | intra_scm_flag | FL | cMax = 1 |
|  | major_color_num | FL | cMax = 15 |
|  | major_color_value | FL | cMax = cIdx == 0 ? (1<<BitDepthY) – 1 : (1<<BitDepthC) – 1 |
|  | major_color_pred_flag | FL | cMax = 1 |
|  | major_color_pred_dir | FL | cMax = 1 |
|  | line_mode | FL | cMax = 2 |
|  | index_pred_symbol | FL | cMax = 2 |
|  | quant_escape_value | FL | cMax = cIdx == 0 ? (1<<BitDepthY) – 1 : (1<<BitDepthC) – 1 |
|  | color_index | FL | cMax = 16 |
|  | mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | rem_intra_luma_pred_mode[ ][ ] | FL | cMax = 31 |

Table 9-39 Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| cu_skip_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| zpred_mode_flag | 0 | na | na | na | na | na |
| intra_scm_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| major_color_num | bypass | bypass | bypass | bypass | na | na |
| major_color_value | bypass | bypass | bypass | bypass | bypass | bypass |
| major_color_pred_flag | 0,1 (subclause Error: Reference source not found) | na | na | na | na | na |
| major_color_pred_dir | 0 | na | na | na | na | na |
| line_mode | 0,1 | 0,1,2 | na | na | na | na |
| index_pred_symbol | 0,1 | 0,1,2 | na | na | na | na |
| quant_escape_value | bypass | bypass | bypass | bypass | bypass | bypass |
| part_mode log2CbSize = = MinCbLog2SizeY | 0 | 1 | 2 | bypass | na | na |