| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0162 |
| --- | --- |

| Title: | On 16-bit coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Aminlou K. Ugur | Tel: Email: | +90 536 567 2997 kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes additional results on separating the 16-bit video into two lower bit depth sources (e.g 8 bits) and coding those separately for lossless operation. It is asserted that this method improves the coding efficiency of 16-bit lossless operation and also provides backwards compatibility.

# Introduction

It was suggested earlier in JCTVC-O0090 to utilize auxiliary pictures for coding 16-bit pictures/videos. This contribution evaluates the performance of such method, specifically for lossless coding, where it was considered that lossless 16-bit coding is an important area.

# Proposal

## Simplified architecture

The proposal could be illustrated in Figure 1.

Figure 1 Illustration of the proposal

As seen in Figure 1, the idea is to separate the 16-bit source into two lower bit-depth sources (8-bits in the above example) and code these with HEVC version 1 codec. The benefits of this proposal are:

- Version 1 HEVC decoders/encoders could be used to code 16-bit video.

- 16-bit is achieved as a backwards compatible extension to 8-bit video.

## Generalized architecture

The generalized architecture is a specific method to separate the 16-bit source into two (or more) 8-bit, 10-bit or 12-bit sources. The main difference the above method in is how to generate the LSB source, and how to combine the reconstructed MSB and LSB to compose the 16-bit reconstruction. In this generalized architecture, LSB includes both the remaining least significant bits and the residual of the MSB.

To generate the reconstructed 16-bit content, the reconstructed MSB and LSB are shifted to left (or right) with appropriate bit numbers, and then added together. This division is more optimal in compression efficiency. For example, to benefit from the LSB coding, it is not needed to code the MSB part losslessly. In turn, the MSB part can be coded with an appropriate quality, and LSB coding takes cares of the coding of the MSB residual as well as the remaining least significant bits.

Figure 2 Illustration of the proposal, generalized architecture

Different steps can be summarized as below (Details of implementation are further described in the next section.):

- Stage-1: Generate a low bit-depth source (MSBorg) picture which mainly includes the most significant bits of the HBDorg source (and the rounded value from the remaining least significant bits).

- Stage-2: Encode the MSBorg using a legacy low bit-depth coder (Let’s call the encoded picture MSBrec)

- Stage-3: Shift MSBrec to left with appropriate number of bits to generate ShiftedMSBrec.

- Stage-4: Generate difference by subtracting the ShiftedMSBrec from HBDorg source. Then generate biased difference by subtracting an offset to difference to make all the values as positive.

- Stage-5: Generate a low bit-depth source (LSBorg) picture which includes the appropriate bit range of biased difference.

- Stage-6: Encode the LSBorg using a legacy low bit-depth coder (let’s call the coded picture LSBrec)

- Stage-7: Indicate in the bitstream information on how to compose the HBDrec picture using the MSBrec and LSBrec.

- Stage-8: Decode from the bitstream information how to combine the bitstreams. Form the reconstructed HBDrec using two or more low bit-depth reconstructed pictures / videos by shifting MSBrec and LSBrec with appropriate bit numbers, and adding them together with an appropriate offset value.

# Implementation and Results

The proposal could be implemented in three ways:

- The MSB and LSB part could be packed into and frame packing arrangement SEI message could be used.

- The LSB part could be signaled as an auxiliary picture layer and MSB picture could be the base picture.

- MSB and LSB parts could form the base and enhancement layers of a scalable video where EL is coded without predicting from the BL picture.

To evaluate the coding efficiency of the proposal, we separated the original 16-bit video into MSB and LSB parts and coded those separately. Those results are compared with the original 16-bit lossless coded results and provided below. It should be noted that for all of these cases, coding efficiency results would be identical or very similar.

## Simplified architecture

Table 1. Simulation results lossless coding, MSB and LSB coded lossless, Intra configuration, number of frames = 50

|  | bitrate (kbps) |  |  |  | Rate savings |
| --- | --- | --- | --- | --- | --- |
| Sequence | 16bit | MSB 8bit | LSB 8bit | MSB+LSB |  |
| CrowdRun | 5170146 | 1679605 | 2700102 | 4379706 | -15.3% |
| DucksTakeOff | 5280572 | 1734432 | 2700147 | 4434579 | -16.0% |
| InToTree | 4980271 | 1584147 | 2699054 | 4283200 | -14.0% |
| OldTownCross | 5219718 | 1701319 | 2699015 | 4400334 | -15.7% |
| ParkJoy | 4939025 | 1575163 | 2702194 | 4277357 | -13.4% |
| Average |  |  |  |  | -14.9% |

Table 2. Simulation results lossless coding, MSB and LSB coded lossless, Lowdelay configuration, number of frames = 50

|  | bitrate (kbps) |  |  |  | Rate savings |
| --- | --- | --- | --- | --- | --- |
| Sequence | 16bit | MSB 8bit | LSB 8bit | MSB+LSB |  |
| CrowdRun | 4902434 | 1563738 | 2699941 | 4263680 | -13.0% |
| DucksTakeOff | 5203950 | 1699171 | 2700041 | 4399213 | -15.5% |
| InToTree | 4777475 | 1492226 | 2698961 | 4191187 | -12.3% |
| OldTownCross | 5075446 | 1623213 | 2698879 | 4322092 | -14.8% |
| ParkJoy | 4717655 | 1479947 | 2702016 | 4181962 | -11.4% |
| Average |  |  |  |  | -13.4% |

## Generalized architecture

The generalized architecture results in a better gain than the simplified architecture.

Table 3. Simulation results for lossless coding, MSB10bit lossy coded with QP=13, LSB12bit lossless coded, Intra configuration, number of frames = 50

|  | bitrate (kbps) |  |  |  | Rate savings |
| --- | --- | --- | --- | --- | --- |
| Sequence | 16bit | MSB | LSB | MSB+LSB |  |
| CrowdRun | 5170146 | 1100126 | 3120699 | 4220825 | -18.4% |
| DucksTakeOff | 5280572 | 1178481 | 3116777 | 4295258 | -18.7% |
| InToTree | 4980271 | 1008456 | 3124676 | 4133132 | -17.0% |
| OldTownCross | 5219718 | 1147577 | 3117242 | 4264819 | -18.3% |
| ParkJoy | 4939025 | 1000396 | 3121820 | 4122216 | -16.5% |
| Average |  |  |  |  | -17.8% |

Table 4. Simulation results for lossless coding, MSB10bit lossy coded with QP=12, LSB12bit lossless coded, Lowdelay configuration, number of frames = 50

|  | bitrate (kbps) |  |  |  | Rate savings |
| --- | --- | --- | --- | --- | --- |
| Sequence | 16bit | MSB | LSB | MSB+LSB |  |
| CrowdRun | 4902434 | 918960.1 | 3213495 | 4132455 | -15.7% |
| DucksTakeOff | 5203950 | 1074202 | 3205133 | 4279335 | -17.8% |
| InToTree | 4777475 | 861407.6 | 3219510 | 4080918 | -14.6% |
| OldTownCross | 5075446 | 1020216 | 3205216 | 4225432 | -16.7% |
| ParkJoy | 4717655 | 826333.2 | 3223275 | 4049608 | -14.2% |
| Average |  |  |  |  | -15.8% |

## Simplified architecture (with PCM for LSB)

It was suggested that using PCM for the LSB part would avoid expansion of the LSB part and further improve the coding efficiency. This table shows the results of such scheme (for the sake of simplicity it is assumed that LSB part is the same as the original file size)

|  | bitrate (kbps) |  |  |  | Rate savings |
| --- | --- | --- | --- | --- | --- |
| Sequence | 16bit | MSB 8bit | LSB 8bit | MSB+LSB |  |
| CrowdRun | 5170146 | 1679605 | 2488320 | 4167925 | -19.4% |
| DucksTakeOff | 5280572 | 1734432 | 2488320 | 4222752 | -20.0% |
| InToTree | 4980271 | 1584147 | 2488320 | 4072467 | -18.2% |
| OldTownCross | 5219718 | 1701319 | 2488320 | 4189639 | -19.7% |
| ParkJoy | 4939025 | 1575163 | 2488320 | 4063483 | -17.7% |
| Average |  |  |  |  | -19.0% |

Compared to second revision of JCTVC-P0199 document where max. rice parameter is set to 10, the result are as follows:

|  | Proposal | JCTVC-P0199r2 |
| --- | --- | --- |
| Sequence |  |  |
| CrowdRun | -19.4% | -18.7% |
| DucksTakeOff | -20.0% | -19.6% |
| InToTree | -18.2% | -17.8% |
| OldTownCross | -19.7% | -19.6% |
| ParkJoy | -17.7% | -16.7% |
| Average | -19.0% | -18.5% |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).