| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0163r1 |
| --- | --- |

| Title: | AHG15: Interlaced to progressive scalability for SHVC hybrid codec use case |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Ye, Yong He, Yuwen He 9710 Scranton Rd, #250 San Diego, CA 92121 | Tel: Email: | +1-858-210-4803 yan.ye@interdigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

SHVC draft 4 supports hybrid codec scalability, where the base layer is coded using AVC, and the enhancement layers are coded using HEVC. However, SHVC draft 4 does not provide a complete solution when the AVC coded base layer is interlaced content. This contribution proposes to fully enable interlaced to progressive scalability for the hybrid codec use case. The proposed solution applies field parity based resampling process on the reconstructed base layer field pictures to generate the inter layer reference pictures, which are then used as additional reference pictures for coding of the progressive content in the enhancement layer. Experiments show that, compared to simulcast, the proposed method achieves {Y, U, V} BD-rate reduction of {-20.3%, -15.6%, -14.8%} for Random Access. Equivalently, for the HEVC coded progressive content (EL-only), {Y, U, V} BD-rate reduction of {-42.6%, -38.6%, -38.0%} can be achieved.

# Introduction

Hybrid codec scalability refers to the use case where the base layer video is coded using an earlier codec (e.g. AVC) and the enhancement layers are coded using the most recent HEVC standard which offers the highest coding efficiency. The need for proper support of hybrid codec scalability was established as a requirement for the SHVC project from the very beginning [1][2][3].

The current SHVC draft 4 [5] supports hybrid codec scalability by allowing a flag, called avc_base_layer_flag, to be set in vps_extensions(). When avc_base_layer_flag is set, only sample prediction from the base layer is allowed, and motion prediction is required to be disabled. However, such a simplistic approach does not properly consider an important use case for hybrid codec scalability, where the AVC coded base layer is interlaced content.

Hybrid codec scalability with interlaced base layer and progressive enhancement layer represents a very significant use case, because it allows a large percentage of AVC coded video content, which today is in the interlaced format, to migrate to the progressive format, without putting significant drain on the network bandwidth, since the progressive content can be coded using the more efficient HEVC codec.

In this contribution we propose the parity based resampling process for interlaced to progressive scalability for the hybrid codec use case.

# The proposed parity based resampling process

SHVC adopts the reference index based framework for efficient scalable video coding, where upsampled base layer pictures are used as additional reference pictures for enhancement layer coding. When the base layer is interlaced, the upsampling process needs to be adjusted to take into account the field parity (top or bottom) of the base layer field picture.  Only vertical upsampling of 2x factor is needed for interlaced to progressive scalability.

## Parity based resampling for luma

For the luma component, the following pseudo code describes the proposed resampling process based on field parity:

Pseudo code for field parity based upsampling:

if (base_layer_bottom_field_flag == 0)  // upsample the top field in BL

{

for (y = 0; y < frame height; y++)

{

y0 = y >> 1;

for (x = 0; x < frame width; x++)

{

if((y0 << 1) == y) {

copy pixel from the top field located at line y0

}

else {

fetch reference pixels between lines y0-TAP/2+1 and y0+TAP/2 from the BL top field

perform interpolation using the phase-8 filter

}

}

else // upsample the bottom field in BL

{

for (y = 0; y < frame height; y++)

{

y0 = y >> 1;

for (x = 0; x < frame width; x++)

{

if((y0 << 1) == y) {

fetch reference pixels between lines y0-TAP/2 and y0+TAP/2-1 from the BL bottom field

perform interpolation using phase-8 filter

}

else {

copy pixel from the bottom field located at line y0

}

}

## Parity based resampling for chroma

Figure 1 and Figure 2 show the default chroma sample locations in YUV4:2:0 video for progressive and interlaced video, respectively. In the vertical direction, the chroma samples are off by ½ pixel for progressive video, but are off by ¼ pixel for interlaced video. Therefore, for interlace to progressive upsampling, different phase filters should be used for chroma. Instead of the phase-6 and phase-14 chroma filters for the usual 2x vertical upsampling, the phase-0 and phase-8 filters should be used to upsample the chroma samples in the interlaced base layer. In other words, the filter phases are the same for the luma and the chroma components for field parity based upsampling.

- Default luma and chroma pixel locations in progressive YUV4:2:0 video

- Default luma and chroma pixel locations in interlaced YUV4:2:0 video

# Propose text change

The required text changes to SHVC draft 4 are relatively minor, and are shown in this section, including: 1) sequence level signaling described section in 3.1; 2) picture level signaling of field parity described in section 3.2; 3) modified phase filter selection for field parity based upsampling described in section 3.3.

## Proposed VPS modification

Table 1. Modified VPS extension

| vps_extension( ) { | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| if ( avc_base_layer_flag ) |  |
| vps_base_layer_frame_pic_only_flag | u(1) |
| vps_vui_present_flag | u(1) |
| …. |  |
| if ( vps_base_layer_frame_pic_only_flag ) |  |
| cross_layer_phase_alignment_flag | u(1) |
| …. |  |
| } |  |

vps_base_layer_frame_pic_only_flag equal to 1 specifies that the coded pictures of the base layer coded video sequence contain only coded frames. vps_base_layer_frame_pic_only_flag equal to 0 specifies that the coded pictures of the base layer coded video sequence may be either coded fields or coded frames. When vps_base_layer_frame_pic_only_flag is not present, it is inferred to be 1.

## Proposed slice header modification

Table 2. Modified slice segment header

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| … |  |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if (nuh_layer_id > 0 && VpsInterLayerSamplePredictionEnabled[ LayerIdxInVPS [nuh_layer_id ] ][ LayerIdxInVPS [ 0 ] ] && !vps_base_layer_frame_pic_only_flag ) { |  |
| base_layer_frame_pic_flag | u(1) |
| if (! base_layer_frame_pic_flag ) |  |
| base_layer_bottom_field_flag | u(1) |
| } |  |
| … |  |
| } |  |
| byte_alignment( ) |  |
| } |  |

base_layer_frame_pic_flag equal to 1 specifies that the base layer picture used in inter layer sample prediction is a frame picture. base_layer_frame_pic_flag equal to 0 specifies that the base layer picture used in inter layer sample prediction is a top or bottom field picture. When base_layer_frame_pic_flag is not present, it is inferred to be 1.

base_layer_bottom_field_flag equal to 1 specifies that the base layer picture used in inter layer sample prediction is a bottom field picture. base_layer_bottom_field_flag equal to 0 specifies that the base layer picture used in inter layer sample prediction is a top field picture. When base_layer_ bottom_field_flag is not present, it is inferred to be 0.

## Proposed resampling modification

Only one line of change to subclause H.6.2 is needed to support the parity based upsampling:

The variables phaseX, phaseY, addX and addY are derived as follows:

phaseX = ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag	(H-1)

phaseY = (base_layer_frame_pic_flag = = 0) ? ( base_layer_bottom_field_flag << 2 ) : ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag + 1 	(H-6)

# Simulation results

The proposed parity based upsampling method is implemented in SHM4.0. Simulations are run using SHVC common test conditions defined for AVC base layer (random access setting). Test sequences include a set of 5 native pairs of 1080i and 1080p sequences that were professionally captured in progressive format and converted to interlaced format. One pair has frame/field rate of 50fps, and the remaining 4 pairs have frame/field rate of 60fps. The base layer interlaced sequences are coded using JM18.3, using the same config files as used to generate the SHVC CfP AVC base layer anchors, except for the following changes to encode interlaced content:

PList0References              = 0   # P slice List 0 reference override (0 disable, N <= NumberReferenceFrames)

CRA                                  =  0  # 0: do not use CRA like random access support; 1 use CRA like random access support

BReferencePictures          =  0  # Referenced B coded pictures (0=off, 1=B references for secondary layer, 2=B references for primary layer)

ExplicitHierarchyFormat = b3r0b1r1b0e2b2e2b5r1b4e2b6e2 # Explicit Enhancement GOP.

PicInterlace                      =  1     # Picture AFF    (0: frame coding, 1: field coding, 2:adaptive frame/field coding)

PAFF was turned off (PicInterlace = 1), because JM18.3 decoder crashed when decoding bitstreams generated with PAFF on (PicInterlace = 2). MBAFF was also turned off due to software issues.

Using simulcast as anchors, Table 3 shows the performance gain for the proposed field parity based upsampling method. On average 20.3% rate reduction is achieved for luma, when both layers are considered (overall BL+EL BD rate). For the HEVC coded progressive content, on average 42.6% bit rate reduction (EL-only rate reduction) is achieved. This could provide significant commercial incentive to use SHVC interlaced to progressive scalability to deliver the progressive content, rather than delivering the progressive content through simulcast. SHM4.0 without modification can also support interlaced to progressive scalability, by simply treating the base layer fields as pictures with half the vertical dimension. Table 4 shows the performance gain of the proposed method compared to unmodified SHM4.0 as anchors. On average 1.6% BL+EL BD-rate reduction can be achieved for the luma component, and average rate reduction of the HEVC coded progressive content (EL-only rate reduction) is 3.5%.

Table 3 1080i AVC to 1080p HEVC using the proposed field parity based upsampling, random access

|  | AVC 1080i to HEVC1080p |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -20.3% | -15.6% | -14.8% |
| Overall (Test vs Ref) | -20.3% | -15.6% | -14.8% |
| Overall (Test vs single layer) | 69.5% | 79.6% | 49.0% |
| EL only (Test vs Ref) | -42.6% | -38.6% | -38.0% |

Table 4 1080i AVC to 1080p HEVC, using unmodified SHM4.0 software, random access

|  | AVC 1080i to HEVC1080p |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -1.6% | -1.4% | -1.1% |
| Overall (Test vs Ref) | -1.6% | -1.4% | -1.1% |
| Overall (Test vs single layer) | 69.5% | 79.6% | 49.0% |
| EL only (Test vs Ref) | -3.5% | -3.4% | -3.1% |

# Conclusion

The proposed field parity based upsampling method achieves efficient scalable coding by reducing 40% of the required bandwidth necessary to code the progressive video content, compared to simulcast. The proposed method requires only minimal changes to the SHVC draft text. It is therefore suggested to adopt the proposed upsampling method to enable interlaced to progressive scalability for the SHVC hybrid codec use case.

# Patent rights declaration(s)

InterDigital Communications, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Luthra, Joint Call for Proposals on the Scalable Video Coding Extensions of HEVC, ISO/IEC JTC-1/SC29/WG11 N12957, July 2012.

- Luthra, Use cases for the scalable enhancement of HEVC, ISO/IEC JTC-1/SC29/WG11 N12955, July 2012.

- Luthra, Requirements for the scalable enhancement of HEVC, ISO/IEC JTC-1/SC29/WG11 N12956, July 2012.

- J. Chen, J. Boyce, Y. Ye, and M. M. Hunnuksela, SHVC Test Model 4 (SHM 4), JCTVC document no JCTVC-O1007, Oct 2013.

- J. Chen, J. Boyce, Y. Ye, and M. M. Hunnuksela, Y. K. Wang, SHVC Draft 4, JCTVC document no JCTVC-O1008, Oct 2013.