| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, USA, 9 – 17 Jan. 2014 | Document: JCTVC-P0166 |
| --- | --- |

| Title: | Some Syntax Modifications for HEVC Range Extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu Limin Wang Arris Incorporated 6450 Sequence Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-404-2251 yue.yue@arrisi.com 1-858-404-3893 limin.wang@arrisi.com |
| Source: | ARRIS Group, Inc. |  |  |

_____________________________

# Abstract

This contribution presents several syntax modifications for HEVC range extension [1]. The first part of this contribution proposes a conditional parsing for PPS extension syntax to allow decoders not to parse unsupported PPS extension syntax elements. In the second part of this contribution, it is proposed to present all transform skip related syntax for range extension at PPS level and all these syntax are conditionally parsed if transform_skip_enabled_flag is enable. In the third part of this contribution, we propose a solution to avoid several unnecessary intra copy block flag check in the current specification. This modification does not involve any coding performance change.

# Conditionally Parsing PPS extension Syntax

The problem for incompatible SPS extension syntax was addressed in the JCTVC-O0142 [2]. Similar problem for parsing PPS extension exists in the current HEVC range extension [1]. pps_extension2_flag is conditionally parsed by checking the value of pps_extension1_flag. However, pps_extension2_flag is always checked no matter whether it is present or not.

| .......... |  |
| --- | --- |
| pps_extension1_flag | u(1) |
| if( pps_extension1_flag ) { |  |
| if( transform_skip_enabled_flag ) |  |
| log2_max_transform_skip_block_size_minus2 | ue(v) |
| luma_chroma_prediction_enabled_flag | u(1) |
| chroma_qp_adjustment_enabled_flag | u(1) |
| if( chroma_qp_adjustment_enabled_flag ) { |  |
| diff_cu_chroma_qp_adjustment_depth | ue(v) |
| chroma_qp_adjustment_table_size_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_adjustment_table_size_minus1; i++ ) { |  |
| cb_qp_adjustment[ i ] | se(v) |
| cr_qp_adjustment[ i ] | se(v) |
| } |  |
| } |  |
| pps_extension2_flag | u(1) |
| } |  |
| if( pps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

A simple solution for this problem is that the status of pps_extension2_flag will be conditionally checked only if it is present. However, such solution may be undesirable because the later standardized extension must include syntax elements related to the previous standardized extension, even if those extensions are not related.

Following the similar design of parsing SPS extension syntax elements, we propose to add additional eight flags to indicate further extension syntax elements if pps_extension_present_flag is equal to 1. Each individual flag controls whether the specific extension syntax are present. A 7-bit syntax is used to indicate further extension. The proposed syntax is shown as follows.

| ........... |  |
| --- | --- |
| pps_extension1_present_flag | u(1) |
| if( pps_extension1_present_flag ) { |  |
| for( i = 0; i < 1; i++ ) |  |
| pps_extension_flag[ i ] | u(1) |
| pps_extension_7bits | u(7) |
| if( pps_extension_flag[0] ) { |  |
| if( transform_skip_enabled_flag ) |  |
| log2_max_transform_skip_block_size_minus2 | ue(v) |
| luma_chroma_prediction_enabled_flag | u(1) |
| chroma_qp_adjustment_enabled_flag | u(1) |
| if( chroma_qp_adjustment_enabled_flag ) { |  |
| diff_cu_chroma_qp_adjustment_depth | ue(v) |
| chroma_qp_adjustment_table_size_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_adjustment_table_size_minus1; i++ ) { |  |
| cb_qp_adjustment[ i ] | se(v) |
| cr_qp_adjustment[ i ] | se(v) |
| } |  |
| } |  |
| } |  |
| pps_extension2_flag | u(1) |
| if(pps_extension_7bits ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| } |  |
| if( pps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

pps_extension_present_flag specifies that pps_extension_flag[ i ] for i in 0 .. 7 are present.

pps_extension_flag[ i ] equal to 1 specifies that the pps_extension[i] syntax structure is present. pps_extension_flag[ i ] equal to 0 specifies that the pps_extension[i] syntax structure is not present.

pps_extension_7bits equal to 0 specifies that no pps_extension_data_flag syntax elements are present in the PPS RBSP syntax structure. When present, pps_extension_7bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Values of pps_extension_7bits not equal to 0 are reserved for future use by ITU-T | ISO/IEC. Decoders shall allow the value of pps_extension_7bits to be not equal to 0 and shall ignore all pps_extension_data_flag syntax elements in a PPS NAL unit.

More specifically,

pps_extension_flag[ 0 ] equal to 1 specifies that log2_max_transform_skip_block_size_minus2, luma_chroma_prediction_enabled_flag, chroma_qp_adjustment_enabled_flag, diff_cu_chroma_qp_adjustment_depth, chroma_qp_adjustment_table_size_minus1, cb_qp_adjustment, cr_qp_adjustment are present in the PPS RBSP syntax structure. pps_extension_flag[ 0 ] equal to 0 specifies that these syntax elements are not present.

# Modifications of Transform Skip Related Syntax for HEVC Range Extension

The current HEVC range extension adds several transform skip related syntax, such as  log2_max_transform_skip_block_size_minus2, transform_skip_rotation_enabled_flag, and transform_skip_context_enabled_flag. In the current HEVC ragne extension, transform_skip_rotation_enabled_flag, and transform_skip_context_enabled_flag  are present at SPS while log2_max_transform_skip_block_size_minus2 is present at PPS. However, all these three syntax should be used only if transform skip is allowed, which is controlled by a flag, transform_skip_enabled_flag, at PPS. Therefore, all these three transform skip related syntax should be conditionally parsed after the value of transform_skip_enabled_flag has been checked.

In this contribution, it is proposed that all the transform skip related syntax, including log2_transform_skip_max_size_minus2, transform_skip_rotation_enabled_flag and transform_skip_context_enabled_flag, are present at PPS. Considering transform_skip_rotation_enabled_flag and transform_skip_context_enabled_flag also can be used for lossless coding. It is proposed that transform_skip_rotation_enabled_flag and transform_skip_context_enabled_flag are present if either transform_skip_enabled_flag or transquant_bypass_enabled_flag is equal to 1. The following tables show new SPS and PPS designs for HEVC range extension.

| ....... |  |
| --- | --- |
| sps_extension_present_flag | u(1) |
| if( sps_extension_present_flag ) { |  |
| for( i = 0; i < 1; i++ ) |  |
| sps_extension_flag[ i ] | u(1) |
| sps_extension_7bits | u(7) |
| if( sps_extension_flag[ 0 ] ) { |  |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| implicit_rdpcm_enabled_flag | u(1) |
| explicit_rdpcm_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| high_precision_offsets_enabled_flag | u(1) |
| fast_rice_adaptation_enabled_flag | u(1) |
| } |  |
| if( sps_extension_7bits ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| .......... |  |
| --- | --- |
| pps_extension1_flag | u(1) |
| if( pps_extension1_flag ) { |  |
| if( transform_skip_enabled_flag ) |  |
| log2_max_transform_skip_block_size_minus2 | ue(v) |
| if( transform_skip_enabled_flag ¦¦ transquant_bypass_enabled_flag){ |  |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| } |  |
| luma_chroma_prediction_enabled_flag | u(1) |
| chroma_qp_adjustment_enabled_flag | u(1) |
| if( chroma_qp_adjustment_enabled_flag ) { |  |
| diff_cu_chroma_qp_adjustment_depth | ue(v) |
| chroma_qp_adjustment_table_size_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_adjustment_table_size_minus1; i++ ) { |  |
| cb_qp_adjustment[ i ] | se(v) |
| cr_qp_adjustment[ i ] | se(v) |
| } |  |
| } |  |
| pps_extension2_flag | u(1) |
| } |  |
| if( pps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Simplification of Intra Block Copy Syntax

In the current HEVC range extension, if intra_block_copy_enabled_flag specified at SPS is equal to 1, a one-bit intra_bc_flag is always coded except the current CU is inter skip mode. The status of intra_bc_flag indicates whether intra block copy is used or not.

In the current design, it is not optimal to present intra_bc_flag and corresponding operation of intra copy block because it may check the status of intra_bc_flag up to three times. In this contribution, we propose a simplication of the current design. As a result, there is only one time check of the status of intra_bc_flag without any performance change.

In addition, when intra_bc_flag is 1, CuPredMode[ x ][ y ] is inferred to be equal to MODE_INTRA. Therefore, the check of (CuPredMode[ x0 ][ y0 ]  = =  MODE_INTRA  &&  intra_bc_flag[ x0 ][ y0 ]) is equivalent to the check of intra_bc_flag[ x0 ][ y0 ] and we can also simplify this part. The new syntax table is shown as follows.

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag[ x0 ][ y0 ] && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| mvd_coding( x0, y0, 2) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| if( ChromaArrayType = = 3 ) |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| intra_chroma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| else if( ChromaArrayType != 0 ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS * 3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS * 3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS * 3 / 4, nCbS ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS * 3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } |  |
| else |  |
| mvd_coding( x0, y0, 2) |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

Note that the yellow color highlights the changes and the red color with strikethrough indicates the removed lines.

References

- D. Flynn, J. Sole, T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 4,” JCTVC-O1005_v3, Nov. 2013.

- Jill Boyce, “Conditional SPS extension syntax for RExt, SHVC, and MV-HEVC,” JCTVC-O142, Nov. 2013

# Patent rights declaration(s)

Arris Group, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).