# Working Draft Text

#### 9.3.3.9 Binarization process for coeff_abs_level_remaining

Input to this process is a request for a binarization for the syntax element coeff_abs_level_remaining[ n ], the current sub-block scan index i, baseLevel, the colour component cIdx, and the luma location ( x0, y0 ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the picture.

Output of this process is the binarization of the syntax element.

The variables cLastAbsLevel and cLastRiceParam are derived as follows:

- If one or more of the following conditions are true, cLastAbsLevel and cLastRiceParam are set equal to 0:

- fast_rice_adaptation_enabled_flag is equal to 0 and this process is invoked for the first time for the current sub-block scan index i,

- this process is invoked for the first time for the current transform block.

- Otherwise, cLastAbsLevel and cLastRiceParam are set equal to the values of cAbsLevel and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element coeff_abs_level_remaining as specified in this subclause.

The variable cAbsLevel is set equal to baseLevel + coeff_abs_level_remaining[ n ].

The variable cRiceParam is derived from cLastAbsLevel and cLastRiceParam as follows:

- If fast_rice_adaptation_enabled_flag is equal to 0, the following applies:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel > ( 3 * ( 1  <<  cLastRiceParam ) ) ? 1 : 0 ), 4 )	(9-1)

- Otherwise (fast_rice_adaptation_enabled_flag is equal to 1), the following applies:

- cRiceParam is initialized as follows:

- -   If extended_precision_processing_flag is equal to 0

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel  >>  ( 2  +  cLastRiceParam ) ), 7)   (9-2)

-    Otherwise,

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel  >>  ( 2  +  cLastRiceParam ) ), 10)   (9-3)

- When this process is invoked for the first time for the current sub-block scan index i, cRiceParam is modified as follows:

- If CuPredMode [x][y] is equal to 1,

cRiceParam = Max( 0, cu_transquant_bypass_flag ?1 :  	4 - bit_depth_luma_minus8/2 + qP /6)

- Otherwise,

- cRiceParam = Max( 0, cu_transquant_bypass_flag ?1 :  4 - bit_depth_luma_minus8/2 + qP /6)/2

(9-4)

The variable cMax is derived from cRiceParam as:

cMax = 4  <<  cRiceParam	(9-5)

The binarization of the syntax element coeff_abs_level_remaining[ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of cu_qp_delta_abs, prefixVal, is derived as follows:

prefixVal = Min( cMax, coeff_abs_level_remaining[ n ] )	(9-6)

- The prefix bin string is specified by invoking the TR binarization process as specified in subclause Error: Reference source not found for prefixVal with the variables cMax and cRiceParam as inputs.

When the prefix bin string is equal to the bit string of length 4 with all bits equal to 1, the suffix bin string is present and it is derived as follows:

- The suffix value of cu_qp_delta_abs, suffixVal, is derived as follows:

suffixVal = coeff_abs_level_remaining[ n ] − cMax	(9-7)

- The suffix bin string is specified by invoking the EGk binarization process as specified in subclause Error: Reference source not found for suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1.