| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San Jose, USA, 9 Jan. – 17 Jan. 2014 | Document: JCTVC-P0168 WG11 Doc: m32098 |
| --- | --- |

| Title: | RCE2: Test A2. Rice parameter initialization based on quantization parameter and bit-depth |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or | Seung-Hwan Kim Misra Kiran Andrew Segall | Email: | kimse@sharplabs.com misrak @sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

In this contribution, a Rice parameter initialization method is introduced based on quantization parameter and bit depth information. In addition, the maximum Rice parameter is extended from 7 to 10 when extened_precision_flag is on. The proposed rice parameter initialization is performed at each transform unit (TU).

# Proposed method

In this contribution, Rice parameter (uiGoRiceParam) is initialized at TU level, and the initial Rice parameter is determined based on the existing information such as bit depth and quantization parameter as follows

uiGoRiceParam = TransQuantBypass_flag ? 1 : Max {0, 4 + (BD-8)/2 –QP/6}

If( !Mode_INTRA) uiGoRiceParam /=2

# Results

The coding gain of the proposed method is evaluated using the common test condition as follows;

- AHG 8 test conditions (SCC lossy and SCC lossless) as defined in [1] are used.

- AHG18 test conditions as defined in [2] [3] are used.

Table 1 Results on AHG 8 lossy coding

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB 4:4:4 SC | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | -0.3% | -0.3% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% |
| YCbCr 4:4:4 Anima. | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Op) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.4% | -0.5% | -0.5% |
| YCbCr 4:4:4 SC (Op) | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.6% | -0.7% | -0.8% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| YCbCr 4:4:4 Anima. | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Op) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC (Op) | 0.0% | 0.1% | 0.0% | 0.1% | 0.2% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| YCbCr 4:4:4 Anima. | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Op) | -0.1% | -0.1% | 0.0% | -0.4% | -0.3% | -0.2% |
| YCbCr 4:4:4 SC (Op) | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2 Results on AHG 8 lossless coding

|  | AI |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression ratio |  |  |  |  |  |  |  | Bit-rate increase |  |  |  |
|  | Total |  | Average |  | Min |  | Max |  | Total | Avg | Min | Max |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 4.58 | 4.58 | 5.59 | 5.59 | 2.27 | 2.27 | 11.20 | 11.20 | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 2.25 | 2.25 | 2.26 | 2.26 | 2.08 | 2.08 | 2.44 | 2.44 | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 9.40 | 9.40 | 11.7 | 11.71 | 6.50 | 6.50 | 19.62 | 19.62 | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 Anima. | 2.83 | 2.83 | 2.9 | 2.86 | 2.41 | 2.41 | 3.14 | 3.14 | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 11.17 | 11.17 | 13.5 | 13.52 | 7.96 | 7.96 | 21.63 | 21.63 | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 Anima. | 3.05 | 3.05 | 3.2 | 3.22 | 2.62 | 2.62 | 3.98 | 3.98 | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 1.98 | 1.98 | 2.44 | 2.44 | 1.52 | 1.52 | 4.43 | 4.43 | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Op.) | 30.91 | 30.91 | 34.30 | 34.30 | 17.27 | 17.27 | 48.88 | 48.88 | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC (Op.) | 35.53 | 35.53 | 38.63 | 38.63 | 24.30 | 24.30 | 58.96 | 58.96 | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |  |

|  | RA |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression ratio |  |  |  |  |  |  |  | Bit-rate increase |  |  |  |
|  | Total |  | Average |  | Min |  | Max |  | Total | Avg | Min | Max |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 8.64 | 8.64 | 35.85 | 35.85 | 3.01 | 3.01 | 88.86 | 88.86 | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 2.60 | 2.60 | 2.60 | 2.60 | 2.57 | 2.57 | 2.64 | 2.64 | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 56.83 | 56.83 | 140.5 | 140.50 | 20.03 | 20.03 | 290.96 | 290.96 | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 Anima. | 3.67 | 3.67 | 3.7 | 3.68 | 3.47 | 3.47 | 3.86 | 3.86 | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 71.01 | 71.01 | 161.3 | 161.25 | 24.73 | 24.73 | 321.23 | 321.23 | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 Anima. | 3.87 | 3.87 | 4.5 | 4.49 | 2.79 | 2.79 | 5.70 | 5.70 | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 2.14 | 2.14 | 2.57 | 2.57 | 1.59 | 1.59 | 4.50 | 4.50 | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Op.) | 70.95 | 70.95 | 329.65 | 329.65 | 19.93 | 19.93 | 498.73 | 498.73 | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC (Op.) | 109.02 | 109.02 | 356.47 | 356.47 | 32.52 | 32.52 | 601.48 | 601.48 | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |  |

|  | LB |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression ratio |  |  |  |  |  |  |  | Bit-rate increase |  |  |  |
|  | Total |  | Average |  | Min |  | Max |  | Total | Avg | Min | Max |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 8.84 | 8.84 | 56.21 | 56.21 | 3.00 | 3.00 | 163.08 | 163.08 | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 2.61 | 2.61 | 2.61 | 2.61 | 2.57 | 2.57 | 2.64 | 2.64 | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 63.65 | 63.65 | 485.60 | 485.60 | 21.03 | 21.03 | 2111.85 | 2111.85 | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 Anima. | 3.65 | 3.65 | 3.67 | 3.67 | 3.45 | 3.45 | 3.83 | 3.83 | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 80.58 | 80.58 | 544.80 | 544.80 | 25.92 | 25.92 | 2333.65 | 2333.65 | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 Anima. | 3.88 | 3.88 | 4.53 | 4.53 | 2.78 | 2.78 | 5.75 | 5.75 | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 2.13 | 2.13 | 2.57 | 2.57 | 1.59 | 1.59 | 4.50 | 4.50 | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Op.) | 77.21 | 77.21 | 1334.89 | 1334.89 | 20.09 | 20.09 | 3012.92 | 3012.92 | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC (Op.) | 125.19 | 125.19 | 1294.26 | 1294.26 | 33.08 | 33.08 | 2516.65 | 2516.65 | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |  |

Table 3 Results on AHG 18 high bit-depth coding (Transform Skip mode on)

|  |  |  | Intra 16-bit (internal bit depth) |  |  | Intra 12-bit (internal bit depth) |  |  | Lowdelay 16-bit (internal bit depth) |  |  | Lowdelay 12-bit (internal bit depth) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | -2.9% | -3.0% | -3.0% | -0.1% | 0.0% | 0.0% | -0.9% | -0.8% | -0.8% | 0.0% | 0.0% | 0.0% |
| RExt RGB |  |  | -1.4% | -1.3% | -1.3% | 0.0% | 0.0% | 0.0% | # | # | # | # | # | # |
| HDR |  |  | 0.0% | 0.2% | 0.2% | 0.3% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% | 0.1% | 0.0% | 0.0% |
| 4K from HDR |  |  | -0.5% | -0.4% | -0.4% | -0.1% | 0.0% | 0.0% |  |  |  |  |  |  |
| HDR 4:0:0 Medical |  |  | 0.0% |  |  | 0.4% |  |  | 0.1% |  |  | 0.1% |  |  |
| 4:0:0 Medical |  |  | -0.6% |  |  | 0.1% |  |  | -0.1% |  |  | 0.0% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | -0.6% | -0.3% | -0.3% | 0.2% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0.4% | 0.4% | 0.4% | 0.3% | 0.1% | 0.1% | 0.2% | 0.1% | -3.8% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 1.1% | 0.6% | 0.7% | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| All Classes |  |  | -0.5% | -0.6% | -0.5% | 0.1% | 0.1% | 0.1% | # | # | # | # | # | # |

Table 4 Results on AHG 18 high bit-depth coding (Transform Skip mode off)

|  |  |  | Intra 16-bit (internal bit depth) |  |  | Intra 12-bit (internal bit depth) |  |  | Lowdelay 16-bit (internal bit depth) |  |  | Lowdelay 12-bit (internal bit depth) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | -3.0% | -3.0% | -3.0% | -0.1% | 0.0% | 0.0% | -0.9% | -0.8% | -0.8% | 0.0% | 0.0% | 0.0% |
| RExt RGB |  |  | -1.4% | -1.3% | -1.3% | 0.0% | 0.1% | 0.0% | # | # | # | # | # | # |
| HDR |  |  | 0.0% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% | 0.1% | 0.0% | 0.0% |
| 4K from HDR |  |  | -0.4% | -0.4% | -0.4% | 0.0% | 0.1% | 0.1% |  |  |  |  |  |  |
| HDR 4:0:0 Medical |  |  | 0.0% |  |  | 0.4% |  |  | 0.1% |  |  | -0.1% |  |  |
| 4:0:0 Medical |  |  | -0.6% |  |  | 0.1% |  |  | -0.1% |  |  | 0.0% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | -0.4% | -0.3% | -0.3% | 0.2% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0.3% | 0.4% | 0.4% | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 1.1% | 0.6% | 0.8% | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| All Classes |  |  | -0.5% | -0.6% | -0.5% | 0.1% | 0.1% | 0.1% | # | # | # | # | # | # |

# Conclusion

The proposed Rice parameter initialization is simple and also provides efficient coding gain for higher bit depth coding environment.

# References

- D. Flynn, K. Sharman, C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-O1006, Geneva, CH, Oct. 2013.

- Chris Rosewarne, Joel Sole, Karl Sharman, S.-H. Kim “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialisation methods”, Joint Collaborative Team on Video Coding, JCTVC-O1122, Geneva, CH, 23 Oct. – 1 Nov. 2013

- Joel Sole, K. Sharman “HEVC Range Extensions Core Experiment 1 (RCE1): High bit rate coding at high bit depths”, JCTVC-O1121, Geneva, CH, 23 Oct. – 1 Nov. 2013

- S.-H. Kim, M. Kiran, A. Segall “Non-RCE2: Rice parameter initialization for higher bit depth coding,” JCTVC-O0327, Geneva, CH, CH, 23 Oct. – 1 Nov. 2013

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).