| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0171 |
| --- | --- |

| Title: | AHG14: Extension of SNR scalability with bit-depth scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 Cheung.Auyeung@am.sony.com |
|  | Ohji Nakagami, Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan |  | +81-50-3750-2746 ohji.nakagami@jp.sony.com Kazushi.Sato@jp.sony.com |
| Source: | Sony Electronics Inc. and Sony Corporation |  |  |

_____________________________

# Abstract

In SHM WD4 JCTVC-O1008_v3, when the base layer and the enhancement layer have the same picture size and the scaled reference layer offsets are zero, video bit-depth scalability is not supported. This contribution proposes to enable SHVC to support bit-depth scalability when both the base layer and enhancement layer have the same picture size. One use case is the encoding of high dynamic range (HDR) video with color gamut scalability tools. In this use case of SHVC, the high dynamic range video with higher bit-depth is encoded in the enhancement layer and the corresponding low dynamic range video with lower bit-depth is encoded in the base layer, and both layers have the same picture size.

# Introduction

In SHM WD4 [4], when the base layer and the enhancement layer have the same picture size and the scaled reference layer offsets are zero, video bit-depth scalability is not supported.  This contribution proposes to relax this constraint in SHVC so that bit-depth scalability is supported when the base layer and the enhancement layer have the same picture size.

# Use Cases

## Videos of the Internet Age

In this internet age, videos come in all sizes, shapes, and bit-depth. To support this internet age, SHVS should support bit-depth scalability regardless of the picture sizes of the base layer and enhancement layer. For example, the videos distributed for SCE1 [3] includes video with 8 bits and 10 bits version of the BT.709 videos.

## High Dynamic Range Video

Scalable codecs were proposed to encode high dynamic range video [1][2]. As shown in Figure 1, the SHVC could support coding of high dynamic range (HDR) video and low dynamic range (LDR) video if bit-depth scalability and weighted prediction are supported when the base layer and enhancement layer have the same picture size.

The inverse tone mapping in Figure 1 could be the simple bit-shift from SHM4 [4] or the 3D LUT from SCE1 [3].

Figure 1: Application of SHVC for the encoding of HDR and LDR videos.

# References

- M. Winken, D. Marpe, H. Schwarz, and T. Wiegand, “Bit-Depth Scalable Video Coding,” in Proceedings of the International Conference on Image Processing, pp. 5-8, 2007.

- C.A. Segall, “Scalable coding of high dynamic range video”,  in Proceedings of the 2007 International Conference on Image Processing, San Antonio, TX, pp. 1-4, 2007.

- Philippe Bordes, Yan Ye, Elena Alshina, Xiang Li, Seung-Hwan Kim, Alberto Duenas, Kemal Ugur, Kazushi Sato, “Description of HEVC Scalable Extensions Core Experiment SCE1: Color Gamut and Bit-Depth Scalability”, JCTVC-O1101, 15th Meeting: Geneva, CH, 23 October – 1 November 2013.

- Jianle Chen, Jill Boyce, Yan Ye, Miska M. Hannuksela, Ye-kui Wang, “High efficiency video coding (HEVC) scalable extension Draft 4”, JCVC-O1008_v3, 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013.

# Patent rights declaration(s)

Sony Electronics Inc. and Sony Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed Working Draft Text

The proposed changes to JCTVC-O1008-v3 is highlighted in yellow with bit-shift and weighted prediction to provide the functionality of inverse tone mapping.

- H.8.1.4	Resampling process for inter layer reference pictures

Input to this process is:

–	a decoded reference layer picture rlPic

–	a variable rLId specifies the layer id of reference layer picture .

Output of this process is the resampled reference layer picture rsPic.

The variables PicWidthInSamplesY and PicHeightInSamplesY are set equal to pic_width_in_luma_samples and pic_height_in_luma_samples, respectively.

The variables RefLayerPicWidthInSamplesY and RefLayerPicHeightInSamplesY are set equal to the width and height of the decoded reference layer picture rlPic in units of luma samples, respectively. The variables RefLayerBitDepthY and RefLayerBitDepthC are set equal to BitDepthY and BitDepthC of the decoded reference layer picture rlPic, respectively.

The variables PicWidthInSamplesC, PicHeightInSamplesC, RefLayerPicWidthInSamplesC, and RefLayerPicHeightInSamplesC are derived as follows:

PicWidthInSamplesC  = PicWidthInSamplesY / subWidthC	(H-1)
PicHeightInSamplesC = PicHeightInSamplesY / subHeightC	(H-2)
RefLayerPicWidthInSamplesC  = RefLayerPicWidthInSamplesY / subWidthC	(H-3)
RefLayerPicHeightInSamplesC = RefLayerPicHeightInSamplesY / subHeightC	(H-4)

The variable currLayerId is set equal to nuh_layer_id of the current picture..

The variables ScaledRefLayerLeftOffset, ScaledRefLayerTopOffset, ScaledRefLayerRightOffset and ScaledRefLayerBottomOffset are derived as follows:

ScaledRefLayerLeftOffset = scaled_ref_layer_left_offset[ rLId ] << 1	(H-5)
ScaledRefLayerTopOffset = scaled_ref_layer_top_offset[ rLId ]  << 1	(H-6)
ScaledRefLayerRightOffset = scaled_ref_layer_right_offset[ rLId  ]  << 1	(H-7)
ScaledRefLayerBottomOffset = scaled_ref_layer_bottom_offset[ rLId ]  << 1	(H-8)

The variables ScaledRefLayerPicWidthInSamplesY and ScaledRefLayerPicHeightInSamplesY are derived as follows:

ScaledRefLayerPicWidthInSamplesY = PicWidthInSamplesY – 
			ScaledRefLayerLeftOffset  – ScaledRefLayerRightOffset	(H-9)
ScaledRefLayerPicHeightInSamplesY = PicHeightInSamplesY – 
			ScaledRefLayerTopOffset – ScaledRefLayerBottomOffset	(H-10)

The variables ScaleFactorX and ScaleFactorY are derived as follows:

ScaleFactorX = ( ( RefLayerPicWidthInSamplesY << 16 ) + ( ScaledRefLayerPicWidthInSamplesY >> 1 ) ) / 
		ScaledRefLayerPicWidthInSamplesY		(H-11)
ScaleFactorY = ( ( RefLayerPicHeightInSamplesY << 16 ) + ( ScaledRefLayerPicHeightInSamplesY >> 1 ) ) / 
		ScaledRefLayerPicHeightInSamplesY		(H-12)

The following steps are applied to derive the resampled inter layer reference picture rsPic.

–	if PicWidthInSamplesY is equal to RefLayerPicWidthInSamplesY, PicHeightInSamplesY is equal to RefLayerPicHeightInSamplesY, the values of ScaledRefLayerLeftOffset, ScaledRefLayerTopOffset, ScaledRefLayerRightOffset and ScaledRefLayerBottomOffset are all equal to 0, RefLayerBitDepthY is equal to BitDepthY, and RefLayerBitDepthC is equal to BitDepthC.

  - rsPic is set equal to rlPic.

  - The picture sample resampling process as specified in subclause  is invoked with the samePictureSizeFlag equals to 1 and the sample arrays, rlPicSampleL, rlPicSampleCb and rlPicSampleCr, of the reference layer picture rlPic as inputs, and with the sample arrays of the resampled picture rsPic as outputs.

–	otherwise, rsPic is derived as follows:

  - The PicOrderCntVal value of rsPic is set equal to the PicOrderCntVal value of rlPic.

  - When VpsInterLayerSamplePredictionEnabled[ LayerIdxInVps[ currLayerId ] ][ LayerIdxInVps[ rLId ] ] is equal to 1, the picture sample resampling process as specified in subclause  is invoked with the samePictureSizeFlag equals to 0 and the sample arrays, rlPicSampleL, rlPicSampleCb and rlPicSampleCr, of the reference layer picture rlPic as inputs, and with the sample arrays of the resampled picture rsPic as outputs.

  - When VpsInterLayerMotionPredictionEnabled[ LayerIdxInVps[ currLayerId ] ][ LayerIdxInVps[ rLId ] ] is equal to 1, the following steps apply:

    - A single slice rsSlice of the resampled picture rsPic is generated as follows:

      - The values of slice_type, num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 for the generated slice rsSlice are inferred to be equal to the slice_type, num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1, respectively, of the first slice in rlPic

      - When rsSlice is a P or B slice, for i in the range of 0 to num_ref_idx_l0_active_minus1 of rsSlice, inclusive, the reference picture with index i in reference picture list 0 of rsSlice is set equal to reference picture with index i in reference picture list 0 of the first slice of  rlPic

      - When rsSlice is a B slice, for i in the range of 0 to num_ref_idx_l1_active_minus1 of rsSlice, inclusive, the reference picture with index i in reference picture list 1 of rsSlice is set equal to reference picture with index i in reference picture list 1 of the first slice of  rlPic

NOTE: When the resampled picture is used as collocated picture for temporal motion vector prediction, all slices of rlPic are constrained to have the same values of slice_type, num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1.

    - The picture motion field resampling process as specified in subclause Error: Reference source not found is invoked with reference layer picture rlPic and its motion field data including an array rlPredMode specifying the prediction modes of the reference layer picture, two arrays rlRefIdxLX specifying the reference indices of the reference layer picture, two arrays rlMvLX specifying the luma motion vectors of the reference layer picture and two arrays rlPredFlagLX specifying the prediction list utilization flags of the reference layer picture, with X = 0,1 as inputs, and with the motion field data of the resampled picture rsPic as output.

- H.8.1.4.1		Resampling process of picture sample values

Inputs to this process are:

- a samePictureSizeFlag

- a ( RefLayerPicWidthInSamplesY ) x ( RefLayerPicHeightInSamplesY ) array rlPicSampleL of luma samples,

–	a ( RefLayerPicWidthInSamplesC ) x ( RefLayerPicHeightInSamplesC ) array rlPicSampleCb of chroma samples of the component Cb,

–	a ( RefLayerPicWidthInSamplesC ) x ( RefLayerPicHeightInSamplesC ) array rlPicSampleCr of chroma samples of the component Cr.

Outputs of this process are:

–	a ( PicWidthInSamplesY ) x ( PicHeightInSamplesY ) array rsPicSampleL of luma samples,

–	a (  PicWidthInSamplesC ) x ( PicHeightInSamplesC ) array rsPicSampleCb of chroma samples of the component Cb,

–	a ( PicWidthInSamplesC) x ( PicHeightInSamplesC ) array rsPicSampleCr of chroma samples of the component Cr.

The luma sample array rsPicSampleL is derived by invoking the luma sample resampling process specified in subclause  with the samePictureSizeFlag and the reference luma sample array rlPicSampleL given as input.

The chroma sample array rsPicSampleCb of the chroma component Cb is derived by invoking the chroma sample resampling process specified in subclause  with the samePictureSizeFlag and the reference chroma sample array rlPicSampleCb given as input.

The chroma sample array rsPicSampleCr of the chroma component Cr is derived by invoking the chroma sample resampling process specified in subclause  with the samePictureSizeFlag and the reference sample array rlPicSampleCr given as input.

- H.8.1.4.1.1	Resampling process of luma sample values

Input to this process is the samePictureSizeFlag and the reference luma sample array rlPicSampleL.

Output of this process is the resampled luma sample array rsPicSampleL.

The variables leftStartL, rightEndL, topStartL, and bottomEndL are derived as follows:

leftStartL = ScaledRefLayerLeftOffset
rightEndL = PicWidthInSamplesY – ScaledRefLayerRightOffset
topStartL = ScaledRefLayerTopOffset
bottomEndL = PicHeightInSamplesY – ScaledRefLayerBottomOffset

The following steps are applied to derive the resampled luma sample array rsPicSampleL.

- if the samplePictreuSizeFlag is equal to 1, the luma samples rsPicSampleL[ xP ][ yP ] with ( xP = 0 ... PicWidthInSamplesY – 1, yP = 0 ... PicHeightInSamplesY – 1) are derived by invoking the luma sample scaling process specified in subclause  with rlPicSampleL[ xP ][ yP ] given as inputs and rsPicSampleL[ xP ][ yP ] as output.

- Otherwise, the luma samples rsPicSampleL [ xP ][ yP ] with ( xP = 0 ... PicWidthInSamplesY – 1, yP = 0 ... PicHeightInSamplesY – 1) are derived by invoking the luma sample interpolation process specified in subclause H.8.1.4.1.4 with rlPicSampleL and luma sample location ( Clip3( leftStartL, rightEndL – 1, xP ), Clip3( topStartL, bottomEndL – 1, yP ) ) given as inputs and rsPicSampleL[ xP ][ yP ] as output.

- H.8.1.4.1.2	Resampling process of chroma sample values

Input to this process is the samePictureSizeFlag and the  reference chroma sample array rlPicSampleC,

Output of this process is the resampled chroma sample array rsPicSampleC.

The variables leftStartC, rightEndC, topStartC, and bottomEndC are derived as follows:

leftStartC = ScaledRefLayerLeftOffset / SubWidthC
rightEndC = ( PicWidthInSamplesY– ScaledRefLayerRightOffset ) / SubWidthC
topStartC = ScaledRefLayerTopOffset / SubHeightC
bottomEndC = ( PicHeightInSamplesY– ScaledRefLayerBottomOffset ) / SubHeightC

The following steps are applied to derive the resampled chroma sample array rsPicSampleC.

- if the samplePictreuSizeFlag is equal to 1, the chroma samples rsPicSampleC[ xPC ][ yPC ] with ( xPC = 0 ... PicWidthInSamplesC – 1, yPC = 0 ... PicHeightInSamplesC – 1) are derived by invoking the chroma sample interpolation process specified in subclause H.8.1.4.1.5 with rlPicSampleC[ xPC ][ yPC ] as inputs and rsPicSampleC[ xPC ][ yPC ] as output.

- otherwise, the chroma samples rsPicSampleC[ xPC ][ yPC ] with ( xPC = 0 ... PicWidthInSamplesC – 1, yPC = 0 ... PicHeightInSamplesC – 1) are derived by invoking the chroma sample interpolation process specified in subclause H.8.1.4.1.6 with rlPicSampleC and chroma sample location ( Clip3( leftStartC, rightEndC - 1, xPC ), Clip3( topStartC, bottomEndC – 1, yPC ) ) given as inputs and rsPicSampleC[ xPC ][ yPC ] as output.

- H.8.1.4.1.3	Luma sample interpolation process with the samePictureSizeFlag equals 1

Inputs to this process is

- the luma reference sample array rlPicSampleL,

Output of this process is a interpolated luma sample value intLumaSample.

intLumaSample = rlPicSampleL  << (BitDepthY - RefLayerBitDepthY)					(H-xx)

- H.8.1.4.1.4	Luma sample interpolation process with the samePictureSizeFlag equals 0

Inputs to this process are

–	the luma reference sample array rlPicSampleL,

–	a luma sample location ( xP, yP ) relative to the top-left luma sample of the current picture.

Output of this process is a interpolated luma sample value intLumaSample.

- Table H-1 specifies the 8-tap filter coefficients fL[ p, x ] with p = 0 ... 15 and x = 0 ... 7 used for the luma resampling process.

Table H-1 – 16-phase luma resampling filter

| phase p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p, 0 ] | fL[ p, 1 ] | fL[ p, 2 ] | fL[ p, 3 ] | fL[ p, 4 ] | fL[ p, 5 ] | fL[ p, 6 ] | fL[ p, 7 ] |
| 0 | 0 | 0 | 0 | 64 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | −3 | 63 | 4 | −2 | 1 | 0 |
| 2 | −1 | 2 | −5 | 62 | 8 | −3 | 1 | 0 |
| 3 | −1 | 3 | −8 | 60 | 13 | −4 | 1 | 0 |
| 4 | −1 | 4 | −10 | 58 | 17 | −5 | 1 | 0 |
| 5 | −1 | 4 | −11 | 52 | 26 | −8 | 3 | −1 |
| 6 | −1 | 3 | −9 | 47 | 31 | −10 | 4 | −1 |
| 7 | −1 | 4 | −11 | 45 | 34 | −10 | 4 | −1 |
| 8 | −1 | 4 | −11 | 40 | 40 | −11 | 4 | −1 |
| 9 | −1 | 4 | −10 | 34 | 45 | −11 | 4 | −1 |
| 10 | −1 | 4 | −10 | 31 | 47 | −9 | 3 | −1 |
| 11 | −1 | 3 | −8 | 26 | 52 | −11 | 4 | −1 |
| 12 | 0 | 1 | −5 | 17 | 58 | −10 | 4 | −1 |
| 13 | 0 | 1 | −4 | 13 | 60 | −8 | 3 | −1 |
| 14 | 0 | 1 | −3 | 8 | 62 | −5 | 2 | −1 |
| 15 | 0 | 1 | −2 | 4 | 63 | −3 | 1 | 0 |

The value of the interpolated luma sample IntLumaSample  is derived by applying the following ordered steps:

- The derivation process for reference layer sample location used in resampling as specified in subclause Error: Reference source not found is invoked with cIdx equal to 0 and luma sample location ( xP, yP ) given as the inputs and ( xRef16, yRef16 ) in units of 1/16-th sample as output.

- The variables xRef and xPhase are derived as follows:

xRef     = ( xRef16 >> 4 )	(H-13)

xPhase = ( xRef16 ) % 16	(H-14)

- The variables yRef and yPhase are derived as follows:

yRef     = ( yRef16 >> 4 )	(H-15)

yPhase = ( yRef16 ) % 16	(H-16)

- The variables shift1, shift2 and offset are derived as follows:

shift1 = RefLayerBitDepthY – 8	(H-17)

shift2 = 20 – BitDepthY	(H-18)

offset = 1 << ( shift2 – 1)	(H-19)

- The sample value tempArray[ n ] with n = 0 … 7, is derived as follows:

yPosRL = Clip3( 0, RefLayerPicHeightInSamplesY – 1, yRef + n – 1 ) 	(H-20)

refW      = RefLayerPicWidthInSamplesY

tempArray[ n ] = ( fL[ xPhase, 0 ] * rlPicSampleL[ Clip3( 0, refW – 1, xRef – 3), yPosRL ] +
	fL[ xPhase, 1 ] * rlPicSampleL[ Clip3( 0, refW – 1, xRef – 2), yPosRL ] +
	fL[ xPhase, 2 ] * rlPicSampleL[ Clip3( 0, refW – 1, xRef – 1), yPosRL ] +
	fL[ xPhase, 3 ] * rlPicSampleL[ Clip3( 0, refW – 1, xRef      ), yPosRL ] +
	fL[ xPhase, 4 ] * rlPicSampleL[ Clip3( 0, refW – 1, xRef + 1), yPosRL ] +	(H-21)
	fL[ xPhase, 5 ] * rlPicSampleL[ Clip3( 0, refW – 1, xRef + 2), yPosRL ] +
	fL[ xPhase, 6 ] * rlPicSampleL[ Clip3( 0, refW – 1, xRef + 3), yPosRL ] +
	fL[ xPhase, 7 ] * rlPicSampleL[ Clip3( 0, refW – 1, xRef + 4), yPosRL ] ) >> shift1

- The interpolated luma sample value intLumaSample is derived as follows:

intLumaSample = ( fL[ yPhase, 0 ] * tempArray [ 0 ] +	
	fL[ yPhase, 1 ] * tempArray [ 1 ] +	
	fL[ yPhase, 2 ] * tempArray [ 2 ] +	
	fL[ yPhase, 3 ] * tempArray [ 3 ] +	
	fL[ yPhase, 4 ] * tempArray [ 4 ] +	(H-22)
	fL[ yPhase, 5 ] * tempArray [ 5 ] +	
	fL[ yPhase, 6 ] * tempArray [ 6 ] +	
	fL[ yPhase, 7 ] * tempArray [ 7 ] + offset ) >> shift2

intLumaSample = Clip3( 0, ( 1 << BitDepthY) – 1 , intLumaSample )	(H-23)

- H.8.1.4.1.5	Chroma sample interpolation process with the samePictureSizeFlag equals 1

Inputs to this process is:

–	the chroma reference sample array rlPicSampleC,

Output of this process is a interpolated chroma sample value intChromaSample:

intChromaSample = rlPicSampleC << (BitDepthC - RefLayerBitDepthC)	(H-xx)

- H.8.1.4.1.6	Chroma sample interpolation process  with samePictureSizeFlag equals 0

Inputs to this process are:

–	the chroma reference sample array rlPicSampleC,

–	a chroma sample location ( xPC, yPC ) relative to the top-left chorma sample of the current picture.

Output of this process is a interpolated chroma sample value intChromaSample.

- Table H-2 specifies the 4-tap filter coefficients fC[ p, x ] with p = 0 ... 15 and x = 0 ... 3 used for the chroma resampling process.

Table H-2 – 16-phase chroma resampling filter

| phase p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p, 0 ] | fC[ p, 1 ] | fC[ p, 2 ] | fC[ p, 3 ] |
| 0 | 0 | 64 | 0 | 0 |
| 1 | −2 | 62 | 4 | 0 |
| 2 | −2 | 58 | 10 | −2 |
| 3 | −4 | 56 | 14 | −2 |
| 4 | −4 | 54 | 16 | −2 |
| 5 | −6 | 52 | 20 | −2 |
| 6 | −6 | 46 | 28 | −4 |
| 7 | −4 | 42 | 30 | −4 |
| 8 | −4 | 36 | 36 | −4 |
| 9 | −4 | 30 | 42 | −4 |
| 10 | −4 | 28 | 46 | −6 |
| 11 | −2 | 20 | 52 | −6 |
| 12 | −2 | 16 | 54 | −4 |
| 13 | −2 | 14 | 56 | −4 |
| 14 | −2 | 10 | 58 | −2 |
| 15 | 0 | 4 | 62 | −2 |

The value of the interpolated chroma sample value intChromaSample is derived by applying the following ordered steps:

- The derivation process for reference layer sample location in resampling as specified in subclause Error: Reference source not foundError: Reference source not found is invoked with cIdx and chroma sample location ( xPC, yPC ) given as the inputs and ( xRef16, yRef16 ) in units of 1/16-th sample as output.

- The variables xRef and xPhase are derived as follows:

xRef     = ( xRef16 >> 4 )	(H-24)

xPhase = ( xRef16 ) % 16	(H-25)

- The variables yRef and yPhase are derived as follows:

yRef     = ( yRef16 >> 4 )	(H-26)

yPhase = ( yRef16 ) % 16	(H-27)

- The variables shift1, shift2 and offset are derived as follows:

shift1 = RefLayerBitDepthC – 8	(H-28)

shift2 = 20 – BitDepthC	(H-29)

offset =1 << ( shift2 – 1)	(H-30)

- The sample value tempArray[ n ] with n = 0 … 3, is derived as follows:

yPosRL = Clip3( 0 , RefLayerPicHeightInSamplesC – 1, yRef + n – 1 )	(H-31)

refWC   = RefLayerPicWidthInSamplesC	(H-32)

tempArray[ n ] = ( fC[ xPhase, 0 ] * rlPicSampleC[ Clip3( 0, refWC – 1, xRef – 1), yPosRL ] +
	fC[ xPhase, 1 ] * rlPicSampleC[ Clip3( 0, refWC – 1, xRef       ), yPosRL ] +
	fC[ xPhase, 2 ] * rlPicSampleC[ Clip3( 0, refWC – 1, xRef + 1 ), yPosRL ] +	(H-33)
	fC[ xPhase, 3 ] * rlPicSampleC[ Clip3( 0, refWC – 1, xRef + 2 ), yPosRL ]  ) >> shift1

- The interpolated chroma sample value intChromaSample is derived as follows:

intChromaSample = (fC[ yPhase, 0 ] * tempArray [ 0 ] +
	fC[ yPhase, 1 ] * tempArray [ 1 ] +
	fC[ yPhase, 2 ] * tempArray [ 2 ] +	(H-34)
	fC[ yPhase, 3 ] * tempArray [ 3 ] + offset ) >> shift2

intChromaSample = Clip3( 0, ( 1 << BitDepthC ) – 1 , intChromaSample )	(H-35)