| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0173 |
| --- | --- |

| Title: | AhG 5 and 18: Coding of high bit-depth source with lower bit-depth encoders and a continuity mapping |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung, Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 Cheung.Auyeung@am.sony.com jun.xu@am.sony.com |
| Source: | Sony Electronics Inc |  |  |

_____________________________

# Abstract

This contribution proposes a method to encode high bit-depth video by lower bit-depth encoders and a continuity transform. In this contribution, the high bit-depth source is divided into MSB part and LSB part. The MSB part is losslessly coded as a primary picture, and the LSB part is transformed by a continuity mapping and then encoded as an auxiliary picture. SEI message is proposed to combine the MSB and the LSB parts, based on the SEI message from JCTVC-O0090. The BDR spreadsheet from the common test conditions, JCTC-L11100, reported that the continuity mapping improved the coding efficiency of the 4 LSB bits of the 10 bit BT.709 test sequences from SCE1 with luma BDR equals to-52.9%, -94.7%, and -93.1% for all intra main, random access main, and low delay B main respectively.

# Introduction

JCTVC-O0090 [1] proposed a method to encode high bit-depth video by two lower bit-depth encoders. In JCTVC-O0090, the high bit-depth source is divided into MSB part and the LSB part. The MSB part is coded as a primary picture part and the LSB part is encoded as an auxiliary picture. It was noted in the meeting note JCTVC-O_Notes_d9 [2] that when the MSB and the LSB parts do not have overlap, the MSB part must be lossless for the LSB part to be useful. However, this contribution asserted that the LSB part appears like noise and it is difficult to compress.

This contribution asserted that when the MSB is lossless, the LSB needed to be transformed to improve continuity so that it can be coded more efficiently. Since the MSB is coded losslessly, the bit-depth of the MSB should be minimized. As shown by the example in Figure 1, to encode a 16 bits video source, the 16 bit samples are split into 4 bits MSB parts and 12 bits MSB parts. The MSB part is encoded losslessly and the LSB part is encoded by lossy means.

Figure 1: To encode a 16 bit source, a 16 bits sample is split into MSB and LSB parts without overlap. The MSB part is encoded losslessly. The LSB part is encoded by lossy means.

# Proposed Method

In this contribution, as shown in Figure 2, the source with n bits is divided into (n-m) bits MSB part and the m bits LSB part without overlap. The MSB part is packed into k1 bits to be encoded losslessly by a k1 bits encoder. The LSB part is transformed by a continuity mapping and then packed into k2 bits to be encoded by a k2 bits lossy encoder.

The continuity mapping is intended to transform the noise like characteristics of the LSB into a form that has more temporal and spatial correlation. Let x be the LSB part and b be the least significant bit of the MSB part. The output of the continuity mapping is defined as follows:

- If b is equal to 0, then y is set equal to x.

- Else, y is set equal to (2m -1- x).

The continuity mapping can also be computed without branches. Let xi and yi for i=0,..,m-1 are the i-th bits of x and y. Then the output of the continuity mapping is derived as follows:

- yi is set equal to the exclusive or of xi and b.

Figure 2: A high bit-depth source is encoded with two lower bit-depth encoders and a continuity mapping.

Figure 3: In the experiment, the test sequences are 10 bits from SCE1 [4]. The LSB part has 4 bits as in the example of JCTVC-O0090 without overlap.

# Experimental Results

This experiment determines the BDR of encoding the LSB parts with and without the continuity mapping on the five BT.709 10 bits test sequences distributed for the SCE1 [4]. Following the example in JCTVC-O0090 [1] without overlap, the LSB part is chosen to have 4 bits in this experiment.

For anchor, the lower 4 bits of the 10 bit test sequences are extracted and packed into the lower 4 bits of the 8 bits anchor input videos. The upper 4 bits of the anchor input videos are set to zero. Then the anchor input videos are encoded by HM-12.1 encoder.

For test, the lower 4 bits of the 10 bit test sequences are extracted and transformed by the continuity mapping and then packed into the lower 4 bits of the 8 bits test input videos. The upper 4 bits of the test input videos are set to zero. Then the test input videos are encoded by HM-12.1 encoder.

The BDR in Table 1 was computed by the spreadsheet from JCTVC-L1100 [5] with the five Class B test sequences replaced by the LSB parts of the five BT.709 10 bits test sequences from SCE1 [4]. The spreadsheet reported a luma BDR of -52.9%, -94.7%, and -93.1% for all intra main, random access main, and low delay B main respectively. Further examination of the spreadsheet indicated that the spreadsheet might have bugs and it was not able to compute the BDR meaningfully at these extreme conditions.

For all intra main test cases, the noise like characteristic of the anchor test sequences may have triggered some bugs in HM12.1 and it was not able to decode some of the bitstreams.

Figure 4: Y PSNR vs bitrate of (A) without the continuity mapping and (B) with the continuity mapping.

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class B | -52.9% | -69.3% | -66.5% |
|  | Random Access Main |  |  |
|  | Y | U | V |
| Class B | -94.7% | -59.5% | -79.0% |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class B | -93.1% | -98.4% | -77.6% |

Table 1: BDR of the continuity mapping.

# References

- Woo-Shik Kim, Wei Pu, Jianle Chen, Ye-Kui Wang, Joel Sole, Marta Karczewicz, “AhG 5 and 18: High Bit-depth Coding Using Auxiliary Picture”, JCTVC-O0090, 15th Meeting: Geneva, CH, 23 October – 1 November 2013.

- Gary Sullivan and Jens-Rainer Ohm, “Meeting report of the 15th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Geneva, CH, 23 Oct. – 1 Nov. 2013”, JCTVC-O_Notes_d9, 15th Meeting: Geneva, CH, 23 October – 1 November 2013.

- M. Hannuksela, “REXT/MV-HEVC/SHVC HLS: auxiliary picture layers”, JCTVC-O0041, 15th Meeting: Geneva, CH, 23 October – 1 November 2013.

- Philippe Bordes,Yan Ye,Elena Alshina, Xiang Li, Seung-Hwan Kim, Alberto Duenas, Kemal Ugur, Kazushi Sato, “Description of HEVC Scalable Extensions Core Experiment SCE1: Color Gamut and Bit-Depth Scalability”, JCTVC-O1101, 15th Meeting: Geneva, CH, 23 October – 1 November 2013.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-L1100, 12th Meeting: Geneva, CH 14 – 23 January 2013.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed Text Changes

The proposed text is derived from JCTVC-O090 [1] with differences highlighted in green. Based on [1], the proposed AuxId is added to the table proposed in JCTVC-O0041 [3] highlighted in yellow.

Table Error: Reference source not found-2 – Mapping of AuxId to the type of auxiliary pictures

| AuxId | Name of AuxId | Type of auxiliary pictures |
| --- | --- | --- |
| 0 | - | Not an auxiliary picture |
| 1 | AUX_ALPHA | Alpha plane |
| 2 | AUX_DEPTH | Depth picture |
| 3 | AUX_CB | Monochrome picture representing a Cb sample array that enhances the Cb sample array of the associated primary picture |
| 4 | AUX_CR | Monochrome picture representing a Cr sample array that enhances the Cr sample array of the associated primary picture |
| 5 | AUX_HBD | A picture with the LSBs to provide high precision to the associated primary picture |
| 6-255 |  | Reserved |

High bit-depth coding SEI message syntax

| hbd_coding( payloadSize ) { | Descriptor |
| --- | --- |
| output_bit_depth_luma_minus9 | u(3) |
| output_bit_depth_chroma_minus9 | u(3) |
| msb_bit_depth_luma_minus1 | u(3) |
| msb_bit_depth_chroma_minus1 | u(3) |
| lsb_bit_depth_luma_minus8 | u(3) |
| lsb_bit_depth_chroma_minus8 | u(3) |
| } |  |

High bit-depth coding SEI message semantics

This SEI message provides information to combine the primary picture and the auxiliary picture with AuxId equal to AUX_HBD to provide higher precision to the primary picture. The primary picture shall be coded without loss.

output_bit_depth_luma_minus9 plus 9 specifies the bit depth of the luma component of the output data to be generated by combining the primary picture and the auxiliary picture.

The value of OutputBitDepthY is derived as follows:

OutputBitDepthY = output_bit_depth_luma_minus9 + 9

output_bit_depth_chroma_minus9 plus 9 specifies the bit depth of the chroma components of the output data to be generated by combining the primary picture and the auxiliary picture.

The value of OutputBitDepthC is derived as follows:

OutputBitDepthC = output_bit_depth_chroma_minus9 + 9

msb_bit_depth_luma_minus1 plus 1 specifies the bit depth of the luma component of the MSB part to be generated from the primary picture.

The value of MsbBitDepthY is derived as follows:

MsbBitDepthY = msb_bit_depth_luma_minus1 + 1

msb_bit_depth_chroma_minus1 plus 1 specifies the bit depth of the chroma components of the MSB part to be generated from the primary picture.

The value of MsbBitDepthC is derived as follows:

MsbBitDepthC = msb_bit_depth_chroma_minus1 + 1

lsb_bit_depth_luma_minus8 plus 8 specifies the bit depth of the luma component of the LSB part to be generated from the auxiliary picture.

The value of LsbBitDepthY is derived as follows:

LsbBitDepthY = lsb_bit_depth_luma_minus8 + 8

lsb_bit_depth_chroma_minus8 plus 8 specifies the bit depth of the chroma components of the LSB part to be generated from the auxiliary picture.

The value of LsbBitDepthC is derived as follows:

LsbBitDepthC = lsb_bit_depth_chroma_minus8 + 8

If BitDepthY of the primary picture is larger than MsbBitDepthY, each luma sample of the primary picture is right-shifted by (BitDepthY – MsbBitDepthY).

If BitDepthC of the primary picture is larger than MsbBitDepthC, each chroma sample of the primary picture is right-shifted by (BitDepthC – MsbBitDepthC).

If BitDepthY of the auxiliary picture is larger than LsbBitDepthY, each luma sample of the auxiliary picture is right-shifted by (BitDepthY – LsbBitDepthY).

If BitDepthC of the auxiliary picture is larger than LsbBitDepthC, each chroma sample of the auxiliary picture is right-shifted by (BitDepthC – LsbBitDepthC).

If (MsbBitDepthY + LsbBitDepthY) is larger than OutputBitDepthY, each luma sample of the primary picture is right-shifted by (MsbBitDepthY + LsbBitDepthY – OutputBitDepthY).

If (MsbBitDepthC + LsbBitDepthC) is larger than OutputBitDepthC, each chroma sample of the primary picture is right-shifted by (MsbBitDepthC + LsbBitDepthC – OutputBitDepthC).

Each luma sample of the output data, OutputSampleY, is derived from the luma sample of the primary picture, PrimarySampleY, the luma sample of the auxiliary picture, AuxiliarySampleY, as follows:

- TransAuxSampleY = Clip3( 0, ( 1 << LsbBitDepthY ) − 1, AuxiliarySampleY )

- TransAuxSampleY = (PrimarySampleY & 1) ?

(1 << LsbBitDepthY) – 1 – TransAuxSampleY : TransAuxSampleY

- OutputSampleY = (PrimarySampleY << LsbBitDepthY) + TransAuxSampleY

Each chroma sample of the output data, OutputSampleC, is derived from the chroma sample of the primary picture, PrimarySampleC, the chroma sample of the auxiliary picture, AuxiliarySampleC, as follows:

- TransAuxSampleC = Clip3( 0, ( 1 << LsbBitDepthC ) − 1, AuxiliarySampleC )

- TransAuxSampleC = (PrimarySampleC & 1) ?

(1 << LsbBitDepthC) – 1 – TransAuxSampleC : TransAuxSampleC

- OutputSampleC = (PrimarySampleC << LsbBitDepthC) + TransAuxSampleC