| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0177-r1 |
| --- | --- |

| Title: | On handling re-sampling phase offsets with fixed filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo, David Baylon, Ajay Luthra 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2521 koohyar.minoo@arrisi.com david.baylon@arrisi.com ajay.luthra@arrisi.com |
| Source: | ARRIS |  |  |

_____________________________

# Abstract

This contribution discusses an approach to signaling phase offsets to improve inter-layer prediction precision and hence the compression performance of SHEVC. The proposed method uses phase offset per phase index per direction and per color type to correct the upsampling behaviour. This information is signalled at PPS so alternative phase correction can be applied per slice or picture, which benefits the use case of upsampling from field to frame.

# Introduction

In order to provide good interlayer prediction, it is important that the BL upsampling process compensate for the downsampling operation in generation of the BL.  However, due to the fixed precision arithmetic in the computation of the filter index and to the limited number of filter phases available using a fixed filter set, errors in the selecting the best filter phase offset can occur.  It was observed that filter index errors can occur with the current fixed precision arithmetic in SHM 4.0 as the picture resolution increases, e.g. for 4k resolutions.  The next section discusses an approach to re-map filter indices to select the appropriate phase offset per filter, direction, and color component.

# Problems with upsampling process of current SEVC (HM4.0)

In the current revision of SHEVC WD (and HM4.0), four problems exist related to upsampling:

## Inaccurate phase offset due to limited precision phase calculation.

Currently the phase offset derivation is conducted by division free operations with 12 bits accuracy ( on a grid 1/16th-pel). This introduces an error of up to 2-13 which can result in producing a wrong phase location.  As an example for the case of 1.5x, the error is 1/3*2^-12 and the accurate phase offsets are at 0, 1/3 and 2/3 pel locations. So the wrong phase calculation starts at the n-th pixel where for that pixel the corresponding BL phase at 1/3 which is phase index 5 in SHEVC, moves to phase index 6 (which yields 3/8th pel):

n * (1/3 * 1/4096)=(1/2 - 1/3) or n=2048

Simulation results show a BDR loss of about 0.3% for the current SHM for PeopleOnStreet when the EL is 3840x2160 and the BL is 2560x1440 produced by the Downconverter in HM4.0 with zero-phase and 0.9pi cutoff).

| Coding Condition | Y | U | V |
| --- | --- | --- | --- |
| All Intra | -0.3% | -0.3% | -0.3% |
| Random Access | -0.3% | -0.2% | -0.2% |
| Low delay | -0.3% | -0.2% | -0.2% |
| Low delay P | -0.3% | -0.2% | -0.2% |

This behavior can be fixed by

- Increasing the precision

- Reverting to the division-base phase derivation

- Signaling phase mapping (this contribution)

## Inconsistancy between phase-calculation and phase realization

The phase realization of interpolation filters (due to support of accurate phase for the specific case of 1.5x with zero phase alignment) are not uniformly spaced, while the phase derivation assumes a uniform grid of phase locations.  As a consequence, a filter may be selected which has a sub-optimal phase, thereby degrading performance.  For example currently the phase 9/32 (4.5/16) would be realized by 5/16 filter which has a phase of 5.33/16.  In this case, using a 4/16 phase would be more appropriate.

One way to correct this behavior is to re-map the filter phase indices to the correct index, as is proposed here

## Lack of Chroma phase compensation alignment between EL and BL for all possible Chroma locations

Due to all the possible chroma locations between EL and BL, for example in 4:2:0 formats, there is a need to be able to apply the correct phase offset between layers per direction.  A general approach using phase index re-mapping described here can be applied for the different chroma locations.

## Lack of phase compensation alignment among fields and frames

In the problem of scalability from field data to frame data, it is important to compensate for the difference in pixel positions between the top and bottom fields in the interpolation to frame data.  Instead of signaling specific top/bottom field syntax to indicate and compensate for this, the proposed phase index re-mapping can achieve this without reference to an interlace format.

Instead of specific different fixes for each of the problems discussed above, the next section describes a single approach that addresses all these problems.  This approach is an extension of the work described in [1].

# Proposed method of fixed filters with phase index re-mapping

As described in the previous section for the case of fixed filters, in addition to the phase offset error due to fixed phase precision, there are also errors that can result from fixed precision arithmetic in computing the best phase index for the upsampling filter.  One possibility for correcting for phase offsets per filter is to re-map the filter index.  One example of this for the case of 16 filters is shown in Table 1, where the signaling (highlighted) occurs at the PPS level, and the following syntax elements are signaled.

luma_filter_offset[ i ][ j ] specifies the offset value used to modify the luma filter index j in the upsampling process along direction i.  This is a signed value between -15 to +15 (given a scaled grid size of 16x).

chroma_filter_offset[ i ][ j ] specifies the offset value used to modify the chroma filter index j in the upsampling process along direction i.  This is a signed value between -15 to +15 (given a scaled grid size of 16x).

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| for( i = 0; i < 2; i++ ) { |  |
| for( j = 0; j < 16; j++ ) { |  |
| luma_filter_offset[ i ][ j ] | se(v) |
| chroma_filter_offset[ i ][ j ] | se(v) |
| } |  |
| } |  |
| . . . |  |
| } |  |

Table 1.  Syntax for signaling of filter offsets

Corresponding modifications to semantics in the draft SHVC text  are indicated below, where deletions are crossed out and additions are highlighted in green.

H.6.2	Derivation process for reference layer sample location used in resampling

Inputs to this process are

–	a variable cIdx specifying the color component index,

–	a sample location ( xP, yP ) relative to the top-left sample of the color component of the current picture specified by cIdx.

Output of this process is a sample location ( xRef16, yRef16 ) specifying the reference layer sample location in units of 1/16-th sample relative to the top-left sample of the reference layer picture.

The variables offsetX and offsetY are derived as follows:

offsetX = ScaledRefLayerLeftOffset / ( ( cIdx = = 0)  ?  1 :  SubWidthC)	(H-1)
offsetY = ScaledRefLayerTopOffset / ( ( cIdx = = 0)  ?  1 :  SubHeightC)	(H-2)

The variables phaseX, phaseY, addX and addY are derived as follows:

phaseX = ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag	(H-3)
phaseY = ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag + 1					(H-4)

addX = ( ScaleFactorX * phaseX + 2 ) >> 2		(H-5) 
addY = ( ScaleFactorY * phaseY + 2 ) >> 2		(H-6)

The variables xRef16 and yRef16 are derived as follows:

xRef16 = ( ( ( xP – offsetX ) * ScaleFactorX  + addX + ( 1 << 11 ) ) >> 12 ) – ( phaseX << 2 )	(H-7)
yRef16 = ( ( ( yP – offsetY ) * ScaleFactorY + addY + ( 1 << 11 ) ) >> 12 ) – ( phaseY << 2 )	(H-8)

The variables phase_map[dir][phase] and pixel_shift[dir][phase] are defined as follows (dir == 0 indicates horizontal and dir==1 indicate vertical directions and phase is a number from 0 to 15).

phase_map[dir][phase]=(((cIdx = = 0) ?  luma_filter_offset[dir][phase]:chroma_filter_offset[dir][phase])+phase)%16

pixel_shift[dir][phase]=(((cIdx = = 0) ?  luma_filter_offset[dir][phase]:chroma_filter_offset[dir][phase])+phase)>>4

- H.8.1.4.1.3 Luma sample interpolation process

Inputs to this process are

–	the luma reference sample array rlPicSampleL,

–	a luma sample location ( xP, yP ) relative to the top-left luma sample of the current picture.

Output of this process is a interpolated luma sample value intLumaSample.

- Table H-1 specifies the 8-tap filter coefficients fL[ p, x ] with p = 0 ... 15 and x = 0 ... 7 used for the luma resampling process.

Table H-1 – 16-phase luma resampling filter

| Phase_map[dir][phase p] | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p, 0 ] | fL[ p, 1 ] | fL[ p, 2 ] | fL[ p, 3 ] | fL[ p, 4 ] | fL[ p, 5 ] | fL[ p, 6 ] | fL[ p, 7 ] |
| 0 | 0 | 0 | 0 | 64 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | −3 | 63 | 4 | −2 | 1 | 0 |
| 2 | −1 | 2 | −5 | 62 | 8 | −3 | 1 | 0 |
| 3 | −1 | 3 | −8 | 60 | 13 | −4 | 1 | 0 |
| 4 | −1 | 4 | −10 | 58 | 17 | −5 | 1 | 0 |
| 5 | −1 | 4 | −11 | 52 | 26 | −8 | 3 | −1 |
| 6 | −1 | 3 | −9 | 47 | 31 | −10 | 4 | −1 |
| 7 | −1 | 4 | −11 | 45 | 34 | −10 | 4 | −1 |
| 8 | −1 | 4 | −11 | 40 | 40 | −11 | 4 | −1 |
| 9 | −1 | 4 | −10 | 34 | 45 | −11 | 4 | −1 |
| 10 | −1 | 4 | −10 | 31 | 47 | −9 | 3 | −1 |
| 11 | −1 | 3 | −8 | 26 | 52 | −11 | 4 | −1 |
| 12 | 0 | 1 | −5 | 17 | 58 | −10 | 4 | −1 |
| 13 | 0 | 1 | −4 | 13 | 60 | −8 | 3 | −1 |
| 14 | 0 | 1 | −3 | 8 | 62 | −5 | 2 | −1 |
| 15 | 0 | 1 | −2 | 4 | 63 | −3 | 1 | 0 |

The value of the interpolated luma sample IntLumaSample  is derived by applying the following ordered steps:

- The derivation process for reference layer sample location used in resampling as specified in subclause  is invoked with cIdx equal to 0 and luma sample location ( xP, yP ) given as the inputs and ( xRef16, yRef16 ) in units of 1/16-th sample as output.

- The variables xRef and xPhase are derived as follows:

xRef     = ( xRef16 >> 4 )	(H-9)

xPhase = ( xRef16 ) % 16	(H-10)

xRef     = ( xRef16 >> 4 ) + pixel_shift[0][ xPhase]	(H-11)

- The variables yRef and yPhase are derived as follows:

yRef     = ( yRef16 >> 4 )	(H-12)

yPhase = ( yRef16 ) % 16   	 (H-13)

yRef     = ( yRef16 >> 4 ) + pixel_shift[1][ yPhase]	(H-14)

## Additional benefits of proposed approach

Some benefits of the proposed approach include:

- Independent and arbitrary phase offsets per filter and color are allowed in each dimension.

- Better matching of filters and phases to scalability ratios other than 2x, 1.5x.

- Support of interlaced field to progressive frame [2]

- Unified approach to correct for phase offset errors in the computation and adaptive selection of the phase index for fixed filters.

# References

[1]  K. Minoo, D. Baylon, A. Luthra, “Non-SCE1: On handling re-sampling phase offsets with fixed filters”, JCTVC-N0272, 14th JCT-VC Meeting, Vienna, AT, July 2013.

[2]  K. Minoo, D. Baylon, A. Luthra, “On field to frame scalability”, JCTVC-P0175, 16th JCT-VC Meeting, San Jose, USA, January 2014.

- Patent rights declaration(s)

ARRIS Group, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).