| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0178 |
| --- | --- |

| Title: | Non-RCE3: Bv coding with default predictor |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu, Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA Ohji Nakagami Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com ali.tabatabai@am.sony.com +81-50-3750-2746 ohji.nakagami@jp.sony.com |
| Source: | Sony Electronics Inc. Sony Corporation |  |  |

_____________________________

# Abstract

Modification for the default predictor for Bv of Intra Block copying is proposed. This proposal has been evaluated under CTC of RExt and smaller CTU sizes. Comparing to RExt5.1, simulation results report bit-rate saving up to -0.2% for AI, -0.2% for RA, and -0.5% for LD under lossy settings and up to -0.1% for AI, -0.1% for RA, and -0.1% for LD under lossless settings for CTC. When CTU size is set to 32, bit-rate saving up to -0.4% for AI, -0.7% for RA, and -1.0% for LD under lossy settings and up to -0.1% for AI, -0.2% for RA, and -0.3% for LD under lossless settings. When CTU size is set to 16, bit-rate saving up to -1.0% for AI, -0.7% for RA, and -0.5% for LD under lossy settings and up to -0.3% for AI, -0.3% for RA, and -0.5% for LD under lossless settings.

# Introduction

In RExt5.0, Bv is predicatively coded. When a previously coded Bv within the same CTU exists, the Bv serves as the predictor; otherwise (the first CU with Intra BC mode), (0, 0) is the default Bv predictor.  When (0, 0) is the predictor, it is actually coding Bv directly. Bv coding efficiency can be further improved by exploring the redundancy.

# Solution

The proposed solution is to modify the Bv coding for the first CU with Intra Block copying mode. The detailed solution is as follows.

For the first CU with Intra Block copying mode in a CTU, if the vertical component block vector Bvy > - H, the default Bv predictor is (-W, 0), where H is the height and W is the width of CU; otherwise, the default Bv predictor is still (0, 0).

# Simulation results

The proposed solution is implemented based on RExt5.1. Simulations are conducted following the test conditions used for RCE3. Encoding and decoding time is not available because simulations are run in a heterogeneous cluster.

## Results under CTC

Summaries of results for both lossy and lossless settings are listed in Error: Reference source not found and Error: Reference source not found. The anchor is the results of RExt5.1. Detailed results are available in the accompanying excel files.

Table 1 Summary of results for lossy settings under CTC

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 Animation | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| YCbCr 4:4:4 SC (Optional) | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -0.2% | -0.1% | 0.0% | -0.2% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| RGB 4:4:4 SC | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 Animation | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| RangeExt | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.1% | 0.0% | 0.0% | -0.2% | -0.1% | -0.1% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -0.2% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.1% | -0.3% | 0.2% | 0.1% | -0.1% | -0.1% |  |  |  |
| Class B | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.3% | -0.1% | -0.3% | -0.2% | -0.1% | -0.2% |  |  |  |
| YCbCr 4:4:4 Animation | 0.1% | 0.2% | 0.0% | 0.1% | 0.2% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.3% | -0.2% | -0.3% | -0.5% | -0.3% | -0.4% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -0.5% | -0.4% | -0.4% | -0.3% | -0.3% | -0.4% |  |  |  |

Table 2 Summary of results for lossless settings under CTC

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 0.0% | 0.2% | 0.0% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 0.0% | -0.1% | -0.1% |
| YCbCr 4:4:4 SC (Optional) | -0.1% | 0.0% | -0.1% |

## Results with CTU=32x32

In this setting, CTU size is set to 32x32. Summaries of results for both lossy and lossless settings are listed in Table 3 and Table 4. The anchor is the results of RExt5.1 with CTU size 32x32. Detailed results are available in the accompanying excel files.

Table 3 Summary of results for lossy settings under CTU=32x32

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% |
| RGB 4:4:4 Animation | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.2% | -0.2% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -0.4% | -0.4% | -0.4% | -0.4% | -0.4% | -0.3% | -0.3% | -0.4% | -0.2% |
| YCbCr 4:4:4 SC (Optional) | -0.4% | -0.3% | -0.5% | -0.3% | -0.3% | -0.4% | -0.3% | -0.3% | -0.3% |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class B | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| RGB 4:4:4 SC | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% |  |  |  |
| RGB 4:4:4 Animation | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.1% | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 Animation | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.5% | -0.5% | -0.6% | -0.5% | -0.5% | -0.6% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -0.5% | -0.3% | -0.4% | -0.7% | -0.6% | -0.6% |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | 0.1% | -1.2% | -0.3% | -0.3% | -0.9% |  |  |  |
| Class B | 0.0% | -0.3% | 0.3% | 0.0% | -0.2% | 0.1% |  |  |  |
| RGB 4:4:4 SC | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.2% | -0.2% | -0.5% | -0.2% | -0.1% | -0.3% |  |  |  |
| YCbCr 4:4:4 Animation | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.8% | -0.9% | -0.6% | -1.0% | -0.9% | -0.7% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -0.8% | -0.7% | -0.7% | -0.8% | -0.6% | -0.9% |  |  |  |

Table 4 Summary of results for lossless settings under CTU=32x32

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.0% | -0.1% | 0.0% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.1% | 0.2% | -0.1% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -0.1% | -0.1% | -0.3% |
| YCbCr 4:4:4 SC (Optional) | -0.1% | -0.2% | -0.2% |

## Results with CTU=16x16

In this setting, CTU size is set to 16x16. Summaries of results for both lossy and lossless settings are listed in Table 5 and Table 6. The anchor is the results of RExt5.1 with CTU size 16x16. Detailed results are available in the accompanying excel files.

Table 5 Summary of results for lossy settings under CTU=16x16

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.3% | -0.2% | -0.3% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% |
| Class B | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.6% | -0.7% | -0.6% | -0.5% | -0.5% | -0.5% | -0.4% | -0.4% | -0.4% |
| RGB 4:4:4 Animation | -0.3% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| YCbCr 4:4:4 SC | -0.6% | -0.8% | -0.7% | -0.5% | -0.6% | -0.5% | -0.4% | -0.4% | -0.5% |
| YCbCr 4:4:4 Animation | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -1.0% | -1.0% | -1.1% | -1.0% | -1.0% | -1.0% | -1.0% | -0.8% | -0.9% |
| YCbCr 4:4:4 SC (Optional) | -0.9% | -0.8% | -0.9% | -0.8% | -0.7% | -0.8% | -0.6% | -0.6% | -0.6% |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -0.1% | 0.1% | -0.2% | -0.2% | 0.1% |  |  |  |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.1% |  |  |  |
| RGB 4:4:4 SC | -0.6% | -0.6% | -0.6% | -0.6% | -0.5% | -0.6% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.5% | -0.5% | -0.5% | -0.4% | -0.5% | -0.5% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.9% | -0.9% | -0.8% | -0.7% | -0.7% | -0.7% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -0.5% | -0.4% | -0.6% | -0.6% | -0.6% | -0.7% |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -1.4% | -0.4% | -0.1% | -0.9% | -0.5% |  |  |  |
| Class B | 0.0% | -0.9% | 0.3% | 0.0% | -0.3% | 0.2% |  |  |  |
| RGB 4:4:4 SC | -0.4% | -0.1% | -0.2% | -0.2% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 Animation | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.7% | 0.1% | -0.5% | -0.6% | -0.2% | -0.4% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | 0.1% | -0.1% | 0.0% | -0.1% |  |  |  |
| RangeExt | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.4% | -0.2% | -0.4% | -0.1% | 0.1% | 0.1% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -0.1% | 0.0% | 0.1% | -0.1% | 0.1% | 0.0% |  |  |  |

Table 6 Summary of results for lossless settings under CTU=16x16

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.1% | -0.1% | -0.1% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.2% | 0.2% | 0.0% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -0.2% | -0.1% | -0.5% |
| YCbCr 4:4:4 SC (Optional) | -0.3% | -0.3% | 0.1% |

# Text changes

The proposed text changes on top of JCTVC-O1005_v3.doc are listed in highlight as follows.

The block vector bvIntra is derived by the following ordered steps, for the variable compIdx proceeding over the values 0..1:

- Depending upon the number of times this process has been invoked for the current coding tree unit, the following applies:

- If this process is invoked for the first time for the current coding tree unit, bvIntra[ compIdx ] is derived as follows:

bvIntra[ compIdx  ] = BvdIntra[ xCb ][ yCb ][ compIdx  ]				(8-1)

if bvIntra[ 1 ] is greater than -nCbS

bvIntra[ 0  ]	=   bvIntra[ 0  ]  - nCbS

- Otherwise, bvIntra[ compIdx ] is derived as follows:

bvIntra[ compIdx ] = BvdIntra[ xCb ][ yCb ][ compIdx ] + BvpIntra[ compIdx ]	(8-2)

- The value of BvpIntra[ compIdx ] is updated to be equal to bvIntra[ compIdx ].

# Conclusion

Bv predictor for the first CU with IntraBC mode is modified. Coding efficiency is improved. It’s recommended to adopt this change to RExt6.0.

# Reference

- J. Sole, E. Alshina, D.-K. Kwon, W.-H. Peng,” HEVC Range Extensions Core Experiment 3 (RCE3): Intra block copy refinement”, JCTVC-O1123, Geneva, CH, Oct-Nov 2013.

# Patent rights declaration(s)

Sony Corporation and Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).