| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0181 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0159 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On Picture Parameter Set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yan Ye 9710 Scranton Road, #250 San Diego, CA 92121 USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications, Inc |  |  |

_____________________________

# Abstract

This contribution proposes to constrain the values of nuh_layer_id, TemporalId and pps_pic_parameter_set_id of picture parameter set NAL units to ensure that picture parameter set can be uniquely identified during PPS activation process.

# Introduction

The syntax elements, nuh_layer_id, nuh_temporal_id_plus1 and pps_pic_parameter_set_id are signaled for each PPS NAL unit, representing the value of layer id, temporal id and PPS id respectively.

In current SHVC specification, a PPS is activated when it is referred to by a coded slice segment NAL unit using a value of slice_pic_parameter_set_id equal to the value of pps_pic_parameter_set_id of the PPS. However, there could be multiple PPSs with identical pps_pic_parameter_set_id value but different values of nuh_layer_id and/or different values of nuh_temporal_id_plus1.

Figure 1 is an example of the PPS activation process with 3 different PPSs, each with identical pps_pic_parameter_id, but different values of nuh_layer_id and/or TemporalId. When a coded slice NAL unit refers to the PPS by setting slice_pic_parameter_set_id equal to 0, the following is possible:

- The PPS with nuh_layer_id equal to 0 and TemporalId equal to 0 can be applied to all coded slices, including base layer slices, enhancement layer temporal sub-layer tId0 slices and enhancement layer temporal sub-layer tId1 slices;

- The PPS with nuh_layer_id equal to 1 and TemporalId equal to 0 can be applied to both temporal sub-layer tId0 and tId 1 slices in the enhancement layer with nuh_layer_id equal to 1, but not to the coded slices with nuh_layer_id equal to 0;

- The PPS with nuh_layer_id equal to 1 and TemporalId equal to 1 can only be applied to enhancement layer temporal sub-layer tId1 slices with nuh_layer_id equal to 1.

As such, PPS cannot always be uniquely identified. Since each PPS can contain different syntax elements values, not being able to uniquely identify the PPS to be activated could cause decoding problems.

- PPS activation example

In order to solve the above problem, two options are proposed below.

## Option 1

In option #1, a bitstream constraint is added as follows:

Any PPS NAL unit containing the same value of pps_pic_parameter_set_id shall have identical nuh_layer_id value and identical nuh_temporal_id_plus1 value.

## Option 2

An alternative way to address the same issue but allow PPS with the same pps_pic_parameter_set_id to have different nuh_temporal_id_plus1 and nuh_layer_id value is to modify the PPS activation process as follows:

When a PPS RBSP (with a particular value of pps_pic_parameter_set_id, a particular value of TemporalId and a particular value of nuh_layer_id) is not active for a particular layer and it is referred to by a coded slice segment NAL unit (using a value of slice_pic_parameter_set_id equal to the pps_pic_parameter_set_id value, and a value of TemporalId equal to tIdA,_and a value of nuh_layer_id equal to nuhLayerIdA) of the particular layer, it is activated for the particular layer. This PPS RBSP is called the active PPS RBSP for the particular layer until it is deactivated by the activation of another PPS RBSP for the particular layer.

The active PPS can be derived from the following pseudo code:

for (i = nuhLayerIdA; i >= 0; i--) {

for (j = tIdA; j > =0; j--) {

if (slice_pic_parameter_set_id == pps_pic_parameter_set_id) {

PPS[i][j] is activated;

break;

}

}

}

where PPS[i][j] is the PPS with its parameter set identifier equal to pps_pic_parameter_set_id, its nuh_layer_id value equal to i and its TemporalId value equal to j.

Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).