| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0183 |
| --- | --- |

| Title: | AHG9: On AVC independent non-base layer indicator |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yan Ye 9710 Scranton Road, #250 San Diego, CA 92121 USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications, Inc |  |  |

_____________________________

# Abstract

This contribution proposes to expand the current avc_base_layer_flag to allow independent non-base layers to be coded in AVC.

# Introduction

The syntax element, avc_base_layer_flag, is signaled in the VPS extension to specify if the base layer bitstream conforms to AVC or HEVC. Current specification also allows multiple independent non-base layers to be present in the SHVC bitstream. This contribution proposes to expand avc_base_layer_flag such that any independent non-base layer bitstream is also allowed to be coded in AVC, as shown in Figure 1. The AVC layer with nuh_layer_id equal to 2 and the HEVC layer  with nuh_layer_id equal to 3 may be further extracted as an sub-bitstream with AVC base layer.

- bitstream example of an independent non-base AVC layer.

# AVC layer indication

The current syntax element avc_base_layer_flag is not sufficient to indicate an independent AVC coded non-base layer. Hence, it is proposed to expand it as shown in Table 1.

- Proposed AVC layer indication

| vps_extension( ) { | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| vps_vui_present_flag | u(1) |
| if( vps_vui_present_flag ) |  |
| vps_vui_offset | u(16) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= MaxLayersMinus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| view_id_len | u(4) |
| if( view_id_len > 0 ) |  |
| for( i = 0; i < NumViews; i++ ) |  |
| view_id_val[ i ] | u(v) |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| for ( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| if (NumDirectRefLayers[layer_id_in_nuh[i]] == 0) |  |
| avc_layer_flag[i] | u(1) |
| ... |  |
| } |  |

avc_layer_flag[i] equal to 1 specifies that the layer with nuh_layer_id equal to layer_id_in_nuh[i] conforms to Rec.ITU-T H.264 | ISO/IEC 14496-10. avc_layer_flag equal to 0 specifies that the layer with nuh_layer_id equal to layer_id_in_nuh[i] conforms to this Specification. When avc_layer_flag is not present, it is inferred to be 0.

When avc_layer_flag[i] is equal to 1, in the Rec. ITU-T H.264 | ISO/IEC 14496-10 conforming layer, after applying the Rec. ITU-T H.264 | ISO/IEC 14496-10 decoding process for reference picture lists construction the output reference picture lists refPicList0 and refPicList1 (when applicable) does not contain any pictures for which the TemporalId is greater than TemporalId of the coded picture. All sub-bitstreams of the Rec. ITU-T H.264 | ISO/IEC 14496-10 conforming layer, that can be derived using the sub-bitstream extraction process as specified in Rec. ITUT H.264 | ISO/IEC 14496-10 subclause G.8.8.1 with any value for temporal_id as the input shall result in a set of CVSs, with each CVS conforming to one or more of the profiles specified in Rec. ITUT H.264 | ISO/IEC 14496-10 Annexes A, G and H.

When avc_layer_flag[i] is equal to 1, it is a requirement of bitstream conformance that the value of sps_scaling_list_ref_layer_id of the active SPS for the layer with nuh_layer_id equal to layer_id_in_nuh[i] shall not be equal to layer_id_in_nuh[i].

When avc_layer_flag[i] is equal to 1, it is a requirement of bitstream conformance that the value of pps_scaling_list_ref_layer_id of the active PPS for the layer with nuh_layer_id equal to layer_id_in_nuh[i] shall not be equal to layer_id_in_nuh[i].

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).