| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San Jose, US, 9th – 17th, Jan. 2014 | Document: JCTVC-P0190 |
| --- | --- |

| Title: | Non-RCE3: On padding of overlapped area for IntraBC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Xiaozhong Xu2, Jicheng An1, Xianguo Zhang1, Shan Liu2, Shawmin Lei1 1 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek USA 2860 Junction Ave San Jose, CA 95134, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 { kai.zhang, xiaozhong.xu, jicheng.an, xianguo.zhang, liu.shan, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current HEVC range extensions draft specification, Intra block copying (Intra BC) was adopted to take advantage of reduplicated content in a picture. In Intra BC mode, the samples in the reference block must have been reconstructed already. Thus the reference block cannot be overlapped with the current block. Two padding methods are proposed in this contribution to remove such a constraint. An unreconstructed sample in the reference block is estimated by its prediction. Experimental results reportedly show that the proposed method A can achieve 0.9%, 0.8%, 1.5% and 1.6% BD-rate reduction respectively for RGB SC, YUV SC, optional RGB SC, and optional YUV SC sequences in AI HE Main-Tier configurations; While the proposed method B can achieve 0.5%, 0.5% 1.7% and 1.1% BD-rate reduction respectively for the corresponding categories.

# Introduction

In the current HEVC range extensions draft specification [1], Intra block copying (Intra BC) was adopted to take advantage of reduplicated content in a picture. As depicted in Fig.1, a reference block in the current picture is copied to the current block as the prediction if Intra BC is applied. The samples in the reference block must have been reconstructed already before the current block is coded or decoded. Thus the reference block cannot be overlapped with the current block as demonstrated in Fig.2.

A straight-forward way to extend Intra BC mode when the reference block is overlapped with the current block is to pad the unreconstructed samples horizontally or vertically.

There is an intrinsic contradiction in the horizontal or vertical padding method. Fig. 3 shows an example of samples in the reference block and in the current block when they are overlapped. It is assumed that the quantization distortion is zero for simplification. In the Intra BC prediction process, sample A is estimated to be equal to sample B, which is white. But since A is in the overlapped area, it is also estimated to be equal to sample D which is black in the horizontal padding process as shown in Fig. 4. So the estimation for sample A is contrary in the prediction and padding process.

Fig.1 Intra block copying.

Fig.2 The reference block and the current block are overlapped.

Fig.3 An example of samples when the reference block and the current block are overlapped.

Fig.4 Horizontal padding.

# Proposed methods

## Method A

In method A, a true padding method is proposed to pad the unreconstructed samples in the overlapped area as its prediction samples directly to overcome the contradiction.

A sample in the overlapped area is in the reference block as well as in the current block. As a sample in the current block, it should be predicted to a prediction value in the prediction process. As a sample in the reference block, it should be padded to a padding value in the padding process on the other hand. In the true padding method, the prediction value in the prediction process is treated as the padding value directly.

Still taking Fig. 2 as an example, sample A is estimated to be equal to sample B which is white in the Intra BC prediction process. And A is estimated to be equal to its predicted value, i.e. B, in the true padding process. As a result, the estimation for sample A is the same in the prediction and padding process. In the example in Fig.2, samples drawn in the overlapped area can just be considered as the padding values.

Actually, there should be some replicated pattern in a block if the Intra BC is applied mode with a reference block overlapped with the current block. It can be seen clearly that the true padding method can reflect the replicated pattern in the reference block much better than the horizontal padding method from Fig. 3 and Fig. 4.

## Method B

The method B is illustrated in Fig. 5. The “BV” is the block vector for IntraBC. In this method, we extend the BV along the same line towards the overlapped area and split the overlapped area in two regions. The reference samples in the bottom left overlapped region are generated by copying the reference samples of the right-most column of the left CU (horizontal padding), and the reference samples in the top right region are generated by copying the reference samples of the bottom-most row of the above CU (vertical padding).

Fig. 5 Horizontal and Vertical Padding using BV as boundary

# Experimental results

The proposed methods are integrated on HM-12.1+RExt-5.1 [2].	For method A, Table 1 and Table 2 show the results for losses and lossy respectively. For method B, Table 3 and Table 4 show the results for losses and lossy respectively.

Table 1. Lossless results for method A

Table 2. Lossy results for method A

Table 3: Lossless results for method B

Table 4: Lossy results for method B

Note: the run time in method B is not accurate.

# Conclusion

In this proposal, two padding methods for padding overlapped area in IntraBC are proposed. An unreconstructed sample in the reference block is estimated by its prediction. Experimental results reportedly show that the proposed method A can achieve 0.9%, 0.8%, 1.5% and 1.6% BD-rate reduction respectively for RGB SC, YUV SC, optional RGB SC, and optional YUV SC sequences in AI HE Main-Tier configurations; the proposed method B can achieve 0.5%, 0.5%, 1.7% and 1.1% BD-rate reduction respectively for RGB SC, YUV SC, optional RGB SC, and optional YUV SC sequences in AI HE Main-Tier configurations.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- D. Flynn, J. Sole, G. J. Sullivan, T. Suzuki, “HEVC Range Extensions Draft 5,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1005, Oct. 2013.

- HM-12.1+RExt-5.1, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-12.1+RExt-5.1