| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San Jose, US, 9th – 17th, Jan. 2014 | Document: JCTVC-P0193 |
| --- | --- |

| Title: | RCE3: Cross-check report of Subtest D.2 on Nx2N/2NxN/NxN Intra Block Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu and Shan Liu | Tel: Email: | +1 408 526 1899 Ext. 88123 xiaozhong.xu@mediatek.com |
| Source: | MediaTek, Inc. |  |  |

_____________________________

# Abstract

This document provides the cross-check report for JCTVC-P0180 contribution which is submitted to RCE3 Subtest D2: Nx2N/2NxN/NxN Intra Block Copy. Simulation results show the identical numbers provided by the proponent.

# Study of the software

In the software provided by the proponents, there are a few options to control the encoder behavior for 2NxN/Nx2N/NxN Intra Block Copy.

#define NCTU_NONSQUARE_INTRABC              1   ///< Support 2NxN/Nx2N intraBC mode. (0: OFF, 1: ON)

#define RCE3B3_INTRABC_NxN                  1   ///< Support NxN intraBC mode. (0: OFF, 1: ON)

#if NCTU_NONSQUARE_INTRABC

#define NS_INTRABC_INTRA_SPLIT_FLAG       1   ///< Set the value of IntraSplitFlag for non-square intraBC. When the flag is on, the minimal RQT level is equal to 1; otherwise, the minimal level is equal to 0. (0: OFF, 1: ON)

#define NS_INTRABC_EVAL_AT_SCU_ONLY       1   ///< Disable the estimation of 16x16 CU nonsquare intraBC. (0: OFF, 1: ON)

///  (Note: this macro works only when INTRABC_FASTME is equal to 1.)

#define NS_INTRABC_RD_THRESHOLD_FAST_MD   1   ///< Fast mode decision for NS IntraBC based on the R-D cost of intra, intraBc 2Nx2N, and intraBc Nx2N. (0: OFF, 1: ON)

///   (Note: This macro works only when INTRABC_FASTME is equal to 1)

#endif

In particular, when NS_INTRABC_EVAL_AT_SCU_ONLY is set to be 1, the 2NxN and Nx2N Intra Block Copy estimation will be performed only at 8x8 CU. When this option is set to be 0, the 2NxN and Nx2N Intra Block Copy estimation will also be enabled at 16x16 CU.

In the non square Intra Block Copy estimation, the Nx2N partition will be tested first, followed by 2NxN partition. When NS_INTRABC_RD_THRESHOLD_FAST_MD is set to be 1, the cost of Nx2N partition, as well as the cost of intra mode and 2Nx2N Intra Block Copy mode, will be used to decide whether the search on 2NxN partition is necessary.

In the NxN (4x4) Intra Block Copy estimation, the results of cost of Nx2N/2NxN partitions, as well as the cost of intra mode and 2Nx2N Intra Block Copy mode, will be used to decide whether the search on NxN partition is necessary.

# Experimental results

All configurations are set as recommended by the proponents (fast NxN option on). Table 1 and Table 2 show the results for losses and lossy respectively. The runtime comparison is not reliable.

Table 1. Lossless results

Table 2. Lossy results

# Conclusion

This document reports the perfectly matched cross-check results for JCTVC-P0180 contribution regarding RCE3 Subtest D2: Nx2N/2NxN/NxN Intra Block Copy (fast NxN option).  Detailed simulation results are shown in attached excel.