| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0194 |
| --- | --- |

| Title: | RCE1: Cross-verification of subtest A.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sunil Lee Chanyul Kim | Email: | sunil.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the cross-verification results of the subtest A.2 from RCE1. In this method, all the context-coded flags for the current 4×4 subblock are bypassed depending on the initial Rice parameter value for the current 4×4 subblock or the Rice parameter value at the end of the previous 4×4 subblock. The coding performance is independently evaluated by Samsung and it is confirmed that AHG18 test results match those provided by the proponent. AHG5 and AHG8 lossless test results were not available when this report was uploaded due to the late release of the test results from the proponent.

# Introduction

This contribution reports the cross-verification results of the subtest A.2 from RCE1. In this method, all the context-coded flags for the current 4×4 subblock are bypassed depending on the initial Rice parameter value for the current 4×4 subblock or the Rice parameter value at the end of the previous 4×4 subblock. Samsung compiled the source code and did the simulation on a linux cluster under the common test condition defined for RCE1 [2].

# Test results

The cross-verification results of the subtest A.2 from RCE1 are summarized in Table 1, 2, and 3. It is confirmed that AHG18 test results match those provided by the proponent. AHG5 and AHG8 lossless test results were not available when this report was uploaded due to the late release of the test results from the proponent. They will be updated as soon as the results are available.

Table 1. Cross-check test results of RCE1 subtest A.2 in AHG5 CTC

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 |  |  |  |  |  |  |  |
| YCbCr 4:4:4 |  |  |  |  |  |  |  |
| YCbCr 4:2:2 |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |

Table 2. Cross-check test results of RCE1 subtest A.2 in AHG8 CTC (lossless)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F |  |  |  |
| Class B |  |  |  |
| RGB 4:4:4 SC |  |  |  |
| RGB 4:4:4 Animation |  |  |  |
| YCbCr 4:4:4 SC |  |  |  |
| YCbCr 4:4:4 Animation |  |  |  |
| RangeExt |  |  |  |
| RGB 4:4:4 SC (Optional) |  |  |  |
| YCbCr 4:4:4 SC (Optoinal) |  |  |  |
| Enc Time[%] |  |  |  |
| Dec Time[%] |  |  |  |

Table 3. Cross-check test results of RCE1 subtest A.2 in AHG18 CTC

# Conclusion

This contribution reports the cross-verification results of the subtest A.2 from RCE1. The coding performance is independently evaluated by Samsung and it is confirmed that AHG18 test results match those provided by the proponent. AHG5 and AHG8 lossless test results were not available when this report was uploaded due to the late release of the test results from the proponent.

# References

- R. Joshi, J. Sole, M. Karczewicz, “AHG5 and AHG18: Bypass of the significance and coefficient level flags for higher throughput,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0209, Oct. 2013.

- R. Joshi, K. Sharman, “HEVC Range Extensions Core Experiment 1 (RCE1): High bit rate coding at high bit depths,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1121, Oct. 2013.