| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0195 |
| --- | --- |

| Title: | RCE1: Cross-verification of subtest B.6 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sunil Lee Chanyul Kim | Email: | sunil.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the cross-verification results of the subtest B.6 from RCE1. In this subtest, the combination of subtest B.1 and A.2 is tested. The coding performance is independently evaluated by Samsung and it is confirmed that the results match those provided by the proponent.

# Introduction

This contribution reports the cross-verification results of the subtest B.6 from RCE1 [1]. In this subtest, the combination of subtest B.1 and A.2 is tested. Samsung compiled the source code and did the simulation on a linux cluster under the common test condition defined for RCE1 [2].

# Test results

The cross-verification results of the subtest B.6 from RCE1 are summarized in Table 1, 2, and 3. It is confirmed that the cross-check results match those provided by the proponent.

Table 1. Cross-check test results of RCE1 subtest B.6 in AHG5 CTC

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |

Table 2. Cross-check test results of RCE1 subtest B.6 in AHG8 CTC (lossless)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 3.5% | 1.7% | 1.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 21.7% | 20.7% | 20.5% |
| RGB 4:4:4 Animation | 0.1% | 0.1% | 0.1% |
| YCbCr 4:4:4 SC | 11.5% | 10.4% | 10.8% |
| YCbCr 4:4:4 Animation | 0.1% | 0.1% | 0.1% |
| RangeExt | 0.2% | 0.2% | 0.2% |
| RGB 4:4:4 SC (Optional) | 54.8% | 54.7% | 61.6% |
| YCbCr 4:4:4 SC (Optoinal) | 44.0% | 44.8% | 52.5% |
| Enc Time[%] | 101% | 109% | 114% |
| Dec Time[%] | 125% | 122% | 136% |

Table 3. Cross-check test results of RCE1 subtest B.6 in AHG18 CTC

# Conclusion

This contribution reports the cross-verification results of the subtest B.6 from RCE1. The coding performance is independently evaluated by Samsung and it is confirmed that the cross-check results match those provided by the proponent.

# References

- R. Joshi, J. Sole, M. Karczewicz, “RCE1: Results for subtests B5, B6, B7,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0074, Jan. 2014.

- R. Joshi, K. Sharman, “HEVC Range Extensions Core Experiment 1 (RCE1): High bit rate coding at high bit depths,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1121, Oct. 2013.