| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0105 |
| --- | --- |

| Title: | RCE3: Cross-verification of subtest D.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sunil Lee Chanyul Kim | Email: | sunil.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the cross-verification results of the subtest D.1 from RCE3. The subtest D.1 is the combination of the subtest B.1 (Nx2N/2NxN PU-based intra block copy) and B.2 (TU-based intra block copy) [1][2]. The coding performance is independently evaluated by Samsung and it is confirmed that the results match the incomplete test results provided by the proponent. The cross-check test is not completed yet due to the late release of the software and test results by the proponent.

# Introduction

This contribution reports the cross-verification results of the subtest D.1 from RCE3 on combination of the subtest B.1 (Nx2N/2NxN PU-based intra block copy) and B.2 (TU-based intra block copy) [1][2]. Samsung compiled the source code and did the simulation on a linux cluster under the common test condition defined for RCE3 [3].

# Test results

The cross-verification results of the subtest D.1 from RCE3 are summarized in Table 1 and 2, and it is confirmed that the results in lossy coding configuration match the incomplete test results provided by the proponent. The cross-check test is not completed due to the late release of the software and test results from the proponent. The results will be updated as soon as they are available.

Table 1. Test results of RCE3 subtest D.1 in lossy coding configuration

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | -4.0% | -3.7% | -3.3% | -2.9% | -2.7% | -1.6% | -1.6% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -9.3% | -9.3% | -9.1% | -7.7% | -7.7% | -5.5% | -5.9% |
| RGB 4:4:4 Animation | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -9.0% | -9.2% | -9.0% | -7.4% | -7.7% | -5.1% | -5.6% |
| YCbCr 4:4:4 Animation | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -11.0% | -11.8% | -12.4% | -9.0% | -10.0% | -7.3% | -8.3% |
| YCbCr 4:4:4 SC (Optoinal) | -13.1% | -14.6% | -15.8% | -11.3% | -13.1% | -10.5% | -12.0% |
| Enc Time[%] | 135% | 137% | 138% | 108% |  |  |  |
| Dec Time[%] | 100% | 100% | 100% | 100% |  |  |  |

Table 2. Test results of RCE3 subtest D.1 in lossless coding configuration

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| Class F |  |  |  |
| Class B |  |  |  |
| RGB 4:4:4 SC |  |  |  |
| RGB 4:4:4 Animation |  |  |  |
| YCbCr 4:4:4 SC |  |  |  |
| YCbCr 4:4:4 Animation |  |  |  |
| RangeExt |  |  |  |
| RGB 4:4:4 SC (Optional) |  |  |  |
| YCbCr 4:4:4 SC (Optoinal) |  |  |  |
| Enc Time[%] |  |  |  |
| Dec Time[%] |  |  |  |

# Conclusion

This contribution reports the cross-verification results of the subtest D.1 from RCE3. It is confirmed that the results in lossy coding configuration match the incomplete test results provided by the proponent. The cross-check test is not completed due to the late release of the software and test results from the proponent.

# References

- C.-C. Chen, T.-S. Chang, R.-L. Liao, W.-H. Peng, H.-M. Hang, C.-L. Lin, F.-D. Jou, “AHG8: Line-based Intra Block Copy,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0205, Oct. 2013.

- B. Li, J. Xu, G. J. Sullivan, “On Intra BC mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0183, Oct. 2013.

- J. Sole, E. Alshina, D.-K. Kwon, W.-H. Peng, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra block copy refinement,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1123, Oct. 2013.