# Draft texts

7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( palette_enabled_flag && CuPredMode[ x0 ][ y0 ] == MODE_INTRA ) |  |
| palette_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( palette_mode_flag[ x0 ][ y0 ] ) |  |
| palette_coding(x0, y0, nCbS, nCbS) |  |
| if(( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY)) && (!palette_mode_flag[ x0 ][ y0 ]) |  |
| part_mode | ae(v) |
| } |  |
| if (!palette_mode_flag[ x0 ][ y0 ]) { |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| … |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

| palette_coding(x0, y0, CbWidth, CbHeight) { | Descriptor |
| --- | --- |
| if( ChromaArrayType == 3 ) { |  |
| palette_coding_component(x0, y0, CbWidth, CbHeight, 3) |  |
| } |  |
| if( ChromaArrayType == 2 ) { |  |
| palette_coding_component(x0, y0, CbWidth, CbHeight, 1) |  |
| palette_coding_component(x0, y0, CbWidth >> 1, CbHeight, 2) |  |
| } |  |
| if( ChromaArrayType == 1 ) { |  |
| palette_coding_component(x0, y0, CbWidth, CbHeight, 1) |  |
| palette_coding_component(x0, y0, CbWidth >> 1, CbHeight >> 1, 2) |  |
| } |  |
| if( ChromaArrayType == 0 ) { |  |
| palette_coding_component(x0, y0, CbWidth, CbHeight, 1) |  |
| } |  |

| palette_coding_component(x0, y0, CbWidth, CbHeight, NumComp) { | Descriptor |
| --- | --- |
| compOffset = (NumComp == 3) ? 0 : (NumComp – 1) |  |
| nCbS = ( 1 << log2CbSize ) |  |
| numPredLeftPalette = 0 |  |
| for( i = 0; i < palette_size[ x0 − 1 ][ y0 ]; i ++ ){ |  |
| left_palette_entry_flag[ i ] | ae(v) |
| if ( left_palette_entry_flag[ i ] ){ |  |
| for ( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx ++ ) |  |
| palette_entries[ x0 ][ y0 ][ cIdx ][ numPredLeftPalette ] = palette_entries[ x0 − 1 ][ y0 ][ cIdx ][ i ] |  |
| numPredLeftPalette ++ |  |
| } |  |
| } |  |
| if( numPredLeftPalette < 32 ) |  |
| palette_num_signalled_entries | ae(v) |
| for (cIdx = compOffset; cIdx < NumComp + compOffset; cIdx ++) |  |
| for( i = 0; i < palette_num_signalled_entries; i ++ ) |  |
| palette_entries[ x0 ][ y0 ][ cIdx ][ numPredLeftPalette + i ] | ae(v) |
| palette_size[ x0 ][ y0 ] = numPredLeftPalette + palette_entries |  |
| scanPos = 0 |  |
| while (scanPos < nCbS * nCbS){ |  |
| palette_mode_type_flag[ xC ][ yC ] | ae(v) |
| if ( palette_mode_type_flag[ xC ][ yC ] == ESCAPE_MODE ){ |  |
| xC = scanPos % nCbS |  |
| yC = scanPos / nCbS |  |
| scanPos++ |  |
| palette_predict_escape_flag[ xC ][ yC ] | ae(v) |
| for (cIdx = compOffset; cIdx < NumComp + compOffset; cIdx ++){ |  |
| if ( palette_predict_escape_flag ) { |  |
| palette_escape_diff_abs | ae(v) |
| if(palette_escape_diff_abs !=0) |  |
| palette_escape_sign_flag | ae(v) |
| samples_array[ cIdx ][ xC ][ yC ] = palette_escape_diff_abs * ( 1 − 2 * palette_escape_sign_flag ) |  |
| } |  |
| else{ |  |
| palette_escape_val | ae(v) |
| samples_array[ cIdx ][ xC ][ yC ] = palette_escape_val |  |
| } |  |
| } |  |
| } |  |
| else{ /* RUN_MODE */ |  |
| palette_run_type_flag | ae(v) |
| if (!palette_run_type_flag) |  |
| palette_index | ae(v) |
| palette_run | ae(v) |
| runPos= 0 |  |
| while ( runPos <= palette_run ){ |  |
| xC = scanPos% nCbS |  |
| yC = scanPos/ nCbS |  |
| if ( palette_run_type_flag == INDEX_RUN_MODE){ |  |
| paletteMap[ xC ][ yC ] = palette_index |  |
| } |  |
| else{ /* COPY_ABOVE_RUN_MODE */ |  |
| paletteMap[ xC ][ yC ] = paletteMap[ xC ][ yC − 1 ] |  |
| } |  |
| for (cIdx = compOffset; cIdx < NumComp + compOffset; cIdx ++) |  |
| samples_array[ cIdx ][ xC ][ yC ] = palette_entries[ x0 ][ y0 ][ cIdx ][ paletteMap[ xC ][ yC ]] |  |
| runPos ++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |
| } |  |

7.4.3.2 Sequence parameter set RBSP semantics

sps_extension1_flag equal to 1 specifies that transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, intra_block_copy_enabled_flag, palette_enabled_flag, residual_dpcm_intra_enabled_flag, residual_dpcm_inter_enabled_flag, extended_precision_processing_flag, intra_smoothing_disabled_flag, and sps_extension2_flag are present in the SPS RBSP syntax structure. sps_extension1_flag equal to 0 specifies that these syntax elements are not present.

palette_enabled_flag equal to 1 specifies that the palette mode may be used for intra blocks. palette_enabled_flag equal to 0 specifies that the palette mode is not applied. When not present, the value of palette_enabled_flag is inferred to be equal to 0.

## Semantics

palette_mode_flag [ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded using the palette mode. palette_mode_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded using the palette mode.

When palette_mode_flag [ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

In the following sematics, the array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

left_palette_entry_flag [ i ] equals to 1 specifies that the palette entry from the palette on the left is copied. left_palette_entry_flag [ i ] equals to 1 specifies that the palette entry from the palette on the left is not copied.

palette_num_signalled_entries specifies the number of entries in the palette that are explicitly signalled for the current coding unit.

When palette_num_signalled_entries is not present, it is inferred to be equal to 0.

palette_entries[ x0 ][ y0 ][ cIdx ][ i ] specifies the i-th element in the palette for the color component cIdx.

When palette_entries[ x0 ][ y0 ][ cIdx ][ i ] is not present, it is inferred to be equal to 0.

palette_mode_type_flag[ xC ][ yC ] equals to ESCAPE_MODE specifies that a single escape value is signalled at location ( xC, yC ). palette_escape_flag[ xC ][ yC ] equals to RUN_MODE specifies that a run of a number of consecutive locations starting at location ( xC, yC ) with the same palette index or with the same palette index as the position in the above row is signalled.

palette_predict_escape_flag [ xC ][ yC ] equals to 1 specifies that the escape value at location ( xC, yC ) is predicted using the pixel value at position ( xC, yC − 1 ). palette_predict_escape_flag [ xC ][ yC ] equals to 0 specifies that the escape value at position ( xC, yC ) is not predicted.

When palette_predict_escape_flag is not present, it is inferred to be equal to 0.

palette_escape_diff specifies the absolute value of the predicted error of the escape value.

palette_escape_sign_flag specifies the sign of the predicted error of the escape value as follows:

- If palette_escape_sign_flag is equal to 0, the corresponding escape value has a positive value.

- Otherwise (palette_escape_sign_flag is equal to 1), the corresponding escape value has a negative value.

When palette_escape_sign_flag is not present, it is inferred to be equal to 0.

palette_escape_val specifies the absolute value of the escape value.

palette_run_type_flag equal to 1 specifies that the decoding process is COPY_ABOVE_RUN_MODE where the decoded pixel value is equal to the pixel at the same location in the above row. palette_run_type_flag equal to 0 specifies that the decoding process mode is INDEX_RUN_MODE where the pixel value is equal to the corresponding value of the index in the palette.

When palette_run_type_flag is not present, it is inferred to be equal to 0.

palette_index is an index to the palette entries.

When palette_index is not present, it is inferred to be equal to 0.

palette_run the number of consecutive locations with the same palette index as the position in the above row when palette_run_type_flag is equal to COPY_ABOVE_RUN_MODE or represents the number of consecutive locations minus1 with the same palette index when palette_run_type_flag is equal to INDEX_RUN_MODE.

8.4.1 General decoding process for coding units coded in intra prediction mode

Depending on the values of pcm_flag[ xCb ][ yCb ], palette_mode_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for luma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SL[ xCb + i ][ yCb + j ] = 
		pcm_sample_luma[ ( nCbS * j ) + i ]  <<  ( BitDepthY − PcmBitDepthY ), with i, j = 0..nCbS − 1	(8-1)

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0 and palette_mode_flag[ xCb ][ yCb ] is equal to 1), the decoding process is specified as follows:

For each i, j = 0..nCbS – 1,

SL[ xCb + i ][ yCb + j ] = sample_array[ 0 ][ i ][ j ]

– If palette_predict_escape_flag[ i ][ j ] is equal to 1,

SL[ xCb + i ][ yCb + j ] = sample_array[ 0 ][ i ][ j ] + ( j > 0 ? SL [ xCb + i ][ yCb + j − 1 ] : 0 )

Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, palette_mode_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0, the following ordered steps apply:

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, palette_mode_flag[ xCb ][ yCb ] is equal to 0 and IntraSplitFlag is equal to 1), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xPb is set equal to xCb + ( nCbS  >>  1 ) * ( blkIdx % 2 ).

- The variable yPb is set equal to yCb + ( nCbS  >>  1 ) * ( blkIdx / 2 ).

- The derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ) as input.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSize − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeY[ xPb ][ yPb ], the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

When ChromaArrayType is not equal to 0, the following applies.

The variable log2CbSizeC is set equal to log2CbSize − ( ChromaArrayType  = =  3 ? 0 : 1 ).

Depending on the value of pcm_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for chroma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SCb[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] = 	pcm_sample_chroma[ ( nCbS / SubWidthC * j ) + i ]  <<
				( BitDepthC − PcmBitDepthC ), with i = 0..nCbS / SubWidthC − 1, and j = 0..nS / SubHeightC − 1	(8-2)

SCr[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] = pcm_sample_chroma[ ( nCbS / SubWidthC * ( j + nCbS / SubHeightC ) ) + i ]  <<
				( BitDepthC − PcmBitDepthC ), with i = 0..nCbS / SubWidthC − 1, and j = 0..nS / SubHeightC − 1	(8-3)

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, palette_mode_flag[ xCb ][ yCb ] is equal to 1), the decoding process is specified as follows

For each i = 0..nCbS/ SubWidthC  – 1, j = 0..nCbS / SubHeightC – 1,

SCb[ xCb + i ][ yCb + j ] = sample_array[ 1 ][ i ][ j ]

– If palette_predict_escape_flag[ i ][ j ] is equal to 1,

SCb[ xCb + i ][ yCb + j ] = sample_array[ 1 ][ i ][ j ] + ( j > 0 ? SCb[ xCb + i ][ yCb + j − 1 ] : 0 )

For each i = 0..nCbS/ SubWidthC  – 1, j = 0..nCbS / SubHeightC – 1,

SCr[ xCb + i ][ yCb + j ] = sample_array[ 2 ][ i ][ j ]

– If palette_predict_escape_flag[ i ][ j ] is equal to 1,

SCr[ xCb + i ][ yCb + j ] = sample_array[ 2 ][ i ][ j ] + ( j > 0 ? SCr[ xCb + i ][ yCb + j − 1 ] : 0 )

Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, palette_mode_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0 or ChromaArrayType is not equal to 3, the following ordered steps apply:

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the chroma intra prediction mode as specified in Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input, and the output is the variable IntraPredModeC.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 1 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 2 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xC ][ yC ] is equal to 0 palette_mode_flag[ xCb ][ yCb ] is equal to 0 and IntraSplitFlag is equal to 1 and ChromaArrayType is equal to 3), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

…

### 8.4.4.3 Scaling process for escape pixels

Inputs to this process are:

–	a value escValue specifying the sample value of the escape pixel within the current block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable qP specifying the quantization parameter.

Output of this process is scaled escape pixel sample value escReco.

The variable escReco is derived as follows:

–	If cIdx is equal to 0,

bdShift = BitDepthY + Log2( nTbS ) − 5

–	Otherwise,

bdShift = BitDepthC + Log2( nTbS ) − 5

The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

For the derivation of the scaled escape pixel sample value escReco, the following applies:

–	The scaling factor m[ x ][ y ] is derived as follows:

m[ x ][ y ] = 16

–	The scaled escape pixel sample value escReco is derived as follows:

escReco = Clip3( −32768, 32767, ( ( escValue * m[ x ][ y ] * 
			levelScale[ qP%6 ]  <<  (qP / 6 ) ) + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift )

Table 9-4 Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit( ) | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | cu_skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | intra_bc_flag[ ][ ] | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
|  | palette_mode_flag[ ][ ] | Table 9-XX | 0..2 | 3..5 | 6..8 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 0 | 1..4 | 5..8 |

| Table 9-1 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| coding_unit( ) | cu_transquant_bypass_flag | FL | cMax = 1 |
|  | cu_skip_flag | FL | cMax = 1 |
|  | intra_bc_flag | FL | cMax = 1 |
|  | palette_mode_flag | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | part_mode | Error: Reference source not found | ( xCb, yCb ) = ( x0, y0), log2CbSize |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | prev_intra_luma_pred_flag[ ][ ] | FL | cMax = 1 |
|  | mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | rem_intra_luma_pred_mode[ ][ ] | FL | cMax = 31 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | - |
|  | rqt_root_cbf | FL | cMax = 1 |
| palette_coding( ) | left_palette_entry_flag | TR | cMax = 31, cRiceParam = 0 |
|  | palette_num_signalled_entries | TR | cMax = 31, cRiceParam = 0 |
|  | palette_entries | FL | cMax = cIdx == 0 ? (1<<BitDepthY) – 1 : (1<<BitDepthC) – 1 |
|  | palette_mode_type_flag | FL | cMax = 1 |
|  | palette_predict_escape_flag | FL | cMax = 1 |
|  | palette_escape_diff | FL | cMax = cIdx == 0 ? (1<<(1+BitDepthY))) – 1 : (1<<(1+BitDepthC)) – 1 |
|  | palette_escape_sign_flag | FL | cMax = 1 |
|  | palette_escape_val | FL | cMax = cIdx == 0 ? (1<<BitDepthY) – 1 : (1<<BitDepthC) – 1 |
|  | palette_run_type_flag | FL | cMax = 1 |
|  | palette_index | FL | cMax = 31 |
|  | palette_run | TR | cMax = CtbSizeY * CtbSizeY , cRiceParam = 3 |

Table 9-39 Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| cu_skip_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| pred_mode_flag | 0 | na | na | na | na | na |
| palette_mode_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| left_palette_entry_flag | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_num_signalled_entries | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_entries | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_mode_type_flag | 0, 1 (subclause Error: Reference source not found) | na | na | na | na | na |
| palette_predict_escape_flag | 0,1 (subclause Error: Reference source not found) | na | na | na | na | na |
| palette_escape_diff | 0 | 1 | 2 | 3 | 4 | 5 |
| palette_escape_sign_flag | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_escape_val | 0 | 1 | 2 | 3 | 4 | 5 |
| palette_run_type_flag | 0, 1 (subclause Error: Reference source not found) | na | na | na | na | na |
| palette_index | bypass | bypass | bypass | bypass | bypass | bypass |
| palette_run | 0 | 1 | 2 | bypass | bypass | bypass |
| part_mode log2CbSize = = MinCbLog2SizeY | 0 | 1 | 2 | bypass | na | na |

Table 9-2 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxInc |
| --- | --- | --- | --- |
| split_cu_flag[ x0 ][ y0 ] | CtDepth[ xNbL ][ yNbL ] > cqtDepth | CtDepth[ xNbA ][ yNbA ] > cqtDepth | ( condL && availableL ) + ( condA && availableA ) |
| cu_skip_flag[ x0 ][ y0 ] | cu_skip_flag[ xNbL ][ yNbL ] | cu_skip_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |
| intra_bc_flag[ x0 ][ y0 ] | intra_bc_flag[ xNbL ][ yNbL ] | intra_bc_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |
| palette_mode_flag[ x0 ][ y0 ] | palette_mode_flag [ xNbL ][ yNbL ] | palette_mode_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |
| palette_mode_type_flag[ x0 ][ y0 ] | na | palette_mode_type_flag[ xNbA ][ yNbA ] | ( condA && availableA ) |
| palette_predict_escape_flag[ x0 ][ y0 ] | na | palette_predict_escape_flag [ xNbA ][ yNbA ] | ( condA && availableA ) |
| palette_run_type_flag [ x0 ][ y0 ] | na | palette_run_type_flag [ xNbA ][ yNbA ] | ( condA && availableA ) |