| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0198 |
| --- | --- |

| Title: | RCE4: Results of Test 2 on Palette Mode for Screen Content Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | L. Guo W. Pu Marta Karczewicz J. Sole R. Joshi F. Zou | Email: | joels@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of RCE4 test 2 on palette coding for screen content.

# Introduction

The coding of screen contents plays an important role in applications like remote desktop control and on-line collaboration. Compared to natural video content, screen content tends to have much fewer distinct pixel values (discrete tone) and much sharper object edges. A palette-based coding method (JCTVC-O0218) [1] was presented in 15-th JCT-VC meeting and was included as test 2 in RCE4 for San Jose [2]. This contribution provides detailed description of the methods and the evaluation of the results.

# Method Description

The following is a description of the palette-based coding. The palette-based coding is designed as a CU mode. The encoding process is as follows:

- Transmission of the palette. The maximum palette size is set to 32. An entry-wise prediction scheme is used to encode the current palette based on the palette of the left CU. Let the size of the left CU to be N. A binary vector of size N is first transmitted to indicate which entry in the palette of the left CU also appears in the current CU. After that, the new entries (i.e., entries in the palette of the current CU but not in the palette of the left CU) are transmitted.

- Transmission of pixel values: the pixels in the CU are encoded in a raster scan order. For each position, a mode is first transmitted to indicate which one of the following 3 modes is used.

  - “Run mode”: In “run mode”, a palette index is first signaled followed by “palette_run” (e.g., M). No further information needs to be transmitted for the current position and the following M positions as they have the same palette index as signaled. The palette index (e.g., i) is shared by all three color components, which means that the reconstructed pixel values are (Y, U, V) = (paletteY[i], paletteU[i], paletteV[i]) (assuming the color space is YUV).

  - “Copy above mode”: In “copy above mode”, a value “copy_run” (e.g., N) is transmitted to indicate that for the following N positions (including the current one), the palette index is equal to the palette index of the one that is at the same location in the row above.

  - “Pixel mode”: In “pixel mode”, a prediction flag is first transmitted. If this flag is 1, the current pixel is predicted using reconstructed top neighboring pixels, and the prediction error/residue is transmitted. If this flag is 0, the pixel value is transmitted without prediction.

# Experimental results

The proposed method is implemented on HM-12.1+RExt-5.1. RCE4 common test conditions specified in [1] are used. Encoding time and decoding time is measured on a homogenous cluster.

Table 1 Summary of AHG 8 Lossless Coding Results

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | -0.4% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -17.8% | -12.0% | -8.2% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -17.4% | -12.4% | -8.4% |
| YCbCr 4:4:4 Animation | -0.8% | -0.1% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -38.9% | -27.8% | -17.3% |
| YCbCr 4:4:4 SC (Optional) | -37.5% | -29.8% | -22.4% |
| Enc Time[%] | 123% | 108% | 105% |
| Dec Time[%] | 100% | 101% | 102% |

Table 2 Summary of AHG8 Lossy Coding Results

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.6% | -1.7% | -1.1% | -1.1% | -1.8% | -1.3% | -1.4% | -1.7% | -1.5% |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -10.0% | -10.0% | -10.3% | -13.0% | -12.9% | -13.0% | -16.3% | -17.2% | -17.0% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -7.0% | -8.9% | -9.9% | -11.1% | -12.6% | -12.9% | -14.7% | -16.6% | -17.3% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -26.8% | -25.8% | -26.6% | -30.5% | -30.3% | -30.9% | -37.1% | -37.1% | -38.1% |
| YCbCr 4:4:4 SC (Optional) | -22.0% | -27.7% | -27.3% | -29.4% | -33.9% | -34.3% | -37.2% | -42.6% | -43.2% |
| Enc Time[%] | 117.6% |  |  | 115.9% |  |  | 114.5% |  |  |
| Dec Time[%] | 99.3% |  |  | 97.8% |  |  | 96.4% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -2.5% | -1.0% | -0.5% | -1.9% | -1.0% |  |  |  |
| Class B | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| RGB 4:4:4 SC | -9.3% | -9.2% | -9.5% | -11.4% | -11.4% | -11.6% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| YCbCr 4:4:4 SC | -6.8% | -9.1% | -10.1% | -10.3% | -12.0% | -12.4% |  |  |  |
| YCbCr 4:4:4 Animation | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -21.8% | -20.7% | -21.4% | -24.9% | -24.3% | -24.7% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -19.3% | -28.6% | -29.0% | -24.3% | -31.0% | -32.0% |  |  |  |
| Enc Time[%] | 107.2% |  |  | 106.4% |  |  |  |  |  |
| Dec Time[%] | 100.0% |  |  | 99.1% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -2.3% | -0.6% | -0.2% | -1.6% | -0.5% |  |  |  |
| Class B | 0.1% | -0.2% | -0.2% | 0.1% | -0.1% | -0.1% |  |  |  |
| RGB 4:4:4 SC | -4.2% | -4.2% | -4.3% | -5.4% | -5.4% | -5.4% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -2.9% | -3.8% | -4.1% | -4.4% | -5.0% | -5.3% |  |  |  |
| YCbCr 4:4:4 Animation | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| RangeExt | 0.2% | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -11.1% | -10.6% | -10.9% | -15.0% | -14.5% | -15.4% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -10.2% | -15.2% | -15.3% | -14.0% | -18.3% | -18.4% |  |  |  |
| Enc Time[%] | 105.8% |  |  | 105.4% |  |  |  |  |  |
| Dec Time[%] | 101.2% |  |  | 99.7% |  |  |  |  |  |

# References

[1] L. Guo, M. Karczewicz, J. Sole and R. Joshi, “Evaluation of Palette Mode Coding on HM-12.0+RExt-4.1”, JCTVC-O0218, Geneva, Switzerland, Oct. 2013.

[2] L. Guo, X. Guo and A. Saxena, “HEVC Range Extensions Core Experiment 4 (RCE 4): Palette Coding For Screen Content”, JCTVC-O1124, Geneva, Switzerland, Oct. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).