| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0199_r2 |
| --- | --- |

| Title: | RCE2: Results of Test 1 on Rice Parameter Initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Liwei Guo, Joel Sole, Rajan Joshi Karl Sharman, Nick Saunders, James Gamei | Email: | martak@qti.qualcomm.com karl.sharman@eu.sony.com |
| Source: | Qualcomm Incorporated, Sony Europe Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the results of RCE2 test 1 on rice parameter initialization.

# Introduction

In HEVC, the binarization of the syntax element coeff_abs_level_remaining depends on the Rice parameter. In the 15-th JCTVC meeting,  a method was presented to initialize the Rice parameter based on previously coded coefficients.

# Method Description

In the proposed scheme, the 4×4 sub-blocks are divided into different categories (“sbType”). For each sub-block, the initial Rice parameter is derived based on previously coded sub-blocks in the same category. The categorization is based on whether the block is a transform-skip block (“isTSFlag”) and whether it is the luma component

sbType = isLuma*2+ isTSFlag

Stats statCoeff  are maintained for each sub-block type (sbType) depending on the absolute coefficient value (uiLevel):

if (uiLevel >=3*(1<<(statCoeff /4)))            statCoeff ++;

else if ((2* uiLevel)<(1<<( statCoeff/4)))           statCoeff --;

This variable is updated at most once per 4×4 sub-block using the value of the first coded coeff_abs_level_remaining of the sub-block. The entries of statCoeff  are reset to 0 at the beginning of the slice (like the CABAC context variables).

The value of statCoeff is used to initialize the Rice parameter at the beginning of each 4×4 sub-block as

cRiceParam = Min(maxRicePara, statCoeff/4).

The Rice parameter update method in HM12.0+RExt-4.1 is used for the proposed method.

# Experimental results

The proposed method is implemented on HM-12.1+RExt-5.1. RCE2 common test conditions specified in [1] are used. Encoding time and decoding time is measured on a homogenous cluster.

Table 1 Summary of AHG 8 Lossless Coding Results

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | -0.7% | -0.5% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -3.0% | -2.8% | -2.7% |
| RGB 4:4:4 Animation | -0.3% | -0.3% | -0.3% |
| YCbCr 4:4:4 SC | -2.6% | -2.0% | -2.4% |
| YCbCr 4:4:4 Animation | -0.2% | -0.1% | -0.2% |
| RangeExt | -1.3% | -0.4% | -0.4% |
| RGB 4:4:4 SC (Optional) | -8.7% | -8.7% | -9.2% |
| YCbCr 4:4:4 SC (Optional) | -6.9% | -6.9% | -7.1% |
| Enc Time[%] | 101% | 98% | 100% |
| Dec Time[%] | 101% | 102% | 100% |

Table 3 Summary of AHG8 Lossy Coding Results

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.4% | -0.5% | -0.5% | -0.7% | -0.8% | -0.8% | -1.1% | -1.2% | -1.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.5% | -0.6% | -0.6% | -1.2% | -1.2% | -1.1% | -2.0% | -2.3% | -2.2% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.4% | -0.5% | -0.4% | -0.9% | -0.9% | -0.8% | -1.6% | -1.7% | -1.7% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -3.7% | -3.5% | -3.5% | -5.8% | -5.8% | -5.9% | -7.9% | -8.2% | -8.2% |
| YCbCr 4:4:4 SC (Optional) | -1.4% | -1.5% | -1.5% | -2.7% | -2.8% | -2.7% | -4.5% | -4.6% | -4.8% |
| Enc Time[%] | 99.4% |  |  | 99.6% |  |  | 99.6% |  |  |
| Dec Time[%] | 97.6% |  |  | 97.3% |  |  | 98.2% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -0.3% | -0.4% | -0.5% | -0.5% | -0.6% |  |  |  |
| Class B | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% |  |  |  |
| RGB 4:4:4 SC | -0.5% | -0.5% | -0.4% | -1.0% | -0.9% | -0.9% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | 0.2% | 0.2% | 0.1% | -0.3% | -0.3% | -0.3% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -3.9% | -3.7% | -3.7% | -6.1% | -6.0% | -6.2% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -1.2% | -1.2% | -1.2% | -2.4% | -2.3% | -2.4% |  |  |  |
| Enc Time[%] | 98.8% |  |  | 98.7% |  |  |  |  |  |
| Dec Time[%] | 102.1% |  |  | 102.3% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -0.5% | 0.2% | -0.2% | -0.4% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -0.5% | -0.5% | -0.5% | -0.9% | -0.8% | -0.8% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.1% | -0.1% | -0.2% | -0.3% | -0.3% | -0.3% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -3.2% | -3.4% | -3.2% | -5.3% | -5.3% | -5.5% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -1.5% | -1.6% | -1.4% | -2.2% | -2.6% | -2.3% |  |  |  |
| Enc Time[%] | 98.6% |  |  | 98.2% |  |  |  |  |  |
| Dec Time[%] | 100.8% |  |  | 100.3% |  |  |  |  |  |

Table 5 Summary of AHG18 Coding Results (Transform Skip Enabled)

|  |  |  | Intra 16-bit (internal bit depth) |  |  | Intra 12-bit (internal bit depth) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | -2.7% | -2.3% | -2.3% | -1.1% | -0.8% | -0.8% |
| RExt RGB |  |  | -1.5% | -1.2% | -1.2% | 0.1% | 0.2% | 0.2% |
| HDR |  |  | 0.2% | 0.4% | 0.3% | 0.3% | 0.3% | 0.3% |
| 4K from HDR |  |  | -1.6% | -1.2% | -1.1% | -0.1% | 0.2% | 0.2% |
| HDR 4:0:0 Medical |  |  | -0.6% |  |  | 0.0% |  |  |
| 4:0:0 Medical |  |  | 1.6% |  |  | 0.9% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | -1.1% | -0.7% | -0.7% | 0.0% | 0.1% | 0.0% |
| Synthetic RGB HDR (+4 MSBs) |  |  | -0.3% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| All Classes |  |  | -0.7% | -0.7% | -0.7% | 0.0% | 0.0% | 0.0% |
| Encoding Time % |  |  | 105% |  |  | 95% |  |  |
| Decoding Time % |  |  | 100% |  |  | 100% |  |  |

|  |  |  | Lowdelay 16-bit (internal bit depth) |  |  | Lowdelay 12-bit (internal bit depth) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | -1.9% | -1.9% | -2.0% | -0.8% | -0.8% | -0.9% |
| RExt RGB |  |  | -1.1% | -1.1% | -1.1% | 0.0% | 0.0% | 0.0% |
| HDR |  |  | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| 4K from HDR |  |  |  |  |  |  |  |  |
| HDR 4:0:0 Medical |  |  | -0.3% |  |  | 0.0% |  |  |
| 4:0:0 Medical |  |  | 1.8% |  |  | 0.5% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | -0.5% | -0.4% | -0.4% | 0.0% | 0.1% | 0.1% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% |
| All Classes |  |  | -0.3% | -0.6% | -0.6% | -0.1% | -0.2% | -0.2% |
| Encoding Time % |  |  | 100% |  |  | 100% |  |  |
| Decoding Time % |  |  | 100% |  |  | 100% |  |  |

Table 7 Summary of AHG18 Coding Results (Transform Skip Disabled)

|  |  |  | Intra 16-bit (internal bit depth) |  |  | Intra 12-bit (internal bit depth) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | -2.6% | -2.3% | -2.3% | -1.1% | -0.8% | -0.8% |
| RExt RGB |  |  | -1.4% | -1.2% | -1.1% | 0.1% | 0.2% | 0.2% |
| HDR |  |  | 0.2% | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% |
| 4K from HDR |  |  | -1.4% | -1.0% | -1.0% | -0.1% | 0.2% | 0.2% |
| HDR 4:0:0 Medical |  |  | -0.7% |  |  | 0.0% |  |  |
| 4:0:0 Medical |  |  | 1.6% |  |  | 0.9% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | -1.1% | -0.7% | -0.7% | 0.0% | 0.1% | 0.0% |
| Synthetic RGB HDR (+4 MSBs) |  |  | -0.3% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| All Classes |  |  | -0.6% | -0.7% | -0.7% | 0.0% | 0.0% | 0.0% |
| Encoding Time % |  |  | 95% |  |  | 103% |  |  |
| Decoding Time % |  |  | 97% |  |  | 97% |  |  |

|  |  |  | Lowdelay 16-bit (internal bit depth) |  |  | Lowdelay 12-bit (internal bit depth) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | -1.9% | -1.9% | -1.9% | -0.8% | -0.8% | -0.8% |
| RExt RGB |  |  | -1.0% | -1.0% | -1.0% | 0.1% | 0.1% | 0.0% |
| HDR |  |  | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| 4K from HDR |  |  |  |  |  |  |  |  |
| HDR 4:0:0 Medical |  |  | -0.3% |  |  | -0.2% |  |  |
| 4:0:0 Medical |  |  | 1.8% |  |  | 0.6% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | -0.5% | -0.4% | -0.4% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| All Classes |  |  | -0.2% | -0.6% | -0.6% | 0.0% | -0.1% | -0.1% |
| Encoding Time % |  |  | 102% |  |  | 104% |  |  |
| Decoding Time % |  |  | 97% |  |  | 97% |  |  |

# Additional results

The accompanying spreadsheets also provide additional results for the proposed method when the maximum Golomb Rice parameter is set to 10 and also when no restriction is placed on the maximum Golomb Rice parameter. Both sets of results are for the case when transform skip is enabled. The results for AHG18 test conditions for the case of “no restriction on maximum Golomb Rice parameter” are summarized below. This case was already tested in an RCE in the previous meeting cycle. [3]. In RCE2 the maximum Golomb Rice parameter was chosen to be 7 to align it with the changes adopted in the previous meeting. No limit on the maximum Golomb Rice parameter means that the parameter can take a maximum value of (internal bit-depth + 6), which corresponds to the dynamic range of the coefficients in the entropy coder (eg. 22 for 16-bit).

Table 9: Summary of AHG18 lossy coding results (MAX Golomb Rice parameter UNLIMITED)

We also present lossless results for proposed method for the SVT class (internal bit-depth = 16) and compare them with those presented in JCTVC-P0162 [2]. It can be seen that the proposed scheme with no restriction on the maximum Golomb Rice parameter outperforms that proposed in [2].

Table 6: Summary of lossless coding results for AHG18 test conditions for the SVT class (internal bit-depth = 16)

# References

[1] C. Rosewarne, M. Karczewicz, K. Sharman, and S.-H. Kim, “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialization methods”, JCTVC-O1122, Geneva, Switzerland, Oct. 2013.

[2] A. Aminlou and K. Ugur, “On 16-bit coding”, JCTVC-P0162, San Jose, USA, January 2014.

[3] M. Karczewicz, L. Guo, J. Sole, R. Joshi, K. Sharman, N. Saunders, J. Gamei, “RCE2: Results of Test D1 on Rice Parameter Initialization”, JCTVC-O0239, Geneva, Switzerland.

# Patent rights declaration(s)

Qualcomm Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).