| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0200 |
| --- | --- |

| Title: | AhG8: Performance and complexity analysis of Range Extensions tools |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joel Sole, Ted Hsieh, Chao Pang, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution provides results of the objective compression performance of HEVC RExt Profile with respect to HEVC without the Range Extensions tools and with respect to the AVC High 4:4:4 Predictive Profile. It also provides the performance gains of intra block copying for some natural content video sequences and analyses the complexity associated to this HEVC Range Extensions tool.

# Performance Comparisons

We use HM-12.1+RExt-5.1 and AhG8 lossy test conditions for these results.

## Performance of Range Extensions tools

For this test, the reference is compared to HEVC with the Range Extensions tools disabled:

- IntraBlockCopyEnabled=0

- ImplicitResidualDPCM=0

- ExplicitResidualDPCM=0

- ResidualRotation=0

- SingleSignificanceMapContext=0

- GolombRiceGroupAdaptation=0

- CrossComponentDecorrelation=0

Maximum transform skip size and intra reference smoothing are kept unchanged as in the reference (i.e., max transform skip size equals 4 and intra reference smoothing enabled).

Table 1 shows the results for this test.

Table 1. Losses incurred by disabling HEVC RExt tools

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 16.3% | 16.9% | 17.4% | 14.8% | 15.9% | 16.0% | 13.2% | 14.2% | 14.3% |
| Class B | 0.3% | 0.4% | 0.4% | 0.2% | 0.2% | 0.3% | 0.2% | 0.1% | 0.2% |
| RGB 4:4:4 SC | 97.3% | 97.3% | 98.0% | 91.4% | 91.6% | 91.8% | 85.6% | 84.3% | 84.6% |
| RGB 4:4:4 Animation | 39.3% | 38.5% | 36.6% | 38.2% | 36.0% | 34.8% | 33.2% | 30.1% | 29.5% |
| YCbCr 4:4:4 SC | 46.9% | 52.9% | 52.2% | 47.0% | 50.3% | 51.0% | 45.6% | 47.2% | 47.9% |
| YCbCr 4:4:4 Animation | 3.7% | 14.5% | 9.3% | 4.1% | 11.7% | 8.6% | 4.3% | 8.7% | 7.3% |
| RangeExt | 1.1% | 3.3% | 7.2% | 1.2% | 2.6% | 6.0% | 1.2% | 2.1% | 4.4% |
| RGB 4:4:4 SC (Optional) | 173.5% | 174.8% | 177.8% | 180.4% | 183.8% | 187.2% | 183.6% | 185.1% | 187.4% |
| YCbCr 4:4:4 SC (Option) | 131.8% | 136.2% | 131.7% | 135.5% | 137.2% | 136.5% | 134.9% | 136.0% | 137.3% |
| Enc Time[%] | 73% |  |  | 72% |  |  | 72% |  |  |
| Dec Time[%] | 110% |  |  | 111% |  |  | 112% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 11.7% | 12.6% | 12.8% | 10.5% | 11.4% | 11.4% |  |  |  |
| Class B | 0.1% | 0.3% | 0.3% | 0.1% | 0.1% | 0.2% |  |  |  |
| RGB 4:4:4 SC | 73.0% | 73.4% | 74.5% | 69.7% | 70.0% | 70.4% |  |  |  |
| RGB 4:4:4 Animation | 26.5% | 26.6% | 24.4% | 27.7% | 26.4% | 24.8% |  |  |  |
| YCbCr 4:4:4 SC | 32.7% | 40.3% | 40.6% | 33.5% | 37.7% | 39.0% |  |  |  |
| YCbCr 4:4:4 Animation | 1.5% | 13.6% | 7.5% | 2.0% | 12.4% | 7.9% |  |  |  |
| RangeExt | 0.5% | 5.0% | 7.2% | 0.6% | 3.5% | 7.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | 146.1% | 146.8% | 149.0% | 155.8% | 157.2% | 160.2% |  |  |  |
| YCbCr 4:4:4 SC (Option) | 107.9% | 113.4% | 109.6% | 111.7% | 114.5% | 113.9% |  |  |  |
| Enc Time[%] | 78% |  |  | 78% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 6.7% | 6.4% | 7.3% | 6.2% | 6.5% | 6.7% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | 55.5% | 53.1% | 54.3% | 55.0% | 53.2% | 53.8% |  |  |  |
| RGB 4:4:4 Animation | 23.9% | 21.1% | 19.3% | 24.9% | 21.2% | 19.9% |  |  |  |
| YCbCr 4:4:4 SC | 20.0% | 25.3% | 25.3% | 22.5% | 25.0% | 25.7% |  |  |  |
| YCbCr 4:4:4 Animation | 0.5% | 10.6% | 4.6% | 1.3% | 10.3% | 5.3% |  |  |  |
| RangeExt | 0.4% | 3.1% | 4.5% | 0.5% | 1.6% | 4.7% |  |  |  |
| RGB 4:4:4 SC (Optional) | 124.5% | 123.0% | 123.8% | 134.6% | 132.9% | 133.3% |  |  |  |
| YCbCr 4:4:4 SC (Option) | 87.3% | 92.0% | 88.3% | 94.7% | 95.3% | 94.8% |  |  |  |
| Enc Time[%] | 81% |  |  | 79% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |  |  |  |

## Performance of HEVC Range Extensions versus AVC High 4:4:4

For the AVC High 4:4:4 we use the results provided in JCTVC-O0186 [1], where the JM software was configured with the same coding structure and referencing structure as HM-12.1+RExt-5.1 using “HM-like” configurations.

Table 2 shows the results for this test.

Table 2. Gains of HEVC Range Extensions with respect AVC High 4:4:4

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F 4:2:0 SC | -38.1% | -35.6% | -36.5% | -40.2% | -33.1% | -33.7% | -40.2% | -31.2% | -31.9% |
| Class B 4:2:0 Natural | -22.8% | -19.3% | -18.0% | -19.5% | -19.4% | -19.4% | -13.5% | -17.5% | -19.6% |
| RGB 4:4:4 SC | -68.6% | -67.3% | -67.3% | -71.5% | -70.0% | -70.2% | -70.2% | -68.6% | -68.6% |
| RGB 4:4:4 Animation | -42.9% | -38.6% | -37.4% | -40.3% | -36.9% | -36.2% | -36.2% | -32.0% | -31.7% |
| YCbCr 4:4:4 SC | -52.1% | -48.7% | -50.6% | -54.2% | -48.9% | -50.8% | -53.9% | -48.7% | -49.9% |
| YCbCr 4:4:4 Animation | -25.1% | -29.3% | -25.1% | -22.9% | -27.8% | -24.9% | -20.6% | -23.9% | -22.0% |
| RangeExt 4:4:4 Natural | -20.9% | -14.2% | -17.5% | -17.0% | -14.6% | -18.4% | -12.0% | -12.6% | -14.6% |
| RGB 4:4:4 SC (Optional) | -89.1% | -88.9% | -89.2% | -88.3% | -88.3% | -88.3% | -86.8% | -86.3% | -86.3% |
| YCbCr 4:4:4 SC (Option) | -81.6% | -75.7% | -75.6% | -81.7% | -76.6% | -76.2% | -79.5% | -77.5% | -77.1% |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F 4:2:0 SC | -38.0% | -38.5% | -39.8% | -37.1% | -35.5% | -36.1% |  |  |  |
| Class B 4:2:0 Natural | -37.9% | -29.2% | -30.5% | -34.6% | -28.3% | -32.5% |  |  |  |
| RGB 4:4:4 SC | -62.9% | -60.3% | -61.0% | -62.4% | -60.5% | -61.0% |  |  |  |
| RGB 4:4:4 Animation | -43.2% | -42.4% | -40.4% | -39.9% | -39.4% | -37.4% |  |  |  |
| YCbCr 4:4:4 SC | -48.0% | -46.9% | -49.1% | -48.5% | -46.3% | -48.3% |  |  |  |
| YCbCr 4:4:4 Animation | -36.1% | -44.9% | -39.9% | -29.7% | -40.8% | -36.3% |  |  |  |
| RangeExt 4:4:4 Natural | -30.6% | -21.9% | -16.1% | -29.2% | -30.8% | -21.6% |  |  |  |
| RGB 4:4:4 SC (Optional) | -85.0% | -84.9% | -85.2% | -85.9% | -85.8% | -85.8% |  |  |  |
| YCbCr 4:4:4 SC (Option) | -74.9% | -73.0% | -72.4% | -76.8% | -74.0% | -73.3% |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F 4:2:0 SC | -38.0% | -39.2% | -41.9% | -36.6% | -35.0% | -36.4% |  |  |  |
| Class B 4:2:0 Natural | -39.2% | -21.3% | -23.5% | -35.0% | -21.1% | -25.2% |  |  |  |
| RGB 4:4:4 SC | -56.7% | -54.7% | -55.5% | -56.6% | -55.3% | -55.9% |  |  |  |
| RGB 4:4:4 Animation | -43.5% | -43.8% | -41.4% | -39.9% | -40.2% | -38.2% |  |  |  |
| YCbCr 4:4:4 SC | -46.6% | -45.4% | -47.5% | -46.6% | -44.1% | -46.1% |  |  |  |
| YCbCr 4:4:4 Animation | -38.3% | -55.2% | -52.3% | -32.1% | -48.9% | -45.9% |  |  |  |
| RangeExt 4:4:4 Natural | -31.5% | -30.3% | -27.0% | -31.9% | -33.7% | -30.8% |  |  |  |
| RGB 4:4:4 SC (Optional) | -82.2% | -81.7% | -82.5% | -84.3% | -84.1% | -84.1% |  |  |  |
| YCbCr 4:4:4 SC (Option) | -72.1% | -70.9% | -70.0% | -75.3% | -72.3% | -71.4% |  |  |  |

# Intra Block Copy

## Performance of Intra BC for natural content

Intra Block Copy (Intra BC) is an efficient coding tool that enables exploiting repetitive patterns in a frame. In this section, the performance of Intra BC is tested for natural content video sequences. Results are shown in Table 3. It should be noted that although the gains are not as significant as for screen content sequences, which is in the range of +20%, it is still quite attractive when comparing to other Intra coding tools in HEVC.

Table 3. Results of Intra BC for natural content

| Test sequences | All Intra Main-tier |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 1080p60_BQSquare | -2.4% | -2.5% | -2.9% |
| old_town_cross_444_720p50 | -2.2% | -1.4% | -2.3% |
| 720p55_Spincalendar | -5.7% | -5.3% | -5.9% |
| 1080p50_Cactus | -0.7% | -0.8% | -0.8% |
| station2_1080p25 | -0.5% | -0.4% | -0.4% |

## Complexity Analysis

Intra BC has been proved to be an efficient way for intra coding, and  no extra run time is needed for its decoding process, while its HW decoder cost has been mentioned in document JCTVC-O0083 which claimed Intra BC may introduce extra complexity in HW decoder implementation.

HW complexity analysis is discussed in the following. Figure 1 shows the intra BC HW block diagram using 4:4:4 and 10 bits per pixel as example.

Figure 1. Intra BC HW Block Diagram

Intra BC HW Cost Analysis:

  - Intra BC HW includes:

    - SRAM: store left reference LCU’s Y/Cb/Cr, and partial current LCU’s one component results

    - Read/write reference/reconstruction pixels related logic

    - Data Processing element: shared with reconstruction module

  - Storage related total cost:

    - SRAM: 3 for left LCU’s Y/Cb/Cr, and 1 auxiliary for the current LCU results

(3+1) x 64 x 64 x 10bit = 4 x 4096 x 10bit = 4 x 1024 x 40bit,

and adding about 5% for related read/write accessing logic as the total cost

  - Comparisons:

    - Based on real HEVC HW implementation, the Intra BC’s percentages are listed below:

      - about 4% of decoder area,

      - about 1% of combined encoder+decoder total area

    - Comparison Intra BC with DB/SAO’s implementation

|  | Decoder | Enc+Dec |
| --- | --- | --- |
| Intra BC | 4% | 1% |
| DB/SAO | 16.5% | 4.4% |

Note: above comparisons are based on Qualcomm’s HW HEVC codec implementation (by 20nm technology)

Conclusions on Intra BC complexity:

- From cost point of view, we can see that Intra BC’s storage accounts for only low percentage of total area of decoder or codec.

  - We can see that Intra BC’s cost is much lower than DB/SAO, but still can get impressive quality gain.

- From HW architecture point of view, if it is implemented in LCU pipeline style, then storage can be shared between stages of Intra BC and DB/SAO, which means no extra cost involved.

# References

[1] B. Li, J. Xu, G. Sullivan, “Comparison of Compression Performance of HEVC Range Extensions Draft 4 with AVC High 4:4:4 Predictive profile,” JCTVC-O0184, Geneva, CH, Nov. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).