| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0201 |
| --- | --- |

| Title: | AhG8: Performance of encoder and parameter only changes for Screen Content Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joel Sole, Chao Pang, Li Zhang, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution provides results of the objective compression performance of some encoder and parameter modifications of HEVC RExt Profile for screen content coding. BD-rate improvements of more than 20% are reported for the RGB 4:4:4 SC and the YUV 4:4:4 SC classes.

# Performance Comparisons

We use HM-12.1+RExt-5.1 and AhG8 lossy test conditions for these results. We introduce no new tools in the design. The modifications consist in encoder-only optimizations and changing parameters for some existing tools, for example, the following ones:

- Search area for intra BC is increased with respect to the reference

- Transform skip size is increased with respect to the reference

Table 1 shows the performance improvements obtained with these changes.

Table 1. BD-rate gains with respect to HM-12.1+RExt-5.1

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -5.9% | -5.0% | -5.0% | -5.5% | -4.7% | -4.8% | -5.1% | -4.6% | -4.7% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB 4:4:4 SC | -25.3% | -19.8% | -20.2% | -22.7% | -19.7% | -19.9% | -21.7% | -19.4% | -19.5% |
| RGB 4:4:4 Animation | -8.2% | 0.5% | 0.9% | -5.3% | 0.4% | 0.5% | -4.5% | 0.6% | 0.6% |
| YCbCr 4:4:4 SC | -20.2% | -19.0% | -19.2% | -20.3% | -19.5% | -19.6% | -20.2% | -19.6% | -19.7% |
| YCbCr 4:4:4 Animation | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |
| RangeExt | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -19.5% | -17.4% | -18.0% | -19.4% | -18.3% | -18.6% | -19.8% | -19.3% | -19.4% |
| YCbCr 4:4:4 SC (Option) | -18.7% | -18.0% | -18.0% | -20.3% | -19.4% | -19.7% | -21.5% | -20.7% | -21.0% |
| Enc Time[%] | 122% |  |  | 122% |  |  | 121% |  |  |
| Dec Time[%] | 77% |  |  | 79% |  |  | 81% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -7.1% | -8.3% | -7.9% | -7.1% | -8.4% | -8.0% |  |  |  |
| Class B | -1.3% | -4.5% | -5.4% | -1.3% | -4.6% | -6.4% |  |  |  |
| RGB 4:4:4 SC | -28.3% | -19.8% | -20.8% | -25.6% | -20.6% | -21.2% |  |  |  |
| RGB 4:4:4 Animation | -13.2% | -2.1% | 0.0% | -10.8% | -2.1% | -0.5% |  |  |  |
| YCbCr 4:4:4 SC | -22.2% | -18.8% | -19.4% | -22.2% | -20.2% | -20.5% |  |  |  |
| YCbCr 4:4:4 Animation | -1.8% | -1.7% | -1.9% | -1.9% | -2.2% | -2.1% |  |  |  |
| RangeExt | -1.9% | -2.3% | -4.4% | -1.0% | -1.5% | -4.9% |  |  |  |
| RGB 4:4:4 SC (Optional) | -16.8% | -14.0% | -14.4% | -16.1% | -14.5% | -14.6% |  |  |  |
| YCbCr 4:4:4 SC (Option) | -18.6% | -18.4% | -20.0% | -20.0% | -19.4% | -21.6% |  |  |  |
| Enc Time[%] | 93% |  |  | 94% |  |  |  |  |  |
| Dec Time[%] | 91% |  |  | 91% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -4.2% | -4.5% | -3.8% | -4.5% | -4.4% | -3.9% |  |  |  |
| Class B | -1.0% | -2.0% | -2.2% | -1.1% | -1.6% | -1.9% |  |  |  |
| RGB 4:4:4 SC | -24.5% | -14.8% | -15.5% | -21.7% | -15.6% | -15.9% |  |  |  |
| RGB 4:4:4 Animation | -13.5% | 1.0% | 2.2% | -10.6% | 1.0% | 1.6% |  |  |  |
| YCbCr 4:4:4 SC | -15.3% | -13.6% | -13.9% | -16.6% | -15.0% | -15.1% |  |  |  |
| YCbCr 4:4:4 Animation | -1.0% | 0.1% | -0.1% | -1.0% | -0.1% | -0.3% |  |  |  |
| RangeExt | -2.5% | -2.9% | -3.7% | -0.8% | -1.4% | -3.2% |  |  |  |
| RGB 4:4:4 SC (Optional) | -14.3% | -11.4% | -11.6% | -15.1% | -12.9% | -13.6% |  |  |  |
| YCbCr 4:4:4 SC (Option) | -14.6% | -14.0% | -14.7% | -16.1% | -15.5% | -16.0% |  |  |  |
| Enc Time[%] | 96% |  |  | 97% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  |  |  |  |

# Conclusions

Additional large coding performance gains can be obtained with the current HEVC Range Extensions design for screen content coding.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).