| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0202 |
| --- | --- |

| Title: | Non-RCE3: On Intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

In this contribution, a restriction on the maximum supported block size for an intra block copied block is introduced.  The case of 64x64 blocks is removed.  Although the current HM encoder does not search this case, the decoder must support the case.  Removing this case is asserted to simplify the decoder design.

# Introduction

In HM-12.1-RExt5.1, intra_bc_flag is signalled for all coding unit sizes, even though the encoder only tests this mode for 8x8 and 16x16 coding units.  Given the buffer dimensions of the current and the left CTU, when the coding unit size is equal to 64x64, the only valid block vector is one that copies the entire left CTU to the current CTU.

# Description of the proposed change

This contribution proposes to remove the signalling of intra block copy for the case of 64x64 coding units, as shown in Figure 1.

Figure 1. Case of 64x64 coding unit.

# Experiment Results

Testing conditions are defined in [3].

Table 1 and Table 2 show the results for using one context for intra_bc_flag under Screen content coding (AHG8) lossy and lossless conditions respectively.

Table 1. Context reduction results under Screen content coding (AHG8) lossy conditions with HM-12.1-RExt5.1 as anchor.

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC (Optional) | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  | 96% |  |  | 97% |  |  |
| Dec Time[%] | 91% |  |  | 93% |  |  | 95% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| RGB 4:4:4 SC | 0.2% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |  |  |  |
| RGB 4:4:4 Animation | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| YCbCr 4:4:4 SC | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% | -0.1% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.1% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | #VALUE! |  |  | #VALUE! |  |  |  |  |  |
| Dec Time[%] | #VALUE! |  |  | #VALUE! |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.1% | -0.3% | -0.1% | 0.1% | -0.1% | -0.1% |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -0.1% | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| RangeExt | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.5% | -0.4% | -0.5% | -0.3% | -0.2% | -0.3% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -0.4% | -0.3% | -0.3% | -0.6% | -0.8% | -0.7% |  |  |  |
| Enc Time[%] | #VALUE! |  |  | #VALUE! |  |  |  |  |  |
| Dec Time[%] | #VALUE! |  |  | #VALUE! |  |  |  |  |  |

Table 2. Context reduction results under Screen content coding (AHG8) lossless conditions with HM-12.1-RExt5.1 as anchor.

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 0.0% | -0.1% | 0.0% |
| YCbCr 4:4:4 SC (Optional) | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 99% | 101% |
| Dec Time[%] | 101% | 100% | 100% |

# Working Draft changes

Working draft text changes are on top of [2].

#### 7.3.8.5	Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag && log2CbSize < 6) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| … | … |
| } |  |

# Conclusion

This contribution proposes to remove the signalling to use intra block copy for coding units of size 64x64, thus preventing intra block copy from being applied in this case.

# References

- C. Pang, J. Sole, L. Guo, M. Karczewicz, R. Joshi “Non-RCE3: Intra Motion Compensation with 2-D MVs”, Joint Collaborative Team on Video Coding, JCTVC-N0256, Vienna, AT, July 2013.

- D. Flynn, J. Sole, T. Suzuki “Range Extensions Draft 5”, Joint Collaborative Team on Video Coding, JCTVC-O1005, Geneva, CT, October 2013.

- D. Flynn, K. Sharman, C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-O1006, 15th JCT-VC Meeting, Geneva, CH, October 2013.

- J. Sole, E. Alshina, D.-K. Kwon, W.-H. Peng, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra block copy refinement,” JCTVC-O1123, 15th JCT-VC Meeting, Geneva, CH, October 2013.

# Patent rights declaration(s)

Canon Information Systems Research Australia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).