| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-P0203 m32140 |
| --- | --- |

| Title: | Hybrid codec scalability profile in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Jack Enhorn Rickard Sjöberg Ericsson Research Färögatan 6 16480 Stockholm | Tel: Email: | +46761263591 jonatan.samuelsson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to include the hybrid codec scalability profile as described in JCTVC-O1012 into the SHVC draft with the following modifications:

- To remove the option of encapsulating AVC NAL units in HEVC NAL units (and just keep the two options of no encapsulation and encapsulating HEVC NAL units in AVC NAL units).

- To specify that the base layer must obey all constraints specified for the High profile in the AVC specification.

The contribution asserts that only one encapsulation format is needed and that it is important that the AVC NAL units are unmodified (i.e. no additional header is put in front of the AVC NAL unit header).

# Problem Statement

The question of how to realize hybrid codec scalability in SHVC with an AVC base layer has previously been brought up in JCTVC-O0190 [1] in which it was suggested to conceptually put HEVC NAL units inside AVC NAL units by using a "prefix-byte" in the HEVC NAL unit header that functions as an AVC NAL unit header. The proposal of [1] was included in JCTVC-O1012 [2] as one of two options for encapsulation together with the third option; no encapsulation (where codec identification is performed by external means). While having all these three options might have some advantage in flexibility there is also the risk of confusion and fragmentation in the market (different systems and/or applications using different schemes). Furthermore, having an encapsulation format that attaches additional data to AVC NAL units (i.e. two additional bytes before the AVC NAL unit header) may make decoding of the base layer impossible for receivers and decoders that are not capable of removing the extra data.

# Proposal

It is proposed to only have one encapsulation method alongside the option of performing codec identification of NAL units by external means.

In the proposed option for encapsulation a "prefix-byte" is added only for enhancement layer HEVC NAL units when the base layer is AVC as indicated in figure 1.

Figure 1. Illustration of the effect of the proposed prefix byte.

# Proposed SHVC specification text

The attached proposed text is based on JCTVC-O1012 which contains change marks relative to JCTVC-N1008_v3. The proposed changes on top of JCTVC-O1012 are marked with reviwer name "Jonatan Samuelsson".

# References

[1] 	JCTVC-O0190 “AHG15: AVC and HEVC encapsulation for hybrid codec scalability” J. Samuelsson, J. Enhorn, R. Sjöberg, Oct 2013

[2]	JCTVC-O1012 "Designs under study for SHVC hybrid scalability" 
J. Boyce, J. Samuelsson, Nov 2013

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).