| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San Jose, US, 9th – 17th, Jan. 2014 | Document: JCTVCP0205 |
| --- | --- |

| Title: | Non-RCE3: Intra block copy syntax clean up |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu 1, Kai Zhang 2, Xianguo Zhang2, Shan Liu1, Shawmin Lei1,2 1 MediaTek USA 2860 Junction Ave San Jose, CA 95134, USA 2 MediaTek Beijing Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 { xiaozhong.xu, kai.zhang, xianguo.zhang, liu.shan, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, the intra_bc_flag is changed from independent of pred_mode_flag to present only when pred_mode_flag equals to 1 (intra coded). The purpose of this cleanup operation is to categorize the intra block copy operation as an intra-picture operation. The simulation results show exactly the same AI coding performance as anchor, while the coding performance change for RA and LD configurations is negligible.

# Introduction

In the current HEVC range extensions draft specification [1], Intra block copying (Intra BC) was adopted to take advantage of reduplicated content in a picture. In the syntax design, the Intra BC enabling flag (intra_bc_flag) is signaled at coding unit level. This flag is put on top of intra/inter prediction mode flag (pred_mode_flag). Generally, a video coding tool is recognized as either intra-picture or inter-picture. There is some ambiguity on how to categorize the Intra BC operation. Conceptually, the Intra BC operation should be under the intra-picture category as it utilizes only the reconstructed information within the current slice. Therefore, it is more natural to make the signaling of Intra BC usage under the “intra branch”, rather than an independent coding mode.

# Proposed method

In this contribution, it is proposed to put intra_bc_flag dependent of pred_mode_flag. More specific, only when the current CU is coded as “intra”, the use of Intra BC mode is signaled. The corresponding change of syntax table is listed as follows:

Table 1 Proposed CU syntax in this contribution

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( intra_block_copy_enabled_flag && pred_mode_flag) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] ¦ ¦ (CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY) ) |  |
| part_mode | ae(v) |
| } |  |
| ...... |  |

pred_mode_flag is inferred to 1 when not present.

The binary tree of CU prediction modes in current RExt text is summarized in Fig.1. The proposed change is summarized in Fig. 2.

Fig. 1 Binary tree of CU prediction modes in current RExt.

Fig. 2 Binary tree of CU prediction modes in this contribution.

# Experimental results

The proposed methods are integrated on HM-12.1+RExt-5.1 [2].	Table 2 and Table 3 show the results for losses and lossy respectively.

Table 2. Lossless results

Table 3. Lossy results

Some summary of the simulation results:

- For lossless simulation,

  - The maximum bitrate increase ( lost in worst case) for this change is

    - 0.0% for mandatory sequences and 0.1% for optional sequences.

  - The maximum bitrate decrease (gain in best case) for this change is

    - 0.0% for mandatory sequences and 0.4% for optional sequences.

- For lossy simulation,

  - The maximum bitrate increase (lost in worst case) for this change is

    - 0.2% for all mandatory and optional sequences.

  - The maximum bitrate decrease (gain in best case) for this change is

    - 0.2% for mandatory sequences and 1.1% for optional sequences.

# Conclusion

In this proposal, intra block copy signaling flag is changed from independent of pred_mode_flag to be present only when pred_mode_flag is 1 (intra CU). By doing this, the syntax design of coding_unit is more clear and logic. Simulation results show that the proposed change has no impact on AI configurations while for RA and LD case, the maximum BD-rate increase is 0.2% and the maximum BD-rate decrease is 1.1%.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- D. Flynn, J. Sole, G. J. Sullivan, T. Suzuki, “HEVC Range Extensions Draft 5,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1005, Oct. 2013.

- HM-12.1+RExt-5.1, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-12.1+RExt-5.1