| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0207 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: San José, US, 11–17 Jan. 2014 | Document: JCT3V-G0166 |
| --- | --- |

| Title: | RExt/MV-HEVC/SHVC: On Auxiliary Alpha Plane Pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Sachin Deshpande and Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com sdeshpande@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a bitstream constraint that requires alpha plane auxiliary pictures associated with IDR primary pictures to be IDR. The proposed constraint guarantees that if random access is performed at the primary picture IDR then the corresponding alpha plane auxiliary picture is also a random access point.

# Introduction

In the current SHVC design [1] auxiliary pictures, including alpha plane auxiliary pictures, are coded in a separate layer from their associated primary pictures. However, no constraint is placed on the alignment of IDR pictures between alpha plane and their associated primary pictures. As a result, if an IDR primary picture is accessed in a random access fashion it is not guaranteed that the alpha plane auxiliary picture is also a random access point.

This contribution proposes a bitstream constraint that requires alpha plane auxiliary pictures associated with IDR primary pictures to be IDR.

# Proposed bitstream constraint

Listed below is the proposed bitstream constraint:

It is a requirement of bitstream conformance that when any associated primary picture of an auxiliary picture with AuxId[nuh_layer_id] equal to AUX_ALPHA has a nal_unit_type value nalUnitTypeA, equal to IDR_W_RADL or IDR_N_LP, then the nal_unit_type value for the corresponding auxiliary picture within the same access unit with AuxID[nuh_layer_id] equal to AUX_ALPHA shall be equal to nalUnitTypeA.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Jianle Chen, Jill Boyce, Yan Ye, Miska M. Hannuksela, Ye-kui Wang, “High efficiency video coding (HEVC) scalable extension Draft 4,” JCTVC-O1008_v3, 15th JCT-VC Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013