| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0208 |
| --- | --- |

| Title: | SHVC upsampling ratio constraint |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra and Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a bitstream constraint on the upsampling ratio for SHVC. It is asserted that the current SHVC draft allows the ratio of dimensions of the reference layer picture and its scaled representation to be greater than 1.In such an event the SHVC decoder operation is not clear. The proposed bitstream constraint bounds this ratio to be less than or equal to 1

# Introduction

In the current SHVC design [1] the ratio between the dimensions of the reference layer picture and its scaled representation can be greater than 1.This contribution proposes a bitstream constraint that requires the dimensions of the reference layer picture and its scaled representation be such that their ratio is always less than or equal to 1.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Jianle Chen, Jill Boyce, Yan Ye, Miska M. Hannuksela, Ye-kui Wang, “High efficiency video coding (HEVC) scalable extension Draft 4,” JCTVC-O1008_v3, 15th JCT-VC Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013