| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0209 |
| --- | --- |

| Title: | On chroma format scalability using spatial scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Sachin Deshpande and Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com sdeshpande@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes enabling chroma format scalability within the existing SHVC design through the use of spatial scalability. It is asserted that the desired functionality can be enabled with the proposed text.

# Introduction

In the current SHVC design [1] the spatial scaling operations for luma and chroma assumes that the chroma format of the reference and current layers are the same.  For example, a scalable bit-stream may contain original HD source content in a base layer and a super-resolved UHD version of the content in the enhancement layer.  In this case, the chroma format of the enhancement layer would be 4:2:0, while the chroma format of the reference layer may be 4:4:4.  Alternatively, a scalable bit-stream may contain 4:2:2 source data in the baselayer with a 4:2:0 enhancement layer.  Of course, other scenarios can also be envisioned that benefit from a mixture of 4:2:0, 4:2:2 and 4:4:4 in different layers.

In each example, we assert that the desired functionality can be enabled in SHVC by supporting different upsampling ratios in the luma and chroma channels.  This contribution proposes specification text to support this functionality.

# Summary of proposed changes

The spatial scaling process is modified to use the chroma formats of both the reference layer and current layer. Listed below is a summary of the proposed changes:

- The derivation process for reference layer sample location used in resampling is modified to use the chroma formats of the reference and the current layer

- The condition to check whether decoded reference layer picture may be assigned directly to resampled picture is modified to include a test that the chroma reference layer resolution and the scaled chroma reference layer resolution match.

- The upsampling of luma/chroma sample array is bypassed and the reference layer luma/chroma sample arrays are used if for the colour component under consideration the spatial scale factor is 1x, the scaled reference layer offsets are 0 and the luma/chroma bit depth for reference and current layer are the same.

- A conditional check is introduced to determine if the picture motion field of decoded reference layer picture may be assigned directly to the motion field of the resampled picture.

The exact specification text corresponding to the proposed modification is integrated within SHVC draft [1] and attached to the contribution.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Jianle Chen, Jill Boyce, Yan Ye, Miska M. Hannuksela, Ye-kui Wang, “High efficiency video coding (HEVC) scalable extension Draft 4,” JCTVC-O1008_v3, 15th JCT-VC Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013