| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0210 |
| --- | --- |

| Title: | RCE2: Cross-check of test A2 (JCTVC-P0168) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Liwei Guo | Tel: Email: | joels@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution verified the results of RCE2 test A2 (JCTVC-P0168) on Rice parameter initialization based on quantization parameter and bit-depth.

# Introduction

JCTVC-P0168 presents a method for Rice parameter initialization. The cross-checkers verified the R-D results of JCTVC-P0168. It is noted that besides modifying the Rice parameter initial value calculation, the max Rice parameter is also increased from 7 to 10.

# Experimental results

The JCTVC-PP0168 software provided by the proponents has been used to generate the results and compare with results generated with HM-12.1+RExt-5.1 under RCE2 common test conditions [1]. Encoding time and decoding time were obtained using heterogeneous cluster. It was confirmed that for the test case, the BD-rate results exactly match to the results provided by the proponents.

Table 1 Summary of AHG 8 Lossless Coding Results

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC (Optional) | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 109% | 104% | 105% |
| Dec Time[%] | 101% | 100% | 102% |

Table 2 Summary of AHG8 Lossy Coding Results

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB 4:4:4 SC | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | -0.3% | -0.3% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.4% | -0.5% | -0.5% |
| YCbCr 4:4:4 SC (Optional) | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.6% | -0.7% | -0.8% |
| Enc Time[%] | 100.1% |  |  | 99.8% |  |  | 99.3% |  |  |
| Dec Time[%] | 95.5% |  |  | 95.5% |  |  | 95.7% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | 0.0% | 0.1% | 0.0% | 0.1% | 0.2% | 0.1% |  |  |  |
| Enc Time[%] | 97.8% |  |  | 98.0% |  |  |  |  |  |
| Dec Time[%] | 100.0% |  |  | 100.2% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.1% | -0.1% | 0.0% | -0.4% | -0.3% | -0.2% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 98.8% |  |  | 98.7% |  |  |  |  |  |
| Dec Time[%] | 98.0% |  |  | 97.6% |  |  |  |  |  |

Table 3 Summary of AHG18 Coding Results (Transform Skip Enabled)

|  |  |  | Intra 16-bit (internal bit depth) |  |  | Intra 12-bit (internal bit depth) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | -2.9% | -3.0% | -3.0% | -0.1% | 0.0% | 0.0% |
| RExt RGB |  |  | -1.4% | -1.3% | -1.3% | 0.0% | 0.0% | 0.0% |
| HDR |  |  | 0.0% | 0.2% | 0.2% | 0.3% | 0.1% | 0.1% |
| 4K from HDR |  |  | -0.5% | -0.4% | -0.4% | -0.1% | 0.0% | 0.0% |
| HDR 4:0:0 Medical |  |  | 0.0% |  |  | 0.4% |  |  |
| 4:0:0 Medical |  |  | -0.6% |  |  | 0.1% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | -0.6% | -0.3% | -0.3% | 0.2% | 0.1% | 0.1% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0.4% | 0.4% | 0.4% | 0.3% | 0.1% | 0.1% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 1.1% | 0.6% | 0.7% | 0.2% | 0.1% | 0.1% |
| All Classes |  |  | -0.5% | -0.6% | -0.5% | 0.1% | 0.1% | 0.1% |
| Encoding Time % |  |  | 97% |  |  | 100% |  |  |
| Decoding Time % |  |  | 100% |  |  | 100% |  |  |

|  |  |  | Lowdelay 16-bit (internal bit depth) |  |  | Lowdelay 12-bit (internal bit depth) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | -0.9% | -0.8% | -0.8% | 0.0% | 0.0% | 0.0% |
| RExt RGB |  |  | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% |
| HDR |  |  | 0.2% | 0.2% | 0.2% | 0.1% | 0.0% | 0.0% |
| 4K from HDR |  |  |  |  |  |  |  |  |
| HDR 4:0:0 Medical |  |  | 0.1% |  |  | 0.1% |  |  |
| 4:0:0 Medical |  |  | -0.1% |  |  | 0.0% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0.2% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| All Classes |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Encoding Time % |  |  | 92% |  |  | 94% |  |  |
| Decoding Time % |  |  | 100% |  |  | 100% |  |  |

Table 4 Summary of AHG18 Coding Results (Transform Skip Disabled)

|  |  |  | Intra 16-bit (internal bit depth) |  |  | Intra 12-bit (internal bit depth) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | -3.0% | -3.0% | -3.0% | -0.1% | 0.0% | 0.0% |
| RExt RGB |  |  | -1.4% | -1.3% | -1.3% | 0.0% | 0.1% | 0.0% |
| HDR |  |  | 0.0% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% |
| 4K from HDR |  |  | -0.4% | -0.4% | -0.4% | 0.0% | 0.1% | 0.1% |
| HDR 4:0:0 Medical |  |  | 0.0% |  |  | 0.4% |  |  |
| 4:0:0 Medical |  |  | -0.6% |  |  | 0.1% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | -0.4% | -0.3% | -0.3% | 0.2% | 0.1% | 0.1% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0.3% | 0.4% | 0.4% | 0.2% | 0.1% | 0.1% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 1.1% | 0.6% | 0.8% | 0.2% | 0.1% | 0.1% |
| All Classes |  |  | -0.5% | -0.6% | -0.5% | 0.1% | 0.1% | 0.1% |
| Encoding Time % |  |  | 103% |  |  | 104% |  |  |
| Decoding Time % |  |  | 100% |  |  | 100% |  |  |

|  |  |  | Lowdelay 16-bit (internal bit depth) |  |  | Lowdelay 12-bit (internal bit depth) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | -0.9% | -0.8% | -0.8% | 0.0% | 0.0% | 0.0% |
| RExt RGB |  |  | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
| HDR |  |  | 0.2% | 0.2% | 0.2% | 0.1% | 0.0% | 0.0% |
| 4K from HDR |  |  |  |  |  |  |  |  |
| HDR 4:0:0 Medical |  |  | 0.1% |  |  | -0.1% |  |  |
| 4:0:0 Medical |  |  | -0.1% |  |  | 0.0% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 0.2% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| All Classes |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Encoding Time % |  |  | 102% |  |  | 100% |  |  |
| Decoding Time % |  |  | 100% |  |  | 100% |  |  |

# References

[1] C. Rosewarne, M. Karczewicz, K. Sharman, and S.-H. Kim, “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialization methods”, JCTVC-O1122, Geneva, Switzerland, Oct. 2013.