| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0212 |
| --- | --- |

| Title: | RCE3: preset of un-available for intra block copy samples |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. Alshina A. Alshin 416, Maetan 3-dong, Yeongtong-gu, Suwon, Korea, 443-742 | Tel: Email: | +82-10-3026-1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

Contribution reports test results for handling unavailable for intra block copy samples by pre-set them to mid grey value. Samples availability check is simplified compare to current intra block copy design. With relaxed restriction for intra block copy search up to 3,5% BD-rate gain was demonstrated.

# Introduction

Intra block copy is powerful tool for screen content coding. It was adopted as part of range extension of HEVC [1]. Later in order to insure copying from already decoded area [2] was adopted. In current specification draft encoder constrain forces intra block copy

    - Don't reference anything outside current CTU + Left CTU,

    - Don't reference anything outside current slice/tile,

    - Don't reference anything not-reconstructed yet.

These restrictions require samples availability check. Samples availability is specified in subclause Error: Reference source not found. But in case of intra block copy this availability check is not easy since for nCbS × nCbS coding unit availability of all (nCbS * nCbS) should be checked. This availability check becomes even more complex than intra block copy itself.

In [3] it was proposed to pre-set all un-available samples to mid grey value and so to avoid availability check.

# Proposal description

Proposed un-available for intra block copy samples handling requires following specification changes:

Derivation process for block vector components in intra block copying prediction mode

…

It is a requirement of bitstream conformance that all of the the following conditions are true:

–	The value of bvIntra[ 0 ] shall be greater than or equal to – ( xCb % CtbSizeY + CtbSizeY ).

–	The value of bvIntra[ 1 ] shall be greater than or equal to – ( yCb % CtbSizeY ).

–	The value of bvIntra[ 0 ] shall in a range

– CtbSizeY ≤ bvIntra[ 0 ] + (xCb% CtbSizeY) ≤ CtbSizeY –1– nCbS

–	The value of bvIntra[ 1 ] shall in a range

0≤ bvIntra[ 1 ] + (yCb% CtbSizeY) ≤ CtbSizeY –1– nCbS

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xCb + bvIntra[ 0 ], yCb + bvIntra[ 1 ] ) as inputs, the output shall be equal to TRUE.

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xCb + bvIntra[ 0 ] + nCbS − 1, yCb + bvIntra[ 1 ] + nCbS – 1 ) as inputs, the output shall be equal to TRUE.

–	One or both of the following conditions shall be true:

–	bvIntra[ 0 ] + nCbS  <=  0

–	bvIntra[ 1 ] + nCbS  <=  0

Specification of intra block copying prediction mode

…

- Each sample at the location ( xRefCmp, yRefCmp ) is assigned to predSamples[ x ][ y ].

- Non-decoded samples of the current picture shall be pre-set to (1<<(BitDepthY      -1)) for luma and (1<<(BitDepthC      -1)) for chroma.

So, intra block copy vector still refers to the area within 2 CTUs (current CTU and CTU at the left), but availability check is not required. Instead at the start of decoding process reconstructed signal shall be pre-set to mid grey value.

# Test results

RexT s/w doesn’t perform intra block copy search for candidates which contain at least one non-available sample.  So w/o search strategy modification proposed method has no impact to the performance. In RCE3 test C2…C4 for overlapping between candidate and predicted area:

(bvIntra[ 0 ] + nCbS)  > 0  || (bvIntra[ 1 ] + nCbS) >  0

padding different methods of padding were proposed. These algorithms also affect performance only with relaxed search for intra block copy vector. Our implementation exactly follows RCE3 test C4, but instead padding pre-set to mid grey value is used.

Test results are reported in Table 1 (for lossy) and in Table 2 (for lossless) AhG8 test configurations.

Some BD-rate gain is observed especially for screen content and optional classes. Maximum gain in lossy test is 3,5% (LD-B, high tier, sc_cad_waveform_1920x1080_20_8bit_200_rgb.rgb). Maximum gain in lossless tests is 2,0% (RA, sc_cad_waveform_1920x1080_20_8bit_200_444_r1.yuv).

Table 1. Pre-set on un-available for intra block copy samples with mid-grey value in lossy test.

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0,0% | -0,1% | -0,1% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| RGB 4:4:4 SC | -0,1% | -0,1% | -0,1% | -0,1% | -0,1% | -0,1% | 0,0% | 0,0% | 0,0% |
| RGB 4:4:4 Animation | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| YCbCr 4:4:4 SC | 0,0% | -0,1% | -0,1% | 0,0% | -0,1% | -0,1% | -0,1% | 0,0% | 0,0% |
| YCbCr 4:4:4 Animation | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| RangeExt | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| RGB 4:4:4 SC (Optional) | -0,3% | -0,3% | -0,3% | -0,2% | -0,2% | -0,3% | -0,2% | -0,2% | -0,2% |
| YCbCr 4:4:4 SC (Optional) | -0,1% | -0,1% | -0,1% | -0,2% | -0,2% | -0,2% | -0,2% | -0,2% | -0,1% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| AVG. SC | 0,0% | non SC | 0,0% |  |  |  |  |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0,0% | -0,1% | 0,1% | -0,1% | -0,1% | 0,0% |  |  |  |
| Class B | 0,0% | 0,0% | 0,2% | 0,0% | 0,1% | 0,1% |  |  |  |
| RGB 4:4:4 SC | -0,1% | -0,1% | -0,1% | -0,1% | -0,1% | -0,1% |  |  |  |
| RGB 4:4:4 Animation | 0,1% | 0,1% | 0,1% | 0,0% | 0,0% | 0,0% |  |  |  |
| YCbCr 4:4:4 SC | 0,1% | 0,0% | 0,0% | -0,1% | -0,1% | -0,1% |  |  |  |
| YCbCr 4:4:4 Animation | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% | 0,0% |  |  |  |
| RangeExt | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0,3% | -0,2% | -0,2% | -0,2% | -0,1% | -0,1% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -0,1% | -0,3% | -0,2% | -0,2% | -0,4% | -0,4% |  |  |  |
| Enc Time[%] | 99% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| AVG. SC | 0,0% | non SC | 0,0% |  |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0,1% | -0,7% | -0,2% | 0,0% | -0,3% | 0,0% |  |  |  |
| Class B | 0,0% | 0,0% | -0,2% | 0,0% | 0,0% | 0,0% |  |  |  |
| RGB 4:4:4 SC | -0,1% | -0,2% | -0,2% | -0,1% | -0,1% | -0,1% |  |  |  |
| RGB 4:4:4 Animation | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |  |  |  |
| YCbCr 4:4:4 SC | -0,1% | -0,2% | 0,1% | 0,0% | -0,1% | 0,0% |  |  |  |
| YCbCr 4:4:4 Animation | 0,0% | 0,1% | 0,0% | 0,0% | 0,1% | 0,0% |  |  |  |
| RangeExt | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0,8% | -0,9% | -0,9% | -1,2% | -1,2% | -1,1% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -0,4% | -0,4% | -0,4% | -0,1% | -0,2% | -0,3% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |
| AVG. SC | 0,0% | non SC | 0,0% |  |  |  |  |  |  |
| Best case | -3,5% | Worst case | 0,6% |  |  |  |  |  |  |

Table 2. Pre-set on un-available for intra block copy samples with mid-grey value in loss-less test.

|  | Average bit-rate increase |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | AI | RA | LB |  |  |
| Class F | 0,0% | 0,0% | 0,0% |  |  |
| Class B | 0,0% | 0,0% | 0,0% |  |  |
| RGB 4:4:4 SC | 0,0% | 0,0% | 0,0% |  |  |
| RGB 4:4:4 Animation | 0,0% | 0,0% | 0,0% |  |  |
| YCbCr 4:4:4 SC | 0,0% | 0,0% | 0,0% |  |  |
| YCbCr 4:4:4 Animation | 0,0% | 0,0% | 0,0% |  |  |
| RangeExt | 0,0% | 0,0% | 0,0% |  |  |
| RGB 4:4:4 SC (Optional) | -0,2% | -0,2% | -0,1% |  |  |
| YCbCr 4:4:4 SC (Optional) | -0,3% | -0,7% | -0,2% |  |  |
| Enc Time[%] | 100% | 98% | 98% |  |  |
| Dec Time[%] | 98% | 94% | 95% |  |  |
| Number of differences from anchor | 34 | 34 | 34 |  |  |
| Total differences | 102 |  |  |  |  |
| AVG. SC | 0,0% | nonSC | 0,0% |  |  |
| Best case | -2,0% | Worst case |  |  | 0,1% |

# Conclusion

Based on performance demonstrated

- Up to 3,5% gain in lossy test

- Up to 2,0% gain in lossless test

it is proposed to pre-set un-available for intra-block copy samples with mid-grey value and so avoid  samples availability check.

# References

- C. Pang, J. Sole, L. Guo, M. Karczewicz, R. Joshi, “Non-RCE3: Intra Motion Compensation with 2-D MVs,” Document of Joint Collaborative Team on Video Coding, JCTVC-N0256, Jul. 2013.

- B. Li, J. Xu, G. J. Sullivan (Microsoft), “On Intra BC mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0183, Oct. 2013

- E.Alshina, A.Alshin, S.Lee, “AhG5: Intra block copy within one LCU,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0074, Oct. 2013

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).