| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0215 |
| --- | --- |

| Title: | Tile Based Resampling for SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Karsten Sühring Yago Sanchez Thomas Schierl | Email: | robert.skupin@hhi.fraunhofer.de karsten.suehring@hhi.fraunhofer.de yago.sanchez@hhi.fraunhofer.de thomas.schierl@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

When performing tile- and layer-parallel processing of an SHVC encoded video sequence, the resampling process can affect parallelization as it does not regard tile boundaries. Constraining the encoder by Inter-Layer Constrained Tile Sets impacts compression efficiency. An alternative tile based resampling is proposed that enables the same degree of parallelism with lesser impact on compression efficiency.

A first revision of the document adds additional results of the Inter-Layer Constrained Tile Sets and the proposed scheme against anchor using the same tile configuration.

A second revision of the document adds further results on the coding penalty of constrained inter-layer prediction at tile boundaries on PU level and spatially coarser tile configurations.

# Introduction

When performing tile- and layer-parallel processing with an SHVC encoded video sequence, the resampling process affects parallelization. Resampling a reference layer picture does not regard the tile structure. Therefore, as illustrated in Figure 1, processing of the top leftmost current layer tile (dashed lines) requires all reference layer tiles (solid lines) to be decoded as the resampling process of the relevant reference picture area requires samples of neighbouring reference layer tiles.

Figure 1: Exemplary overlay of tile boundaries in reference (solid lines) and current layer (dashed lines).

This drawback for parallelization can be avoided through use of Inter-Layer Constrained Tile Sets to form an inter-layer prediction constraint tile sets out of each individual tile and enforce an encoder constraint preventing inter prediction from samples around tile boundaries [2]. The encoder constraint impacts compression efficiency.

The following reports results of SHVC encodings under the common test conditions with a 4x4 uniformly spaced tile structure in Base and Enhancement layer that form Inter-Layer Constrained Tile Sets using the reference software implementation in SHM 4.0 versus an anchor without Inter-Layer-Constraint Tile Sets.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 15,0% | 14,2% | 14,7% |  |  |  |
| Class B | 15,1% | 13,8% | 13,7% | 27,4% | 27,3% | 27,4% |
| Overall (Test vs Ref) | 15,1% | 13,9% | 14,0% | 27,4% | 27,3% | 27,4% |
| Overall (Test vs single layer) | 31,1% | 31,9% | 31,3% | 42,2% | 41,1% | 40,5% |
| Overall (Ref vs single layer) | 14,1% | 16,0% | 15,6% | 12,0% | 11,3% | 10,8% |
| EL only (Test vs Ref) | 30,0% | 28,3% | 28,4% | 106,6% | 103,5% | 102,9% |
| Enc Time[%] | 93,7% |  |  | 93,9% |  |  |
| Dec Time[%] | 110,8% |  |  | 111,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 8,9% | 3,3% | 3,7% |  |  |  |
| Class B | 9,7% | 4,7% | 3,5% | 19,2% | 14,1% | 12,5% |
| Overall (Test vs Ref) | 9,4% | 4,3% | 3,6% | 19,2% | 14,1% | 12,5% |
| Overall (Test vs single layer) | 33,4% | 42,6% | 39,9% | 42,2% | 51,8% | 49,4% |
| Overall (Ref vs single layer) | 21,9% | 37,1% | 35,3% | 19,5% | 33,9% | 33,6% |
| EL only (Test vs Ref) | 18,2% | 12,2% | 11,3% | 60,2% | 52,0% | 49,3% |
| Enc Time[%] | 98,4% |  |  | 102,5% |  |  |
| Dec Time[%] | 105,7% |  |  | 107,8% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 5,8% | 1,0% | 1,5% |  |  |  |
| Class B | 5,9% | 3,5% | 2,7% | 15,2% | 12,4% | 11,5% |
| Overall (Test vs Ref) | 5,9% | 2,8% | 2,3% | 15,2% | 12,4% | 11,5% |
| Overall (Test vs single layer) | 43,9% | 47,7% | 46,8% | 53,7% | 54,8% | 53,7% |
| Overall (Ref vs single layer) | 36,0% | 43,9% | 43,6% | 33,9% | 38,4% | 38,4% |
| EL only (Test vs Ref) | 11,6% | 8,1% | 7,6% | 53,7% | 49,1% | 47,5% |
| Enc Time[%] | 101,7% |  |  | 103,3% |  |  |
| Dec Time[%] | 107,8% |  |  | 104,9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

As can be seen from the above result, the inter-layer prediction constraint as implemented in SHM 4.0 has a significant impact on the coding performance of inter-layer prediction in general using a spatially relatively dense tile configuration 4x4. This is due to the inter-layer prediction being restricted on CU level.

# PU level inter-layer prediction constraint

The following reports results of a PU level implementation of the inter-layer prediction constraint using an aligned 2x2 tile configuration.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.4% | 1.7% | 1.9% |  |  |  |
| Class B | 1.9% | 2.4% | 2.3% | 2.4% | 3.3% | 3.3% |
| Overall (Test vs Ref) | 1.7% | 2.2% | 2.2% | 2.4% | 3.3% | 3.3% |
| Overall (Test vs single layer) | 15.1% | 17.3% | 16.9% | 13.6% | 13.5% | 12.9% |
| Overall (Ref vs single layer) | 13.2% | 14.9% | 14.5% | 11.0% | 9.9% | 9.4% |
| EL only (Test vs Ref) | 3.4% | 3.9% | 3.9% | 9.2% | 10.0% | 9.9% |
| Enc Time[%] | 99.0% |  |  | 101.7% |  |  |
| Dec Time[%] | 101.4% |  |  | 105.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.8% | 0.6% | 0.7% |  |  |  |
| Class B | 1.1% | 1.0% | 0.9% | 1.8% | 1.9% | 1.8% |
| Overall (Test vs Ref) | 1.1% | 0.9% | 0.8% | 1.8% | 1.9% | 1.8% |
| Overall (Test vs single layer) | 21.1% | 35.2% | 33.6% | 19.1% | 32.6% | 32.5% |
| Overall (Ref vs single layer) | 19.8% | 34.1% | 32.6% | 17.1% | 30.3% | 30.3% |
| EL only (Test vs Ref) | 2.0% | 1.8% | 1.7% | 5.9% | 5.8% | 5.6% |
| Enc Time[%] | 98.5% |  |  | 100.8% |  |  |
| Dec Time[%] | 98.2% |  |  | 100.6% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.5% | 0.3% | 0.4% |  |  |  |
| Class B | 0.7% | 0.6% | 0.6% | 1.4% | 1.4% | 1.4% |
| Overall (Test vs Ref) | 0.6% | 0.5% | 0.5% | 1.4% | 1.4% | 1.4% |
| Overall (Test vs single layer) | 34.5% | 40.9% | 40.9% | 33.1% | 35.8% | 36.0% |
| Overall (Ref vs single layer) | 33.7% | 40.2% | 40.1% | 31.4% | 34.0% | 34.3% |
| EL only (Test vs Ref) | 1.2% | 1.1% | 1.1% | 5.2% | 5.1% | 5.0% |
| Enc Time[%] | 102.1% |  |  | 100.4% |  |  |
| Dec Time[%] | 104.6% |  |  | 98.2% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

The following reports results of a PU level implementation  of the inter-layer prediction constraint using an aligned 3x2 tile configuration.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.8% | 2.3% | 2.4% |  |  |  |
| Class B | 2.4% | 3.1% | 3.1% | 3.1% | 4.3% | 4.3% |
| Overall (Test vs Ref) | 2.2% | 2.9% | 2.9% | 3.1% | 4.3% | 4.3% |
| Overall (Test vs single layer) | 15.8% | 18.4% | 18.0% | 14.6% | 14.9% | 14.4% |
| Overall (Ref vs single layer) | 13.4% | 15.1% | 14.7% | 11.2% | 10.3% | 9.8% |
| EL only (Test vs Ref) | 4.4% | 5.1% | 5.1% | 11.9% | 13.1% | 12.9% |
| Enc Time[%] | 98.1% |  |  | 102.4% |  |  |
| Dec Time[%] | 99.6% |  |  | 105.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 1.1% | 0.8% | 0.8% |  |  |  |
| Class B | 1.5% | 1.3% | 1.2% | 2.3% | 2.5% | 2.2% |
| Overall (Test vs Ref) | 1.4% | 1.2% | 1.1% | 2.3% | 2.5% | 2.2% |
| Overall (Test vs single layer) | 21.8% | 36.2% | 34.5% | 20.1% | 34.0% | 33.7% |
| Overall (Ref vs single layer) | 20.2% | 34.7% | 33.1% | 17.5% | 30.9% | 31.0% |
| EL only (Test vs Ref) | 2.7% | 2.4% | 2.3% | 7.5% | 7.5% | 7.1% |
| Enc Time[%] | 101.8% |  |  | 100.6% |  |  |
| Dec Time[%] | 102.1% |  |  | 100.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.7% | 0.5% | 0.5% |  |  |  |
| Class B | 0.9% | 0.7% | 0.8% | 1.8% | 1.8% | 1.7% |
| Overall (Test vs Ref) | 0.8% | 0.6% | 0.7% | 1.8% | 1.8% | 1.7% |
| Overall (Test vs single layer) | 35.2% | 41.8% | 41.7% | 34.0% | 37.3% | 37.7% |
| Overall (Ref vs single layer) | 34.1% | 40.9% | 40.7% | 31.8% | 34.9% | 35.4% |
| EL only (Test vs Ref) | 1.6% | 1.4% | 1.5% | 6.6% | 6.4% | 6.2% |
| Enc Time[%] | 99.4% |  |  | 101.2% |  |  |
| Dec Time[%] | 97.7% |  |  | 99.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

- Proposal

This document proposes an alternative tile based resampling that enables the same degree of parallelism as Inter-Layer Constrained Tile Sets scheme with lesser impact on compression efficiency [3].

It is proposed to allow resampling of reference layer pictures based on the tile structure of the current layer, i.e. the relevant reference layer picture areas for each tile of the current layer is resampled individually.

Results of the proposed tile based resampling solution with respect to the same anchors as in the above Inter-Layer Constrained Tile Sets schemes are reported in the following.

The following reports results for a 4x4 tile configuration.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,3% | 0,3% | 0,3% |  |  |  |
| Class B | 0,8% | 0,7% | 0,7% | 0,2% | 0,2% | 0,2% |
| Overall (Test vs Ref) | 0,7% | 0,6% | 0,6% | 0,2% | 0,2% | 0,2% |
| Overall (Test vs single layer) | 14,9% | 16,6% | 16,3% | 12,3% | 11,5% | 11,0% |
| Overall (Ref vs single layer) | 14,1% | 16,0% | 15,6% | 12,0% | 11,3% | 10,8% |
| EL only (Test vs Ref) | 1,2% | 1,1% | 1,1% | 0,6% | 0,7% | 0,7% |
| Enc Time[%] | 100,6% |  |  | 100,5% |  |  |
| Dec Time[%] | 99,6% |  |  | 96,6% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,2% | 0,2% | 0,2% |  |  |  |
| Class B | 0,7% | 1,0% | 0,7% | 0,1% | 0,1% | 0,1% |
| Overall (Test vs Ref) | 0,5% | 0,7% | 0,5% | 0,1% | 0,1% | 0,1% |
| Overall (Test vs single layer) | 22,5% | 38,1% | 36,1% | 19,6% | 34,0% | 33,7% |
| Overall (Ref vs single layer) | 21,9% | 37,1% | 35,3% | 19,5% | 33,9% | 33,6% |
| EL only (Test vs Ref) | 0,9% | 1,2% | 0,9% | 0,3% | 0,3% | 0,3% |
| Enc Time[%] | 101,1% |  |  | 101,3% |  |  |
| Dec Time[%] | 102,9% |  |  | 99,5% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,1% | 0,1% |  |  |  |
| Class B | 0,5% | 0,6% | 0,5% | 0,1% | 0,1% | 0,0% |
| Overall (Test vs Ref) | 0,4% | 0,5% | 0,4% | 0,1% | 0,1% | 0,0% |
| Overall (Test vs single layer) | 36,5% | 44,6% | 44,1% | 34,1% | 38,5% | 38,4% |
| Overall (Ref vs single layer) | 36,0% | 43,9% | 43,6% | 33,9% | 38,4% | 38,4% |
| EL only (Test vs Ref) | 0,7% | 0,8% | 0,7% | 0,3% | 0,3% | 0,2% |
| Enc Time[%] | 100,5% |  |  | 103,7% |  |  |
| Dec Time[%] | 99,5% |  |  | 98,2% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

The following reports results on the proposed tile based resampling a 2x2 tile configuration.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.2% |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Overall (Test vs Ref) | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Overall (Test vs single layer) | 13.3% | 15.0% | 14.6% | 11.1% | 10.1% | 9.5% |
| Overall (Ref vs single layer) | 13.2% | 14.9% | 14.5% | 11.0% | 9.9% | 9.4% |
| EL only (Test vs Ref) | 0.2% | 0.2% | 0.2% | 0.2% | 0.3% | 0.3% |
| Enc Time[%] | 109.4% |  |  | 99.6% |  |  |
| Dec Time[%] | 111.8% |  |  | 94.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Overall (Test vs Ref) | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Overall (Test vs single layer) | 19.9% | 34.1% | 32.7% | 17.2% | 30.3% | 30.3% |
| Overall (Ref vs single layer) | 19.8% | 34.1% | 32.6% | 17.1% | 30.3% | 30.3% |
| EL only (Test vs Ref) | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 99.3% |  |  | 100.7% |  |  |
| Dec Time[%] | 96.8% |  |  | 97.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Overall (Test vs single layer) | 33.8% | 40.2% | 40.2% | 31.4% | 34.1% | 34.4% |
| Overall (Ref vs single layer) | 33.7% | 40.2% | 40.1% | 31.4% | 34.0% | 34.3% |
| EL only (Test vs Ref) | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |
| Enc Time[%] | 98.1% |  |  | 97.1% |  |  |
| Dec Time[%] | 93.7% |  |  | 93.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

The following reports results on a 3x2 tile configuration.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.2% | 0.2% |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |
| Overall (Test vs Ref) | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |
| Overall (Test vs single layer) | 13.5% | 15.3% | 14.9% | 11.3% | 10.4% | 9.9% |
| Overall (Ref vs single layer) | 13.4% | 15.1% | 14.7% | 11.2% | 10.3% | 9.8% |
| EL only (Test vs Ref) | 0.2% | 0.2% | 0.2% | 0.3% | 0.3% | 0.4% |
| Enc Time[%] | 104.1% |  |  | 102.8% |  |  |
| Dec Time[%] | 100.5% |  |  | 97.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.1% |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Overall (Test vs Ref) | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Overall (Test vs single layer) | 20.3% | 34.8% | 33.2% | 17.6% | 31.1% | 31.1% |
| Overall (Ref vs single layer) | 20.2% | 34.7% | 33.1% | 17.5% | 30.9% | 31.0% |
| EL only (Test vs Ref) | 0.2% | 0.1% | 0.2% | 0.2% | 0.2% | 0.2% |
| Enc Time[%] | 98.8% |  |  | 96.5% |  |  |
| Dec Time[%] | 95.9% |  |  | 92.7% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% |
| Overall (Test vs Ref) | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% |
| Overall (Test vs single layer) | 34.2% | 41.0% | 40.8% | 31.9% | 35.1% | 35.5% |
| Overall (Ref vs single layer) | 34.1% | 40.9% | 40.7% | 31.8% | 34.9% | 35.4% |
| EL only (Test vs Ref) | 0.1% | 0.0% | 0.1% | 0.2% | 0.2% | 0.1% |
| Enc Time[%] | 100.4% |  |  | 101.2% |  |  |
| Dec Time[%] | 95.4% |  |  | 94.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Results of the proposed tile based resampling solution with respect to an Inter-Layer Constrained Tile Sets scheme under the common test conditions with a 4 x 4 tile structure in base and enhancement layer are reported in the following.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -12.8% | -12.1% | -12.5% |  |  |  |
| Class B | -12.1% | -11.3% | -11.1% | -21.0% | -21.0% | -21.0% |
| Overall (Test vs Ref) | -12.3% | -11.5% | -11.5% | -21.0% | -21.0% | -21.0% |
| Overall (Test vs single layer) | 14.9% | 16.6% | 16.3% | 12.3% | 11.5% | 11.0% |
| Overall (Ref vs single layer) | 31.1% | 31.9% | 31.3% | 42.2% | 41.1% | 40.5% |
| EL only (Test vs Ref) | -21.1% | -20.2% | -20.2% | -44.5% | -44.2% | -44.2% |
| Enc Time[%] | 107.3% |  |  | 107.0% |  |  |
| Dec Time[%] | 89.9% |  |  | 86.7% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -8.0% | -3.0% | -3.4% |  |  |  |
| Class B | -8.0% | -3.4% | -2.6% | -15.6% | -11.6% | -10.4% |
| Overall (Test vs Ref) | -8.0% | -3.3% | -2.8% | -15.6% | -11.6% | -10.4% |
| Overall (Test vs single layer) | 22.5% | 38.1% | 36.1% | 19.6% | 34.0% | 33.7% |
| Overall (Ref vs single layer) | 33.4% | 42.6% | 39.9% | 42.2% | 51.8% | 49.4% |
| EL only (Test vs Ref) | -13.9% | -9.1% | -8.6% | -32.7% | -28.7% | -27.4% |
| Enc Time[%] | 102.7% |  |  | 98.9% |  |  |
| Dec Time[%] | 97.3% |  |  | 92.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -5.2% | -0.8% | -1.3% |  |  |  |
| Class B | -5.0% | -2.6% | -2.1% | -12.6% | -10.4% | -9.8% |
| Overall (Test vs Ref) | -5.1% | -2.1% | -1.9% | -12.6% | -10.4% | -9.8% |
| Overall (Test vs single layer) | 36.5% | 44.6% | 44.1% | 34.1% | 38.5% | 38.4% |
| Overall (Ref vs single layer) | 43.9% | 47.7% | 46.8% | 53.7% | 54.8% | 53.7% |
| EL only (Test vs Ref) | -9.1% | -6.2% | -5.9% | -28.1% | -25.8% | -25.2% |
| Enc Time[%] | 98.8% |  |  | 100.4% |  |  |
| Dec Time[%] | 92.3% |  |  | 93.6% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Corresponding comparisons of tile based resampling against a PU level implementation of the inter-layer prediction constraint are reported in the following for the 2x2 tile configuration.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.2% | -1.5% | -1.7% |  |  |  |
| Class B | -1.8% | -2.3% | -2.2% | -2.3% | -3.1% | -3.1% |
| Overall (Test vs Ref) | -1.6% | -2.1% | -2.0% | -2.3% | -3.1% | -3.1% |
| Overall (Test vs single layer) | 13.3% | 15.0% | 14.6% | 11.1% | 10.1% | 9.5% |
| Overall (Ref vs single layer) | 15.1% | 17.3% | 16.9% | 13.6% | 13.5% | 12.9% |
| EL only (Test vs Ref) | -3.1% | -3.5% | -3.5% | -7.8% | -8.5% | -8.4% |
| Enc Time[%] | 110.5% |  |  | 97.9% |  |  |
| Dec Time[%] | 110.2% |  |  | 89.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.7% | -0.5% | -0.6% |  |  |  |
| Class B | -1.1% | -1.0% | -0.8% | -1.7% | -1.8% | -1.7% |
| Overall (Test vs Ref) | -1.0% | -0.9% | -0.7% | -1.7% | -1.8% | -1.7% |
| Overall (Test vs single layer) | 19.9% | 34.1% | 32.7% | 17.2% | 30.3% | 30.3% |
| Overall (Ref vs single layer) | 21.1% | 35.2% | 33.6% | 19.1% | 32.6% | 32.5% |
| EL only (Test vs Ref) | -1.9% | -1.7% | -1.6% | -5.2% | -5.2% | -5.0% |
| Enc Time[%] | 100.8% |  |  | 99.9% |  |  |
| Dec Time[%] | 98.7% |  |  | 96.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.2% | -0.3% |  |  |  |
| Class B | -0.6% | -0.6% | -0.5% | -1.3% | -1.3% | -1.3% |
| Overall (Test vs Ref) | -0.6% | -0.5% | -0.5% | -1.3% | -1.3% | -1.3% |
| Overall (Test vs single layer) | 33.8% | 40.2% | 40.2% | 31.4% | 34.1% | 34.4% |
| Overall (Ref vs single layer) | 34.5% | 40.9% | 40.9% | 33.1% | 35.8% | 36.0% |
| EL only (Test vs Ref) | -1.1% | -1.0% | -1.0% | -4.5% | -4.4% | -4.3% |
| Enc Time[%] | 96.1% |  |  | 96.7% |  |  |
| Dec Time[%] | 89.6% |  |  | 95.2% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

The following reports corresponding results for a 3x2 tile configuration

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.6% | -2.1% | -2.2% |  |  |  |
| Class B | -2.2% | -2.9% | -2.9% | -3.0% | -4.0% | -4.0% |
| Overall (Test vs Ref) | -2.0% | -2.7% | -2.7% | -3.0% | -4.0% | -4.0% |
| Overall (Test vs single layer) | 13.5% | 15.3% | 14.9% | 11.3% | 10.4% | 9.9% |
| Overall (Ref vs single layer) | 15.8% | 18.4% | 18.0% | 14.6% | 14.9% | 14.4% |
| EL only (Test vs Ref) | -4.0% | -4.6% | -4.6% | -9.9% | -10.8% | -10.7% |
| Enc Time[%] | 106.1% |  |  | 100.4% |  |  |
| Dec Time[%] | 100.9% |  |  | 91.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -1.0% | -0.8% | -0.8% |  |  |  |
| Class B | -1.4% | -1.2% | -1.1% | -2.2% | -2.3% | -2.1% |
| Overall (Test vs Ref) | -1.3% | -1.1% | -1.0% | -2.2% | -2.3% | -2.1% |
| Overall (Test vs single layer) | 20.3% | 34.8% | 33.2% | 17.6% | 31.1% | 31.1% |
| Overall (Ref vs single layer) | 21.8% | 36.2% | 34.5% | 20.1% | 34.0% | 33.7% |
| EL only (Test vs Ref) | -2.4% | -2.2% | -2.1% | -6.5% | -6.5% | -6.1% |
| Enc Time[%] | 97.0% |  |  | 95.9% |  |  |
| Dec Time[%] | 93.9% |  |  | 92.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | -0.3% | -0.4% |  |  |  |
| Class B | -0.8% | -0.7% | -0.7% | -1.7% | -1.7% | -1.7% |
| Overall (Test vs Ref) | -0.8% | -0.6% | -0.6% | -1.7% | -1.7% | -1.7% |
| Overall (Test vs single layer) | 34.2% | 41.0% | 40.8% | 31.9% | 35.1% | 35.5% |
| Overall (Ref vs single layer) | 35.2% | 41.8% | 41.7% | 34.0% | 37.3% | 37.7% |
| EL only (Test vs Ref) | -1.5% | -1.3% | -1.3% | -5.6% | -5.4% | -5.4% |
| Enc Time[%] | 101.0% |  |  | 100.0% |  |  |
| Dec Time[%] | 97.6% |  |  | 95.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Signalling for the tile based resampling in the SPS extension is proposed in the following.

| sps_multilayer_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| resampling_across_tiles_enabled_flag | u(1) |
| num_scaled_ref_layer_offsets | ue(v) |
| for( i = 0; i < num_scaled_ref_layer_offsets; i++) { |  |
| scaled_ref_layer_id[ i ] | u(6) |
| scaled_ref_layer_left_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_top_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_right_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_bottom_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| } |  |
| } |  |

resampling_across_tiles_enabled_flag equal to 1 specifies that resampling operations on reference pictures of pictures referring to the PPS are performed across tile boundaries. resampling_across_tiles_enabled_flag equal to 0 specifies that resampling operations on reference pictures are not performed across the tile boundaries of pictures referring to the PPS. When not present, the value of resampling_across_tiles_enabled_flag is inferred to be equal to 1.

Corresponding additions to the resampling process for inter layer reference pictures can be found attached based on [1].

# References

- J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, Y.-K. Wang (Eds.), “High efficiency video coding (HEVC) scalable extension Draft 4 “, JCTVC-O1008_v3, Geneva, CH, Nov. 2013

- K. Ugur, M. M. Hannuksela (Nokia), K. Suehring, R. Skupin, Y. Sanchez (FhG HHI), K. Rapaka, J. Chen (Qualcomm), C. Auyeung, S. Hattori (Sony), “Indication of inter-layer and motion constrained prediction constraints”, JCTVC-N0383, Vienna, AT, July 2013

- K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (FhG HHI), “Independent tile upsampling for SHVC”, JCTVC-N0086, Vienna, AT, July 2013

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).