| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0217-v2 |
| --- | --- |

| Title: | Initialization of block vector predictor for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lihua Zhu Jizheng Xu Gary J. Sullivan Sridhar Sankuratri B. Anil Kumar Feng Wu Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA | Email: | lizhu@microsoft.com jzxu@microsoft.com garysull@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

In the current intra block copy design, the block vector (BV) prediction value (BVx, BVy) is initalized to (0, 0) at the beginning of each CTU, and the BV is subsequently predicted based on the value of the vector for the preceding CU within the same CTU. It is noted that the initialization value (0, 0) is not actually an allowed value for the BV, and it is thus always necessary for a non-zero BV difference to be sent for the first CU in each CTU. This contribution proposes alternatives for for initialization of the BV predictor. The proposal is suggested to be motivated primarily as a clean-up fix.

The proposed preferred approach is to note that the value of the vertical component of the BV affects the validity of the horizontal component and that the coding efficiency of test sequences with strong vertical motion suffers if the initialization value assumes horizontal-dominant motion. Thus, the suggestion for initialization of the BV value is to initialize the vertical BV prediction to 0 and decode its residual difference, and then set the initial horizontal BV prediction to 0 if the vertical component is sufficiently large to allow that value to be valid (i.e., when the vertical component is less than −H, where H is the height of the CU), and otherwise set the horizontal BV prediction to −2*W, where W is the width of the CU. Preliminary testing indicates a small-but-significant coding efficiency benefit for this method (approximately 0.2% average benefit for the tested YUV screen content coding cases, and generally little impact on performance for other cases).

A slightly simpler alternative is to always initialize the BV value to (−2*W, 0), perhaps with clipping for the largest CU size to ensure validity of that value. Statistical analysis and coding efficiency test results are reported, and the coding efficiency impact of this change is reportedly generally slightly beneficial, although acknowledged to be very small (approximately 0.1% average benefit for the tested YUV screen content coding cases, and generally no impact on performance for other cases).

It is further noted that when the CU size is equal to the CTU size, the only allowed value for the BV is (−W, 0), unless overlapping or extrapolation are supported (which are currently not allowed in the design). To account for this, it is suggested that two adjustments may be appropriate in this case –clipping the initialization of the BV prediction value to (−W, 0) in this case, and not sending a BV difference (since the difference would never be allowed to be non-zero).

A "ping-pong" approach with an encoder-selectable predictor for BV prediction is also described. This approach reportedly provides some additional gain (approximately 0.5% average benefit for the tested YUV and RGB screen content coding cases, and generally little impact on performance for other cases).

# Proposed approaches

Proposal 1: A fixed initialization value for each CU size (the simplest approach)

In the HEVC Range Extension, the Intra Block Copy method uses the vector of the last coded CU within the current CTU as the block vector predictor. This prediction improves compression performance. When the predictor is not available, e,g, for the first CU with intra block copy, currently (0, 0) is used as default. However, the value (0, 0) does not make sense for intra block copy. Thus, we propose to use a meaningful value as the default. One potential initization value can be a horizontal vector (BVx, 0), where BVx is a fixed value or the width of the current CU, which is the smallest block vector to avoid overlap between the current CU and the reference block. Likewise, a vertical vector (0, BVy) could also be a default predictor. We have analyzed all of screen content test sequences, and the selected BV values for more than 80% of BVx are smaller than −12, i.e., their absolute values are bigger than 12.

Candidate solution 1 for proposal 1: We use the offset of minus 2 times the width of the predicted CU as the default block vector predictor. Here we use W to indicate the width of a CU.

Thus the proposed default PBVx (horizontal) and PBVy (vertical) components of the block vector (BV) are (PBVx, PBVy) = (−2*W, 0).

Candidate solution 2 for proposal 1: For the largest CU size, since the horizontal element of the BV is not allowed to be less than –W in this case, the prediction can be set to −W instead in this case.

Both candidate solutions use a fixed initialization value based only on the width of the CU. Candidate solution 2 is suggested as preferred, as the complexity seems the same for the two solutions.

Candidate solution 3 for proposal 1: We use the offset of the minus width of the predicted CU as the default block vector predictor. Here we use W to indicate the width of a CU.

Thus the proposed default PBVx (horizontal) and PBVy (vertical) components of the block vector (BV) are (PBVx, PBVy) = (-W, 0).

Proposal 2: Set the horizontal predictor based on the vertical component value

The proposed preferred approach is to note that the value of the vertical component of the BV affects the validity of the horizontal component and that the coding efficiency of test sequences with strong vertical motion suffer if the initialization value assumes horizontal-dominant motion. Thus, the suggestion for initialization of the BV value is to initialize the vertical BV prediction to 0 and decode its residual difference, and then set the initial horizontal BV prediction to 0 if the vertical component is sufficiently large to allow that value to be valid (i.e., when the vertical component is less than −H, where H is the height of the CU), and otherwise set the horizontal BV prediction to −2*W, where W is the width of the CU. Preliminary testing indicates a small-but-significant coding efficiency benefit for this method.

Proposal 3: A "Ping-Pong" block vector predictor as described below:

When a new block vector predictor is produced, the PMV1 will be replaced by the PMV0. And the PMV0 is replaced by the new block vector predictor.

On the encoder site, the encoder will select the best PMV as the block vector prediction from the Ping-Pong block vector predictor sets , and an additional PMV flag will indicate which block vector should be used.

On the decoder site, the decoder parses the PMV flag to decide which PMV should be selected, and use the selected PMV as the block vector predictor.

We need additional memory to keep one additional PMV for this approach.

# Proposed text changes for the simplest approach

#### 7.4.9.9 Motion vector difference semantics

abs_mvd_greater0_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 0.

abs_mvd_greater1_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 1.

When abs_mvd_greater1_flag[ compIdx ] is not present, it is inferred to be equal to 0.

abs_mvd_minus2[ compIdx ] plus 2 specifies the absolute value of a motion vector component difference.

When abs_mvd_minus2[ compIdx ] is not present, it is inferred to be equal to −1.

mvd_sign_flag[ compIdx ] specifies the sign of a motion vector component difference as follows:

- If mvd_sign_flag[ compIdx ] is equal to 0, the corresponding motion vector component difference has a positive value.

- Otherwise (mvd_sign_flag[ compIdx ] is equal to 1), the corresponding motion vector component difference has a negative value.

When mvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

The motion vector difference lMvd[ compIdx ] for compIdx = 0..1 is derived as follows:

lMvd[ compIdx ] = abs_mvd_greater0_flag[ compIdx ] *
		( abs_mvd_minus2[ compIdx ] + 2 ) * 	( 1 − 2 * mvd_sign_flag[ compIdx ] ) 	(7-1)

The variable MvdLX[ x0 ][ y0 ][ compIdx ], with X being 0 or 1, specifies the difference between a list X vector component to be used and its prediction. The value of MvdLX[ x0 ][ y0 ][ compIdx ] shall be in the range of −215 to 215 − 1, inclusive. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

The variable BvIntra[ x0 ][ y0 ][ compIdx ] specifies the vector used for the intra block copying prediction mode.  The value of BvIntra[ x0 ][ y0 ] shall be in the range of 0 to 128, inclusive. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

The variable LastMvIntra[ compIdx ] specifies the last decoded vector for the intra block copying prediction mode of the current CTB. The variable LastMvIntra[ compIdx ] is set equal to (−2 * nCbS, 0) for the first decoded Intra block copy prediction of the current CTB. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

- If refList is equal to 0, MvdL0[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 1), MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 2),

  - BvIntra[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] + LastMvIntra[ compIdx ] for compIdx = 0..1.

  - LastMvIntra[ compIdx ] is set equal to BvIntra[ x0 ][ y0 ][ compIdx ] for compIdx = 0..1.

# Simulation results

Table 1 shows the coding results for proposal 1. More results will be presented in a revised version of this document.

Table 1. Summary of simulation results for proposal 1. Detailed results can be found in the accompanying excel file.

Table 2 shows the coding results for solution 3 of the proposal 1. More results will be presented in a revised version of this document.

Table 2. Summary of simulation results for solution 3 of proposal 1. Detailed results can be found in the accompanying excel file.

Table 3 shows the coding results for proposal 2. More results will be presented in a revised version of this document.

Table 3. Summary of simulation results for proposal 2. Detailed results can be found in the accompanying excel file.

Table 4 shows the coding results for proposal 3. More results will be presented in a revised version of this document.

Table 4. Summary of simulation results for proposal 3. Detailed results can be found in the accompanying excel file.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).