| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0218 |
| --- | --- |

| Title: | Non-RCE3 subtest B.2 – Results and Search Methods for Intra block copying for CU-level block vectors with TU-level prediction processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lihua Zhu Ji-Zheng Xu Gary Sullivan | Tel: Email: | +1 425 882 8080 lizhu@microsoft.com jzxu@microsoft.com garysull@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

In the current Intra Block Copying (intra BC) design, the block vector value is sent at the CU level and the prediction process is specified to operate at the TU level. The question thus arises whether it is allowed for there to be any overlap between the TU-level referenced block regions and the region covered by the current CU (but outside the current TU). This can be allowed if intra BC prediction is performed on a TU-by-TU basis, which is asserted to be well aligned to the way that intra modes ordinarily operate. Prohibiting overlap within a CU would prevent the current block from referencing nearby areas, which would be undesirable from a coding efficiency perspective. RCE3 subtest B.2 was planned to investigate this ability to perform IntraBC at the TU-level and measure the associated performance.

Encoding search techniques are described in this contribution to perform the selection of CU-level Intra BC block vector (BV) values that are appropriate for use with such overlapping regions. (This contribution does not consider techniques that require decoder-side region padding, as overlapping within the TU is not considered.)

Using the best-performing of the described (suboptimal) encoding search techniques, an average luma coding efficiency benefit of about 0.9% is reportedly achieved for the tested YUV and RGB screen content test sequences (mandatory and optional sequences for the CE) by enabling the prediction process on a TU basis when the encoder search is performed as described. It is asserted that additional benefit could be obtained with more sophisticated search techniques.

Based on these results, it is advocated that block vector values should not be constrained to prohibit overlapping regions within a CU (although, as tested, they are constrained to prohibit overlapping regions within a TU).

# Introduction and Problem Statement

We propose to operate the prediction process on a TU-by-TU basis within the CU, to enable prediction for block vector (BV) values that create an overlap of the CU and the referenced block region (but not an overlap of the TU and the referenced block region). This can allow the reconstruction of Intra BC over the overlapped area.

In addition, we describe some (non-normative) search methods for identifying BV values for use with this type of operation.

We consider two different search methods to search for BV values with overlap during Intra BC motion search:

- The first BV search method uses the block from the original picture as the reference block when the reconstructed block is not available during Intra BC motion search.

- The second search method integrates the transform, quantization, de-quantization and inverse transform inside TU instead of CU to achieve the better gains.

In the first method, the overlap area in the reference picture is filled with samples from the original input picture for encoding search decision purposes. The motion search is still CU based. After the motion search, the reconstruction is performed on the TU level. Besides, all of TUs share the same BV value. For example, if the CU is 8x8, the 8x8 transform is skipped, and only TU 4x4 is applied, and meanwhile the reconstruction procedure is taken on the TU 4x4 level, and so the transform, quantization, de-quantization, inverse transform and other reconstruction related are performed on the TU4x4 level. In the example, TU 4x4 0 is first to be processed and the reconstruction procedure is performed on this TU unit.

Note, from the figure below that only the lower-right TU is problematic for overlap. Since we assume that overlap within the same TU is prohibited, only BV values for which the lower-right TU (TU number 3 in the figure) references the gray areas need to be considered.

If there is overlap, the prediction process in the other three blocks can all be carried out together.

For the second method, which performs better but requires extra processing, the overlap area samples are filled with samples that are the result of additional processing. We apply simple BV prediction from the neighbor block and then a reconstruction procedure is performed to get the processed original sample shown. The left is the same as the first method. In the below figure, block 4x4 (0) takes the left neighbor as the prediction to obtain the residue and then the transform, quantization, De-quantitation and inverse transform are performed, and finally, the Intra BC displacement compensation is applied to get the reconstruction picture.

We also apply special handling for cases where the number of coefficients for a TU after quantization is zero. This allows skipping of subsequent processing steps.

As shown in the figure below, only references to the shaded areas can involve overlap within a CU.

# Simulation results

Simulation results are provided in the attached Excel file.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).