| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0219 |
| --- | --- |

| Title: | Cross-check report of ' RCE2: Results of Test 1 on Rice Parameter Initialization' (JCTVC-P0199) by Qualcomm and Sony |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides a cross-check of RCE2 test A1 by Qualcomm and Sony.  In test A1, the Rice parameter is initialised based on statistics of the coefficient magnitudes.  The cross check result confirms the simulation result reported by Qualcomm and Sony.

# Cross check procedure

This is a cross-check of RCE2 test A1, as described in proposal JCTVC-P0199 [1].

The source code for JCTVC-P0199 was provided by Qualcomm and Sony. Canon compiled the source code using GCC 4.7.1 and the simulation was performed on a Linux cluster under the AHG18 conditions, as defined in the RCE description [2]. Note that timing results are not comparable due to differences in the simulation environment.

# Analysis

The source code was studied and the following observations were made:

- The Rice parameter update mechanism adopted at the previous JCT-VC meeting [3] is disabled, reverting to the usual HM Rice parameter update mechanism, however the maximum Rice parameter value of 7 is retained.

- Four statistic counters are maintained, recording luma vs chroma and transformed vs non-transformed sub-blocks.

- For each sub-block, the Rice parameter is initialised with the appropriate statistic counter (divided by GOLOMB_STAT_DELAY or 4).

- For the first coefficient to include coeff_abs_level_remaining (i.e. uiLevel) in each sub-block, the appropriate statistic counter is updated as follows:

  - If uiLevel exceeds an upper threshold, the statistic counter is incremented. The upper threshold is dependent on the statistic counter value.

  - If uiLevel is below a lower threshold, the statistic counter is decremented.  The lower threshold is dependent on the statistic counter threshold.

- The range of each statistic counter is restricted to [0, 31] so 5-bits are required for each statistic counter.

These observations are consistent with the description in [1].

# Coding performance measurement

Table 1 and Table 2 provide the cross-check results for JCTVC-P0199 under AHG18 conditions, with transform skip enabled and disabled, respectively.

Table 3 and Table 4 provide the cross-check results for JCTVC-P0199 under AHG8 conditions, with lossy and lossless results presented respectively.

Table 1. JCTVC-P0199 AHG18 simulation result.

|  |  |  | Average BD-rate Change (%) - All Tiers |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra 16-bit (internal bit depth) |  |  | Intra 12-bit (internal bit depth) |  |  | Lowdelay 16-bit (internal bit depth) |  |  | Lowdelay 12-bit (internal bit depth) |  |  |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | -2.7% | -2.3% | -2.3% | -1.1% | -0.8% | -0.8% | -1.9% | -1.9% | -2.0% | -0.8% | -0.8% | -0.9% |
| RExt RGB |  |  | -1.5% | -1.2% | -1.2% | 0.1% | 0.2% | 0.2% | -1.1% | -1.1% | -1.1% | 0.0% | 0.0% | 0.0% |
| HDR |  |  | 0.2% | 0.4% | 0.3% | 0.3% | 0.3% | 0.3% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| 4K from HDR |  |  | -1.6% | -1.2% | -1.1% | -0.1% | 0.2% | 0.2% |  |  |  |  |  |  |
| HDR 4:0:0 Medical |  |  | -0.6% |  |  | 0.0% |  |  | -0.3% |  |  | 0.0% |  |  |
| 4:0:0 Medical |  |  | 1.6% |  |  | 0.9% |  |  | 1.8% |  |  | 0.5% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | -1.1% | -0.7% | -0.7% | 0.0% | 0.1% | 0.0% | -0.5% | -0.4% | -0.4% | 0.0% | 0.1% | 0.1% |
| Synthetic RGB HDR (+4 MSBs) |  |  | -0.3% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% |
| All Classes |  |  | -0.7% | -0.7% | -0.7% | 0.0% | 0.0% | 0.0% | -0.3% | -0.6% | -0.6% | -0.1% | -0.2% | -0.2% |
| Encoding Time % |  |  | 102% |  |  | 101% |  |  | #NUM! |  |  | #NUM! |  |  |
| Decoding Time % |  |  | 105% |  |  | 103% |  |  | #NUM! |  |  | #NUM! |  |  |

Table 2. JCTVC-P0199 AHG18 simulation result with transform skip disabled.

|  |  |  | Average BD-rate Change (%) - All Tiers |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra 16-bit (internal bit depth) |  |  | Intra 12-bit (internal bit depth) |  |  | Lowdelay 16-bit (internal bit depth) |  |  | Lowdelay 12-bit (internal bit depth) |  |  |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | -2.6% | -2.3% | -2.3% | -1.1% | -0.8% | -0.8% | -1.9% | -1.9% | -1.9% | -0.8% | -0.8% | -0.8% |
| RExt RGB |  |  | -1.4% | -1.2% | -1.1% | 0.1% | 0.2% | 0.2% | -1.0% | -1.0% | -1.0% | 0.1% | 0.1% | 0.0% |
| HDR |  |  | 0.2% | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| 4K from HDR |  |  | -1.4% | -1.0% | -1.0% | -0.1% | 0.2% | 0.2% |  |  |  |  |  |  |
| HDR 4:0:0 Medical |  |  | -0.7% |  |  | 0.0% |  |  | #DIV/0! |  |  | -0.2% |  |  |
| 4:0:0 Medical |  |  | 1.6% |  |  | 0.9% |  |  | 1.8% |  |  | 0.6% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | -1.1% | -0.7% | -0.7% | 0.0% | 0.1% | 0.0% | -0.5% | -0.4% | -0.4% | 0.0% | 0.0% | 0.0% |
| Synthetic RGB HDR (+4 MSBs) |  |  | -0.3% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| All Classes |  |  | -0.6% | -0.7% | -0.7% | 0.0% | 0.0% | 0.0% | #DIV/0! | -0.6% | -0.6% | 0.0% | -0.1% | -0.1% |
| Encoding Time % |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Decoding Time % |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3. JCTVC-P0199 AHG8 lossy simulation result.

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.4% | -0.5% | -0.5% | -0.7% | -0.8% | -0.8% | -1.1% | -1.2% | -1.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.5% | -0.6% | -0.6% | -1.2% | -1.2% | -1.1% | -2.0% | -2.3% | -2.2% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.4% | -0.5% | -0.4% | -0.9% | -0.9% | -0.8% | -1.6% | -1.7% | -1.7% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -3.7% | -3.5% | -3.5% | -5.8% | -5.8% | -5.9% | -7.9% | -8.2% | -8.2% |
| YCbCr 4:4:4 SC (Optional) | -1.4% | -1.5% | -1.5% | -2.7% | -2.8% | -2.7% | -4.5% | -4.6% | -4.8% |
| Enc Time[%] | 74.0% |  |  | 73.7% |  |  | 73.4% |  |  |
| Dec Time[%] | #VALUE! |  |  | #VALUE! |  |  | #VALUE! |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -0.3% | -0.4% | -0.5% | -0.5% | -0.6% |  |  |  |
| Class B | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% |  |  |  |
| RGB 4:4:4 SC | -0.5% | -0.5% | -0.4% | -1.0% | -0.9% | -0.9% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | 0.2% | 0.2% | 0.1% | -0.3% | -0.3% | -0.3% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -3.9% | -3.7% | -3.7% | -6.1% | -6.0% | -6.2% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -1.2% | -1.2% | -1.2% | -2.4% | -2.3% | -2.4% |  |  |  |
| Enc Time[%] | 80.9% |  |  | 80.4% |  |  |  |  |  |
| Dec Time[%] | #VALUE! |  |  | #VALUE! |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -0.5% | 0.2% | -0.2% | -0.4% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -0.5% | -0.5% | -0.5% | -0.9% | -0.8% | -0.8% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.1% | -0.1% | -0.2% | -0.3% | -0.3% | -0.3% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -3.2% | -3.4% | -3.2% | -5.3% | -5.3% | -5.5% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -1.5% | -1.6% | -1.4% | -2.2% | -2.6% | -2.3% |  |  |  |
| Enc Time[%] | 80.7% |  |  | 80.2% |  |  |  |  |  |
| Dec Time[%] | #VALUE! |  |  | #VALUE! |  |  |  |  |  |

Table 4. JCTVC-P0199 AHG8 lossless simulation result.

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | -0.7% | -0.5% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -3.0% | -2.8% | -2.7% |
| RGB 4:4:4 Animation | -0.3% | -0.3% | -0.3% |
| YCbCr 4:4:4 SC | -2.6% | -2.0% | -2.4% |
| YCbCr 4:4:4 Animation | -0.2% | -0.1% | -0.2% |
| RangeExt | -1.3% | -0.4% | -0.4% |
| RGB 4:4:4 SC (Optional) | -8.7% | -8.7% | -9.2% |
| YCbCr 4:4:4 SC (Optional) | -6.9% | -6.9% | -7.1% |
| Enc Time[%] | 99% | 100% | 100% |
| Dec Time[%] | 105% | 101% | 102% |

# Conclusion

This cross-check includes results for RCE2 Test A1 provided by Qualcomm and Sony. The provided software matches the description in the contribution. The cross-check result confirms the simulation result reported by the proponent.

# References

- C. Rosewarne, M. Karczewicz, K. Sharman, S.-H. Kim “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialization methods,” JCTVC-O1122, 15th JCT-VC meeting, Geneva, CH, Oct. 2013.

- M. Karczewicz, L. Guo, J. Sole, K. Sharman, N. Saunders, J. Gamei, “RCE2: Results of Test 1 on Rice Parameter Initialization,”, JCTVC-P0199, 16th JCT-VC Meeting: San Jose, US, Jan 2014.

- J. Sole, M. Karczewicz, “RCE2 Test A1: Simplified update of the coefficient level Rice parameter, ” JCTVC-O0206, 15th JCT-VC Meeting: Geneva, CH, Oct 2013.