| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0222 |
| --- | --- |

| Title: | AhG 18: On SAO performance at high bit-depth and high bit-rate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. Alshina, A. Alshin 416, Maetan 3-dong, Yeongtong-gu, Suwon, Korea, 443-742 Woo-Shik Kim, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA Alexis Michael Tourapis Apple Inc. 1 Infinite Loop Cupertino CA Yu-Wen Huang, Shih-Ta Hsiang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +82-10-3026-1305 elena_a.alshina@samsung.com +1-858-845-8893 wooshikk@qti.qualcomm.com +1 408 228 7983 atourapis@apple.com +886 3 5670766 ext. 26297 yuwen.huang@mediatek.com shih-ta.hsiang@mediatek.com |
| Source: | Samsung Electronics, Ltd., Qualcomm Inc, Apple Inc, MediaTek Inc. |  |  |

_____________________________

# Abstract

SAO performance is analysed under AhG8 and AhG18 test conditions. It is reported that currently, for 16 and 12 bit sequences, the average coding gain observed for SAO gain is 0%. Two reasons for poor SAO performance at high bit-rates and bit-depths were identified: quantization for sao_abs_values and non-robust edge classification. It is shown that quantization removal for sao_abs_values can provide a 2.1% and a 0.5% average gain in the 12 bit LD and AI tests respectively. Modification of the edge classification process on top of the quantization removal, in the tests performed, does not appear to provide any additional gain and further investigation may be recommended.

# Introduction

The sample adaptive offset (SAO) is a new coding tool that was introduced in version 1 of the High Efficiency Video Coding (HEVC) standard. SAO is a mechanism that was added after the in-loop deblocking filter of HEVC that tries to reduce distortion by adding an offset to each sample based on a predefined classification/categorization process. To improve performance, a set of SAO parameters is signaled for each LCU, while advanced prediction mechanisms between LCUs are utilized to reduced the overhead related to signalling these parameters.

HEVC supports two different types of classification. A band offset (BO) method, which classifies the samples based on their magnitude, and an edge offset (EO) method, where classification is based on a simple edge analysis technique around each sample.

It was reported in several JCT-VC contributions that SAO performance at high bit-rates and high bit-depths is poorer than expected [1] [2].  This contribution analyses reasons for this phenomenon and suggests solutions.

# SAO-off tests results

Two types of tests are typically used in RExt development: AhG8 and AhG18 test conditions.

AhG8 tests include both screen content and camera captured content, while the internal bit-depth is set to 8 for the majority of the tests and 10 for the “RangeExt” test category. The QP range used is 22 to 37 for the Medium Tier (MT) tests, 17 to 32 for the High Tier (HT) tests, and 12 to 27 for the Super High Tier (SHT, only all-intra) tests. The PSNR of the reconstructed video is in the interval of 33dB to 61dB (MT and HT).

On the other hand, AhG18 tests include camera captured content, medical images and synthetic high dynamic range content. For the synthetic, high dynamic range content category, 2, 4, or 6 MSBs are added to the original 10 bit videos. The internal bit-depth is 12 and 16 in AhG 18 tests. RExt__HIGH_BIT_DEPTH_SUPPORT should be enabled for AhG18 tests. The QP range is between -16 and -37 for the 16 bit test, and between 8 and -12 for the 12 bit test. The quality of the reconstructed video is very high in these tests. PSNR is in the 55dB to 75dB and 78dB to 99dB intervals for the 12 and 16 bit tests respectively.

The brief summary of the tests conditions can be found in Table 1. It should be mentioned that neither the AhG8 nor the AhG18 test conditions can cover all practical application scenarios.

Details of the SAO–off tests under AhG 8 and AhG 18 test conditions can be found in the appendix and the excel spreadsheet attached to this contribution.

Table 1.  Test conditions and SAO performance gain

| AhG | Internal bitDepth | QP | PSNR, dB | SAO gain non- screen content | SAO gain screen content |
| --- | --- | --- | --- | --- | --- |
| 8 | 8 | 37…12 | 32…53 | 0.9% | 4.7% |
| 8 | 10 | 37…12 | 30…43 | 0.9% |  |
| 18 | 12 | 8…–12 | 55...75 | <0.1% |  |
| 18 | 16 | –16…–37 | 78…99 | 0.0% |  |

# Key points of HEVC SAO design and suggested modifications

In SAO, the samples of the reconstructed picture are first classified into several categories, while correcting offsets are extracted from the bit-stream and added onto the reconstructed picture samples after deblocking.

The magnitude of the coded offset values is limited to a maximum value of

saoOffsetAbsThr = (1 << (Min(bitDepth, 10) – 5)) – 1,                                      (1)

whereas the sign values are either signaled in the bit stream (for the BO method), or predefined for each edge category.

The final offset value is computed as

SaoOffset = offsetSign * saoOffsetAbs << ( bitDepth − Min( bitDepth, 10 ) ),                    (2)

where offsetSign is the specified sign for an offset, saoOffsetAbs is the signalled offset magnitude, and bitDepth is the specified bitdepth of the samples processed. In an example, the offset range for 8 bit data is within [-7, 7], for 10 bit data is within [-31,31], whereas for 12 bit data the range is within [-31,31] * 4.

Quantization for saoOffsetAbs values is applied starting from bitDepth>10.

Edge classification rules are as follows:

z1 = px − px-1; z2 = px − px+1

edgeIdx = 2  + Sign(z1)  +  Sign( z2)                       (3)

If edgeIdx results in 2 then the local area is considered to be monotonic and not corrected by an edge offset.

Therefore, the difference of 1 in neighbour samples values affects classification. If a sequence is very noisy then taking into account this small difference of 1 can be quite risky since edgeIdx could be different from a value of 2 even for non edge samples.

Table 1 reports very poor SAO performance in AhG 18 tests. Similar results were presented earlier in [1] and [2]. In the HM reference s/w, SAO parameters are decided as follows. The accumulated error divided by the number of samples in each category gives a correcting offset value for each LCU. This value is 1) rounded, 2) quantized, and then 3) clipped to the range of -31…31 for bitDepth>10. The analysis of the SAO parameters at the encoder side shows that before rounding (step 1), SAO parameters values are in the range of -3 to 3 for most of the 16 bit AhG18 tests and in the range of -7 to 7 for the 12 bit AhG18 tests. After quantization (step 2), the saoOffsetAbs values become all equal to zero and therefore the encoder ends up turning SAO off. This is the reason for poor SAO performance in AhG 18 tests.

Two ideas pop up in this regard.

The first is to remove quantization (highlighted in the yellow part of (2)). This simplifies the decoder, prevents saoOffsetAbs from being always zero and thus can improve SAO performance.

Another idea suggested in [1] and [2] is to modify the edge offset classification process as described in (3) and make it more robust for the encoding of higher bit-depth data. This can be done using a formulation of edgeIdx as:

edgeIdx = 2  + Sign( (z1+roundOffset)>>N )  +  Sign( (z2+roundOffset)>>N )     	(4)

assuming an arithmetic shift operation, or alternatively as:

edgeIdx = 2 + Sign( ((px )>>N ) − ((px-1 )>>N )) + Sign( ((px )>>N ) − ((px+1 )>>N )) 	(5)

or as:

edgeIdx = 2  + Sign( z1) * (Abs(z1) >>N )  +  Sign( z2) * (Abs (z2)>>N )       	(6)

Here N is ( bitDepth − Min( bitDepth, 10 ) ) and roundOffset = (1<<(N-1)). Therefore, these methods introduce a quantization process prior to the classification stage.

Since the proposed methods in this contribution were studied only under AhG 18 test conditions, which do not cover all practical applications, it is proposed to introduce a flag in slice segment header which can turn on and off the proposed modifications.

# Test results

First, quantization removal, i.e.:

SaoOffset = offsetSign * saoOffsetAbs << ( bitDepth − Min( bitDepth, 10 ) ),

was tested.

Then, in addition to the previous modification the EO sample classification process modification was also tested under the same AhG 18 test conditions. Both tested modifications do not affect performance under the AhG 8 test conditions.

No encoder modifications (such as lambda change) were used in the presented tests.

A brief summary of the tested results is presented in Table 2.

Table 2.  Test results for proposed SAO modifications

| Test | AI-16 | AI-12 | LD-16 | LD-12 |
| --- | --- | --- | --- | --- |
| SAO disabled | 0.0% | 0.0% | 0.0% | 0.0% |
| no SAO parameter quantization | 0.0% | -0.5% | -0.2% | -2.1% |
| no SAO parameter quantization + EO classification modified (4) | 0.0% | -0.3% | 0.0% | -1,0% |
| no SAO parameter quantization + EO classification modified (5) | 0.0% | -0.2% | 0,0% | -1,4% |

A maximum gain of 7.8% using the quantization removal process was achieved in the LD 12 bit test for the “EBURainFruits_10bit_RGB+6 MSBs” sequence.

The maximum gain from both modifications, i.e. the quantization removal + EO classification modification (5), was found to be equal to 5,4% for the LD 12 bit test for the “EBURainFruits_10bit_RGB+4 MSB” sequence. It should be noted that combining both results together, i.e. by allowing a sequence adaptation between the two methods, does allow for further gains.

The test results can be summarized as follows:

- SAO performance was studied under AhG 18 and AhG 8 test conditions for different internal bitDepth values and QP intervals.

- For internal bitDepth 8 and QP=12…37, the average SAO gain for camera captured content is roughly 0.9% and 4.7% for screen content.

- For internal bitDepth 10 and QP=12…37, the average SAO gain for camera captured content is roughly 0.9%.

- For internal bitDepth 12 and QP=–12…8, the average SAO gain for camera captured and medical content is lower than 0.1%.

- For internal bitDepth 16 and QP=–16…–37, the average SAO gain for camera captured and medical content is 0.0%.

- Quantization removal increases the average SAO gain for internal bitDepth 12 with QP=–12…8 to 1.3% and for bitDepth 16 with QP=–16…–37 to 0.1%.

In the current implementation, it appears that the edge classification modification seems to provide minimal performance improvement, i.e. by switching the process on and off, on top of the quantization removal in AhG18 tests.

# Additional test results

During discussion of this contribution additional test results were requested. It was requested to study the performance in tests using internal bitDepth higher than 10, but not that high bit-rate compare to AhG18 test conditions.

Additional tests were performed using the same test sequences as AhG18 uses but with QP=17…38 for both internal bitDepth 12 and 16.

During these tests preparation s/w bug (RdCost overflow) was identified and fixed by MedaTek’s experts. First performance impact of this bug fix was studied and confirmed that only bitDepth = 16 tests with QP higher than AhG18 used (performance difference appears starting from QP=17 in our tests and results in up to 60% BDrate gain).

Performance of SAO quantization removal at AhG18 QP range has been confirmed on top of bug-fix.

At QP range 17…38 SAO quantization removal leads in avg to 0,8% drop in 16 bits test and 0,4% in 12 bits tests. This confirms necessity of the control for SAO quantization bits.

Additionally performance of slice level control for SAO quantization bits was investigated. This technology was proposed in [3] before. Quantization bits for SAO parameters are signalled in slice header for Luma and Chroma. Current implementation follows [3]. Encoder calculates the current picture RD cost resulting from different parameter values and then selects the parameter leading to the lowest RD cost. The SAO process at the current HM encoder can be broke into three steps:

1.      Perform SAO pixel classification and gather statistics

2.      Determine and entropy code SAO parameters for each CTU & calculate the picture RD cost

3.      Perform SAO filtering using the selected SAO parameters

In current implementation for slice-level SAO bits adaptation encoder needs to perform Steps 1 & 3 once and  Step 2 multiple times, each for one candidate parameter value. So, the slice header parameter is determined by the picture-based RD optimization. Such restrictions can be removed by using the statistics gathered from the previous coded pictures to determine the slice header SAO parameter for the current picture.

Performance impact of picture based SAO RDO optimization is 0,1% in all-intra tests and 0,4% in LD tests.

Slice level adaptive SAO quantization bits provides 0,7% and 0,6% average BD-rate gain in LD test on top of picture based SAO RDO optimization.

Additional test results are summarized in Table 3.

Table 3. Additional test results.

| Test | AI-16 | AI-12 |  | LD-16 | LD-12 |
| --- | --- | --- | --- | --- | --- |
| AhG 18 QPs (-16…-37 16b, 8…-12 12b) |  |  |  |  |  |
| Rext 5.0.1 vs Rext 5.0.1 bf | 0.0% | 0.0% | 0.0% |  | 0.0% |
| Rext 5.0.1 bf vs SAO no Quant | 0.0% | -0.5% | -0.2% |  | -2.1% |
| QP = 17…38 (PSNR 27…61 dB) |  |  |  |  |  |
| Rext 5.0.1 vs Rext 5.0.1 bf | 0%…-60% | 0,0% |  | 0%…-60% | 0,0% |
| Rext 5.0.1 bf vs SAO noQ | 0,3% | 0,1% |  | 1,2% | 0,6% |
| Rext 5.0.1 bf vs SAO pic RDO | -0.1% | -0.1% |  | -0,4% | -0,4% |
| Rext 5.0.1 bf vs SAO Adapt.Q (both pic RDO) | 0.0% | 0.0% |  | -0,7% | -0,6% |

# Conclusion

Based on the test data provided it is proposed

- Fix s/w bug which leads to overflow in RDCost calculation

- to remove the quantization step of the SAO magnitude value for higher bitdepth content.

- to add the control quantization step of the SAO magnitude value on picture level.

- to continue investigating the quantization process for the Edge Classification method using different quantization values and adaptation mechanisms.

# References

- W.-S. Kim, J. Sole, M. Karczewicz, “AhG 5 and 18: Sample adaptive offset on high bit-depth,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0091, Oct. 2013

- A. Tourapis, “SAO extension for higher bit-depth coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-N0201, Jul. 2013.

- S.-T. Hsiang, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), “AHG5: Modified SAO for range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-N0246, Jul. 2013

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Specification changes

Un-conditional removal of SAO quantization

#### Sample adaptive offset semantics

…

The list SaoOffsetVal[ cIdx ][ rx ][ ry ][ i ] for i ranging from 0 to 4, inclusive, is derived as follows:

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0
for( i = 0; i < 4; i++ )
SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] =		(7-1)
		offsetSign * sao_offset_abs[ cIdx ][ rx ][ ry ][ i ]  <<  ( bitDepth − Min( bitDepth, 10 ) )

Sequence level control

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sample_adaptive_offset_enabled_flag | u(1) |
| … |  |
| sps_extension_present_flag | u(1) |
| if( sps_extension_present_flag ) { |  |
| for( i = 0; i < 1; i++ ) |  |
| sps_extension_flag[ i ] | u(1) |
| sps_extension_7bits | u(7) |
| if( sps_extension_flag[ 0 ] ) { |  |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| implicit_rdpcm_enabled_flag | u(1) |
| explicit_rdpcm_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| high_precision_offsets_enabled_flag | u(1) |
| fast_rice_adaptation_enabled_flag | u(1) |
| if (sample_adaptive_offset_enabled_flag) |  |
| sao_quant _enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Sequence parameter set RBSP semantics

…

sao_quant _enabled_flag equal to 1 specifies that the quantization is applied for sao_offset_abs. sao_quant _enabled_flag equal to 0 specifies that quantization for sao_offset_abs is not used. When not present, the value of sao_quant _enabled_flag is inferred to be equal to 0.

…

#### Sample adaptive offset semantics

…

The variable bitDepth is derived as follows:

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

- Otherwise (cIdx is equal to 1 or 2), bitDepth is set equal to BitDepthC.

The variable SaoQBits is derived as follows

if (sao_quant _enabled_flag)

SaoQBits  = ( bitDepth − Min( bitDepth, 10 ) )

else

SaoQBits  = 0.

The list SaoOffsetVal[ cIdx ][ rx ][ ry ][ i ] for i ranging from 0 to 4, inclusive, is derived as follows:

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0
for( i = 0; i < 4; i++ )
SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] =		(7-2)
		offsetSign * sao_offset_abs[ cIdx ][ rx ][ ry ][ i ]  <<  SaoQBits  ( bitDepth − Min( bitDepth, 10 ) )

…

Slice-level control

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ….. |  |
| if( sps_extension_flag[ 0 ] ) { |  |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| implicit_rdpcm_enabled_flag | u(1) |
| explicit_rdpcm_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| high_precision_offsets_enabled_flag | u(1) |
| fast_rice_adaptation_enabled_flag | u(1) |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| adaptive_sao_bit_shift_active_flag | u(1) |
| } |  |
| } |  |
| ……. |  |
| } |  |

#### 7.3.6.1 General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ……. |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType > 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_sao_luma_flag && adaptive_sao_bit_shift_active_flag ) |  |
| sao_luma_bit_shift | ue(v) |
| if( slice_sao_chroma_flag && adaptive_sao_bit_shift_active_flag ) |  |
| sao_chroma_bit_shift | ue(v) |
| ……. |  |
| } |  |

#### 7.4.3.2 Sequence parameter set RBSP semantics

adaptive_sao_bit_shift_active_flag: equal to 1 specifies that sao_luma_bit_shift and sao_chroma_bit_shift may be present at the slice segment header. adaptive_sao_bit_shift_active_flag equal to 0 specifies that sao_luma_bit_shift and sao_chroma_bit_shift are not present at the slice segment header. When not present, the value of adaptive_sao_bit_shift_active_flag is inferred to be equal to 0.

7.4.7.1 General slice segment header semantics

sao_luma_bit_shift specifies the parameter that is used to derive the SAO offset values for the luma samples. The value of sao_luma_bit_shift shall be in the range of 0 to BitDepthY -6, inclusive. When not present, the value of sao_luma_bit_shift is inferred to be equl to bitDepthY − Min( bitDepth Y , 10 )

sao_chroma_bit_shift specifies the parameter that is used to derive the SAO offset values for the chroma samples. The value of sao_chroma_bit_shift shall be in the range of 0 to BitDepthC -6, inclusive. When not present, the value of sao_chroma_bit_shift is inferred to be equl to bitDepthC  − Min( bitDepth C, 10 )

7.4.9.3 Sample adaptive offset semantics

The variable saoOffsetBitShift is derived as follows:

- If cIdx is equal to 0, saoOffsetBitShift is set equal to sao_luma_bit_shift.

- Otherwise (cIdx is equal to 1 or 2), saoOffsetBitShift is set equal to sao_chmora_bit_shift.

The list SaoOffsetVal[ cIdx ][ rx ][ ry ][ i ] for i ranging from 0 to 4, inclusive, is derived as follows:

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0
for( i = 0; i < 4; i++ )
SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] =		(7-3)
		offsetSign * sao_offset_abs[ cIdx ][ rx ][ ry ][ i ]  <<  ( bitDepth − Min( bitDepth, 10 ) )

offsetSign * sao_offset_abs[ cIdx ][ rx ][ ry ][ i ]  <<  saoOffsetBitShift

…

PPS control

Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pps_extension1_flag ) { |  |
| if( transform_skip_enabled_flag ) |  |
| log2_max_transform_skip_block_size_minus2 | ue(v) |
| luma_chroma_prediction_enabled_flag | u(1) |
| chroma_qp_adjustment_enabled_flag | u(1) |
| if( chroma_qp_adjustment_enabled_flag ) { |  |
| diff_cu_chroma_qp_adjustment_depth | ue(v) |
| chroma_qp_adjustment_table_size_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_adjustment_table_size_minus1; i++ ) { |  |
| cb_qp_adjustment[ i ] | se(v) |
| cr_qp_adjustment[ i ] | se(v) |
| } |  |
| } |  |
| sao_luma_bit_shift | ue(v) |
| sao_chroma_bit_shift | ue(v) |
| pps_extension2_flag | u(1) |
| } |  |
| if( pps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Picture parameter set RBSP semantics

…

sao_luma_bit_shift specifies the parameter that is used to derive the SAO offset values for the luma samples. The value of sao_luma_bit_shift shall be in the range of 0 to BitDepthY -6, inclusive. When not present, the value of sao_luma_bit_shift is inferred to be equal to 0

sao_chroma_bit_shift specifies the parameter that is used to derive the SAO offset values for the chroma samples. The value of sao_chroma_bit_shift shall be in the range of 0 to BitDepthC -6, inclusive. When not present, the value of sao_chroma_bit_shift is inferred to be equal to 0

Sample adaptive offset semantics

The variable saoOffsetBitShift is derived as follows:

- If cIdx is equal to 0, saoOffsetBitShift is set equal to sao_luma_bit_shift.

- Otherwise (cIdx is equal to 1 or 2), saoOffsetBitShift is set equal to sao_chmora_bit_shift.

The list SaoOffsetVal[ cIdx ][ rx ][ ry ][ i ] for i ranging from 0 to 4, inclusive, is derived as follows:

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0
for( i = 0; i < 4; i++ )
SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] =		(7-4)
		offsetSign * sao_offset_abs[ cIdx ][ rx ][ ry ][ i ]  <<  ( bitDepth − Min( bitDepth, 10 ) )

offsetSign * sao_offset_abs[ cIdx ][ rx ][ ry ][ i ]  <<  saoOffsetBitShift

# Appendix

Table 3.  SAO off test results (AhG 8 lossy test configuration)

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 1,3% | 3,4% | 4,6% | 1,2% | 2,4% | 3,3% | 0,8% | 1,4% | 1,9% |
| Class B | 0,6% | 0,1% | 0,4% | 0,4% | 0,2% | 0,4% | 0,1% | 0,1% | 0,3% |
| RGB 4:4:4 SC | 2,8% | 1,6% | 2,0% | 2,7% | 1,6% | 1,9% | 2,3% | 1,3% | 1,4% |
| RGB 4:4:4 Animation | 0,7% | 1,4% | 1,4% | 0,7% | 1,4% | 1,3% | 0,4% | 0,9% | 0,8% |
| YCbCr 4:4:4 SC | 3,4% | 2,7% | 3,2% | 2,9% | 2,3% | 2,7% | 2,2% | 1,7% | 2,0% |
| YCbCr 4:4:4 Animation | 0,6% | 2,0% | 1,8% | 0,6% | 1,9% | 1,8% | 0,4% | 1,7% | 1,5% |
| RangeExt | 0,4% | 0,3% | 0,7% | 0,5% | 0,5% | 0,7% | 0,4% | 0,4% | 0,7% |
| RGB 4:4:4 SC (Optional) | 3,4% | 2,3% | 2,3% | 3,4% | 2,8% | 2,9% | 2,9% | 2,5% | 2,6% |
| YCbCr 4:4:4 SC (Optional) | 3,6% | 3,8% | 2,5% | 3,6% | 3,7% | 2,5% | 3,2% | 3,4% | 2,1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 93% |  |  | 93% |  |  | 94% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 2,5% | 4,7% | 6,7% | 2,5% | 3,6% | 4,8% |  |  |  |
| Class B | 0,7% | 0,8% | 0,7% | 1,2% | 1,1% | 0,4% |  |  |  |
| RGB 4:4:4 SC | 5,1% | 3,2% | 3,7% | 4,8% | 3,3% | 3,7% |  |  |  |
| RGB 4:4:4 Animation | 0,7% | 1,0% | 0,9% | 0,8% | 0,9% | 0,8% |  |  |  |
| YCbCr 4:4:4 SC | 6,4% | 4,4% | 5,1% | 5,7% | 3,8% | 4,5% |  |  |  |
| YCbCr 4:4:4 Animation | 0,6% | 2,1% | 1,9% | 0,7% | 2,0% | 1,9% |  |  |  |
| RangeExt | 0,8% | 3,8% | 1,9% | 1,4% | 7,3% | 3,3% |  |  |  |
| RGB 4:4:4 SC (Optional) | 4,1% | 3,1% | 3,2% | 4,3% | 3,7% | 4,0% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | 3,9% | 5,9% | 4,2% | 4,3% | 5,2% | 3,8% |  |  |  |
| Enc Time[%] | 102% |  |  | 103% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 5,3% | 8,5% | 11,8% | 4,9% | 7,0% | 9,1% |  |  |  |
| Class B | 1,1% | 1,8% | 2,4% | 1,5% | 3,1% | 2,7% |  |  |  |
| RGB 4:4:4 SC | 7,0% | 5,5% | 6,0% | 6,3% | 5,1% | 5,5% |  |  |  |
| RGB 4:4:4 Animation | 1,5% | 2,3% | 1,9% | 1,9% | 2,4% | 2,0% |  |  |  |
| YCbCr 4:4:4 SC | 10,4% | 6,4% | 7,0% | 8,9% | 5,5% | 6,2% |  |  |  |
| YCbCr 4:4:4 Animation | 1,0% | 5,4% | 5,0% | 1,1% | 5,6% | 5,3% |  |  |  |
| RangeExt | 0,9% | 3,4% | 2,5% | 1,3% | 5,0% | 4,0% |  |  |  |
| RGB 4:4:4 SC (Optional) | 7,6% | 7,3% | 8,4% | 9,0% | 8,2% | 8,5% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | 5,6% | 7,7% | 5,4% | 6,0% | 8,2% | 6,1% |  |  |  |
| Enc Time[%] | 103% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |  |  |  |
| Screen content 8 bit | ()4,9% |  |  | 4,4% |  |  |  |  | 4,7% |
| Camera captured 8 bit | 0,8% |  |  | 1,0% |  |  |  |  | 0,9% |
| RangeExt 10 bit | 0,7% |  |  | 1,1% |  |  |  |  | 0,9% |

Table 4.  SAO off test results (AhG 18 test configuration)

16 bits

|  |  |  | Average BD-rate Change (%) - All Tiers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra 16-bit (internal bit depth) |  |  | Lowdelay 16-bit (internal bit depth) |  |  |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| RExt RGB |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| HDR |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| 4K from HDR |  |  | 0,0% | 0,0% | 0,0% |  |  |  |
| HDR 4:0:0 Medical |  |  | 0,0% |  |  | 0,0% |  |  |
| 4:0:0 Medical |  |  | 0,0% |  |  | 0,0% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| All Classes |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |

12 bits

|  |  |  | Average BD-rate Change (%) - All Tiers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra 12-bit (internal bit depth) |  |  | Lowdelay 12-bit (internal bit depth) |  |  |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| RExt RGB |  |  | 0,0% | 0,1% | 0,0% | 0,0% | 0,1% | 0,0% |
| HDR |  |  | 0,0% | 0,0% | 0,0% | 0,1% | 0,0% | 0,0% |
| 4K from HDR |  |  | 0,0% | 0,0% | 0,0% |  |  |  |
| HDR 4:0:0 Medical |  |  | 0,0% |  |  | 0,1% |  |  |
| 4:0:0 Medical |  |  | 0,0% |  |  | 0,0% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | 0,0% | 0,0% | 0,1% | 0,2% | 0,1% | 0,1% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0,0% | 0,0% | 0,1% | 0,4% | 0,1% | 0,1% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | -0,1% |
| All Classes |  |  | 0,0% | 0,0% | 0,0% | 0,1% | 0,0% | 0,0% |

Table 5.  SAO w/o saoOffsetAbs quantization (AhG 18 test configuration)

16 bits

|  |  |  | Average BD-rate Change (%) - All Tiers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra 16-bit (internal bit depth) |  |  | Lowdelay 16-bit (internal bit depth) |  |  |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| RExt RGB |  |  | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% | 0,0% |
| HDR |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| 4K from HDR |  |  | 0,0% | 0,0% | 0,0% |  |  |  |
| HDR 4:0:0 Medical |  |  | 0,0% |  |  | -0,1% |  |  |
| 4:0:0 Medical |  |  | 0,0% |  |  | 0,0% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0,0% | 0,0% | -0,1% | -0,1% | -0,1% | -0,2% |
| Synthetic RGB HDR (+6 MSBs) |  |  | -0,3% | -0,1% | -0,7% | -1,3% | -1,1% | -1,2% |
| All Classes |  |  | 0,0% | 0,0% | -0,1% | -0,2% | -0,2% | -0,2% |

12 bits

|  |  |  | Average BD-rate Change (%) - All Tiers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra 12-bit (internal bit depth) |  |  | Lowdelay 12-bit (internal bit depth) |  |  |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| RExt RGB |  |  | -0,1% | -0,1% | -0,1% | -0,2% | -0,1% | -0,1% |
| HDR |  |  | -0,2% | -0,3% | -0,3% | -0,7% | -0,8% | -0,7% |
| 4K from HDR |  |  | 0,0% | -0,1% | 0,0% |  |  |  |
| HDR 4:0:0 Medical |  |  | -0,5% |  |  | -2,3% |  |  |
| 4:0:0 Medical |  |  | -0,2% |  |  | -0,4% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | -0,3% | -0,1% | -0,6% | -1,2% | -1,0% | -1,1% |
| Synthetic RGB HDR (+4 MSBs) |  |  | -1,1% | -0,7% | -1,8% | -4,8% | -2,8% | -3,3% |
| Synthetic RGB HDR (+6 MSBs) |  |  | -2,6% | -1,8% | -4,4% | -6,7% | -3,4% | -4,6% |
| All Classes |  |  | -0,5% | -0,4% | -1,0% | -2,1% | -1,4% | -1,6% |

Table 6.  SAO w/o saoOffsetAbs quantization plus EO classification (4) (AhG 18 test configuration)

16 bits

|  |  |  | Average BD-rate Change (%) - All Tiers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra 16-bit (internal bit depth) |  |  | Lowdelay 16-bit (internal bit depth) |  |  |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| RExt RGB |  |  | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% | 0,0% |
| HDR |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| 4K from HDR |  |  | 0,0% | 0,0% | 0,0% |  |  |  |
| HDR 4:0:0 Medical |  |  | 0,0% |  |  | 0,0% |  |  |
| 4:0:0 Medical |  |  | 0,0% |  |  | 0,0% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| All Classes |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |

12 bits

|  |  |  | Average BD-rate Change (%) - All Tiers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra 12-bit (internal bit depth) |  |  | Lowdelay 12-bit (internal bit depth) |  |  |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| RExt RGB |  |  | -0,1% | 0,0% | -0,1% | -0,2% | -0,1% | -0,1% |
| HDR |  |  | -0,2% | -0,3% | -0,3% | -0,6% | -0,7% | -0,7% |
| 4K from HDR |  |  | 0,0% | -0,1% | 0,0% |  |  |  |
| HDR 4:0:0 Medical |  |  | -0,5% |  |  | -2,1% |  |  |
| 4:0:0 Medical |  |  | -0,2% |  |  | -0,6% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | -0,3% | -0,2% | -0,8% | -1,3% | -1,2% | -1,2% |
| Synthetic RGB HDR (+4 MSBs) |  |  | -0,8% | -0,7% | -1,4% | -2,6% | -0,8% | -1,4% |
| Synthetic RGB HDR (+6 MSBs) |  |  | -0,5% | -0,5% | -1,3% | -1,0% | -0,4% | -0,4% |
| All Classes |  |  | -0,3% | -0,2% | -0,6% | -1,0% | -0,5% | -0,6% |

Table 7.  SAO w/o saoOffsetAbs quantization plus EO classification (5) (AhG 18 test configuration)

16 bits

|  |  |  | Average BD-rate Change (%) - All Tiers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra 16-bit (internal bit depth) |  |  | Lowdelay 16-bit (internal bit depth) |  |  |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| RExt RGB |  |  | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% | 0,0% |
| HDR |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| 4K from HDR |  |  | 0,0% | 0,0% | 0,0% |  |  |  |
| HDR 4:0:0 Medical |  |  | 0,0% |  |  | 0,0% |  |  |
| 4:0:0 Medical |  |  | 0,0% |  |  | 0,0% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Synthetic RGB HDR (+4 MSBs) |  |  | 0,0% | 0,0% | -0,1% | -0,1% | 0,0% | -0,1% |
| Synthetic RGB HDR (+6 MSBs) |  |  | 0,0% | 0,0% | -0,2% | -0,2% | -0,1% | -0,1% |
| All Classes |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |

12 bits

|  |  |  | Average BD-rate Change (%) - All Tiers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Intra 12-bit (internal bit depth) |  |  | Lowdelay 12-bit (internal bit depth) |  |  |
|  |  |  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT |  |  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| RExt RGB |  |  | -0,1% | -0,1% | -0,1% | -0,2% | -0,1% | -0,1% |
| HDR |  |  | -0,1% | -0,3% | -0,3% | -0,7% | -0,7% | -0,7% |
| 4K from HDR |  |  | 0,0% | -0,1% | 0,0% |  |  |  |
| HDR 4:0:0 Medical |  |  | -0,3% |  |  | -1,9% |  |  |
| 4:0:0 Medical |  |  | -0,2% |  |  | -0,5% |  |  |
| Synthetic RGB HDR (+2 MSBs) |  |  | -0,3% | -0,1% | -0,7% | -1,2% | -1,1% | -1,1% |
| Synthetic RGB HDR (+4 MSBs) |  |  | -0,6% | -0,6% | -1,2% | -3,6% | -1,6% | -2,0% |
| Synthetic RGB HDR (+6 MSBs) |  |  | -0,6% | -0,7% | -1,6% | -3,1% | -1,0% | -1,2% |
| All Classes |  |  | -0,2% | -0,3% | -0,6% | -1,4% | -0,8% | -0,8% |