| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0227 |
| --- | --- |

| Title: | Crosscheck report of JCTVC-P0124 on color gamut scalability using modified weighted prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kiran Misra and Andrew Segall 5750 NW Pacific Rim Blvd Camas WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP Labs of America, Inc. |  |  |

_____________________________

# Abstract

This documents reports the crosscheck results for JCTVC-P0124 on color gamut scalability using modified weighted prediction. JCTVC-P0124 investigates: gain-offset and polynomial color space mapping, YUV region sizes and its utilization in weighted prediction. The source code provided by the proponents was studied and found to be consistent with the description in JCTVC-P0124. The rate-distortion performance for the four combinations considered in JCTVC-P0124 was measured and the BD rate reduction achieved was found to be identical to that reported by the proponents.

# Introduction

In [1] the following different YUV color space mappings are investigated:

Configuration 1: Different 8x8x8 regions of YUV color space are mapped using gain-offset model

Configuration 2: A single gain offset mapping model is used for the YUV color space

Configuration 3: Different 8x8x8 regions of YUV are mapped using polynomial model

Configuration 4: A single polynomial mapping is used for the YUV color space

# Experimental results:

The performance of four different configurations was measured versus SCE1 use case 1 anchors [2]. The detailed results can be found in the attached excel sheet.

Configuration 1: N=8, linear matrix mapping (YUV color space is divided into 8x8x8 regions)

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -8.6% | -12.3% | -16.8% | -8.5% | -12.3% | -16.7% |
| Overall (Test vs Ref) | -8.6% | -12.3% | -16.8% | -8.5% | -12.3% | -16.7% |
| Overall (Test vs single layer) | 8.1% | 7.4% | -1.4% | 10.6% | 9.1% | -0.2% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -16.7% | -19.9% | -24.4% | -16.7% | -20.0% | -24.4% |
| Overall (Test EL+BL vs single EL+BL) | -29.1% | -29.1% | -36.2% | -27.4% | -28.0% | -35.6% |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -6.4% | -10.0% | -14.3% | -6.3% | -10.0% | -14.1% |
| Overall (Test vs Ref) | -6.4% | -10.0% | -14.3% | -6.3% | -10.0% | -14.1% |
| Overall (Test vs single layer) | 19.2% | 20.5% | 8.5% | 20.9% | 21.9% | 9.8% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.8% |
| EL only (Test vs Ref) | -11.8% | -15.0% | -19.3% | -11.7% | -15.0% | -19.0% |
| Overall (Test EL+BL vs single EL+BL) | -20.4% | -17.5% | -27.1% | -19.4% | -16.9% | -26.5% |
| BL Match | Matched |  |  | Matched |  |  |

Configuration 2: N=1, linear matrix mapping (Region based WP is disabled and only difference to HEVC WP is linear matrix based equations)

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -3.0% | -3.0% | -4.4% | -3.2% | -3.2% | -4.4% |
| Overall (Test vs Ref) | -3.0% | -3.0% | -4.4% | -3.2% | -3.2% | -4.4% |
| Overall (Test vs single layer) | 15.0% | 18.9% | 13.0% | 17.2% | 20.6% | 14.3% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -6.1% | -6.0% | -7.5% | -6.5% | -6.3% | -7.5% |
| Overall (Test EL+BL vs single EL+BL) | -25.1% | -22.5% | -26.7% | -23.7% | -21.5% | -26.0% |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -1.9% | -1.6% | -3.7% | -2.0% | -1.7% | -3.5% |
| Overall (Test vs Ref) | -1.9% | -1.6% | -3.7% | -2.0% | -1.7% | -3.5% |
| Overall (Test vs single layer) | 25.0% | 31.5% | 22.0% | 26.5% | 32.9% | 23.2% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.8% |
| EL only (Test vs Ref) | -3.6% | -3.1% | -5.3% | -3.8% | -3.3% | -5.2% |
| Overall (Test EL+BL vs single EL+BL) | -17.1% | -11.5% | -18.1% | -16.3% | -10.9% | -17.6% |
| BL Match | Matched |  |  | Matched |  |  |

Configuration 3: N=8, no matrix based WP (Matrix based WP is disabled and only difference to HEVC WP is signaling different WP parameters for different regions)

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -5.2% | -4.3% | -9.9% | -5.2% | -4.4% | -9.9% |
| Overall (Test vs Ref) | -5.2% | -4.3% | -9.9% | -5.2% | -4.4% | -9.9% |
| Overall (Test vs single layer) | 12.3% | 17.5% | 6.6% | 14.9% | 19.3% | 7.9% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -10.8% | -9.5% | -15.1% | -10.7% | -9.6% | -15.0% |
| Overall (Test EL+BL vs single EL+BL) | -26.4% | -22.8% | -31.0% | -24.6% | -21.7% | -30.2% |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -4.3% | -4.4% | -9.6% | -4.2% | -4.6% | -9.3% |
| Overall (Test vs Ref) | -4.3% | -4.4% | -9.6% | -4.2% | -4.6% | -9.3% |
| Overall (Test vs single layer) | 22.0% | 28.1% | 14.5% | 23.7% | 29.3% | 15.8% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.8% |
| EL only (Test vs Ref) | -8.2% | -7.8% | -13.1% | -8.1% | -8.1% | -12.8% |
| Overall (Test EL+BL vs single EL+BL) | -18.7% | -13.1% | -23.3% | -17.7% | -12.7% | -22.7% |
| BL Match | Matched |  |  | Matched |  |  |

Configuration 4: N=1, polynomial matrix mapping enabled (Region based WP is disabled and only difference to HEVC WP is polynomial matrix based equations)

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -6.8% | -7.8% | -8.1% | -6.7% | -7.8% | -8.0% |
| Overall (Test vs Ref) | -6.8% | -7.8% | -8.1% | -6.7% | -7.8% | -8.0% |
| Overall (Test vs single layer) | 10.4% | 12.9% | 8.7% | 12.9% | 14.8% | 10.0% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -13.6% | -14.3% | -14.6% | -13.5% | -14.3% | -14.5% |
| Overall (Test EL+BL vs single EL+BL) | -28.1% | -26.1% | -30.5% | -26.4% | -25.0% | -29.8% |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -4.6% | -4.7% | -6.0% | -4.5% | -4.6% | -5.6% |
| Overall (Test vs Ref) | -4.6% | -4.7% | -6.0% | -4.5% | -4.6% | -5.6% |
| Overall (Test vs single layer) | 21.5% | 27.5% | 19.1% | 23.3% | 29.0% | 20.5% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.8% |
| EL only (Test vs Ref) | -8.8% | -8.5% | -9.8% | -8.7% | -8.5% | -9.4% |
| Overall (Test EL+BL vs single EL+BL) | -19.2% | -13.8% | -21.2% | -18.2% | -13.0% | -20.6% |
| BL Match | Matched |  |  | Matched |  |  |

# Conclusion

It is confirmed that all the rate-distortion data generated for the cross-check match the results provided by the proponents in JCTVC-P0124. It is also confirmed that source code matches the description in JCTVC-P0124.

# References

[1] A. Aminlou, K. Ugur, M. M. Hannuksela, “Non-SCE1: Color gamut scalability using modified weighted prediction,” JCTVC-P0124, 16th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, San José, US, 9–17 Jan. 2014

[2] P. Bordes, Y. Ye, E. Alshina, X. Li, S.-H. Kim, A. Duenas, K. Ugur, K. Sato, “Description of HEVC Scalable Extensions Core Experiment SCE1: Color Gamut and Bit-Depth Scalability”, JCTVC-O1101, 15th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, 23 Oct. – 1 Nov. 2013.