| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0231 |
| --- | --- |

| Title: | Non-RCE4: Refinement of the Palette in RCE4 Test 2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu Feng Zou Joel Sole Marta Karczewicz Rajan Joshi 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-651-7749 wpu@qti.qualcomm.com fzou@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

RCE4 Test 2 evaluated a palette coding tool. This proposal refines the design with the feedback given in the RCE process. The refinement reportedly improves the performance and increases the throughput by reducing the number of context-coded bins.

# Introduction

A palette-based coding method [1] was presented in 15-th JCT-VC meeting and was included as test 2 in RCE4 for further discussion in San Jose meeting [2, 3].

The palette design is divided in two stages. In the first one, for a CU, the palette is derived and signaled. In the second stage, pixels in the CU are coded using one of the three candidate modes:

- “Run mode”

- “Copy above mode”

- “Pixel mode” (escape mode)

# Method Description

## Skip the Index Coding when Palette has One Entry

When the palette has only one item, i.e. there is only one color in the CU, there is no necessity to code further syntax elements during the second palette coding stage. The information can be implicitly derived by the decoder. Specifically, when the decoder sees a palette coded CU with only one entry in the palette, all the pixels in the CU have the same value as specified in the palette.

## Skip the Coding of Escape Flag

In [3], during the second palette encoding stage, the logic can be expressed using the following pseudo code:

for (each pixel x in the CU)

{

Code a flag indicating whether x is escape;

if (x is escape)

{

code x with ‘pixel mode’;

}

else

{

code x with ‘run mode’ or ‘copy above mode’;

}

}

However, if the palette size is less than the predefined maximum palette size (default=32 in RCE4 Test 2), then no escape pixel can present, since if an escape pixel still exists, it should have already been added into the palette.

Based on this fact, if the palette size is less than 32, coding the escape indication flag is avoided.

## Escape Pixel Coding in ‘Pixel Mode’

### Increase CABAC Throughput

In [3], to code ‘pixel mode’, fixed length coding is used for each color component with one CABAC context for each bit-plane. To increase the entropy coding throughput, only the MSB is context-coded using a single context while the other bits are coded using bypass CABAC coding. This refinement also reduces the number of contexts for the palette method.

### Removal of the Escape Pixel Prediction

To reduce the coding complexity and to make the syntax cleaner, escape pixel prediction is disabled in ‘pixel mode’. The quantized escape pixel value is directly coded using the fixed length coding described above.

### Reducing the Fixed Length Coding Bit-Depth

In [3], to code an escape pixel, each component is fixed length coded using input bit-depth bits. For lossy coding, due to quantization and depending on the quantization step size, the necessary bit-depth to represent a quantized pixel value might be less than the input bit-depth. This fact is used to reduce the number of signalled bits.

# Experimental Results

The proposed method is implemented on HM-12.1+RExt-5.1. RCE4 common test conditions specified in [2] are used.

## Compared with RExt-5.1 Anchor

Table 1 Summary of AHG8 Lossy Coding Results (Proposed v.s. RExt-5.1 Anchor)

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.5% | -3.4% | -2.7% | -1.7% | -2.8% | -2.4% | -1.9% | -2.2% | -2.0% |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -13.4% | -12.4% | -12.8% | -16.0% | -15.4% | -15.4% | -18.7% | -19.2% | -19.1% |
| RGB 4:4:4 Animation | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| YCbCr 4:4:4 SC | -9.3% | -12.4% | -13.8% | -13.7% | -15.8% | -16.5% | -17.1% | -19.5% | -20.5% |
| YCbCr 4:4:4 Animation | 0.1% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -30.0% | -28.9% | -30.1% | -34.0% | -33.6% | -34.5% | -40.6% | -40.9% | -41.9% |
| YCbCr 4:4:4 SC (Optional) | -23.4% | -29.3% | -28.1% | -30.2% | -33.8% | -34.0% | -37.2% | -42.4% | -43.7% |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.8% | -3.6% | -2.1% | -0.9% | -2.4% | -1.6% |  |  |  |
| Class B | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |  |  |  |
| RGB 4:4:4 SC | -11.7% | -10.8% | -11.1% | -12.9% | -12.4% | -12.6% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -8.7% | -11.6% | -13.1% | -11.9% | -14.1% | -14.9% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -22.1% | -20.9% | -22.1% | -24.9% | -24.1% | -24.9% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -19.3% | -27.1% | -27.2% | -23.5% | -29.0% | -30.5% |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -3.3% | -0.9% | -0.2% | -1.8% | -0.6% |  |  |  |
| Class B | 0.1% | -0.2% | -0.2% | 0.1% | -0.1% | -0.1% |  |  |  |
| RGB 4:4:4 SC | -4.8% | -4.6% | -4.8% | -5.9% | -5.8% | -5.9% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -3.6% | -4.5% | -5.2% | -4.9% | -5.6% | -6.2% |  |  |  |
| YCbCr 4:4:4 Animation | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| RangeExt | 0.2% | -0.1% | 0.0% | 0.1% | -0.1% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -10.9% | -10.5% | -11.1% | -15.0% | -14.6% | -15.7% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -10.7% | -15.1% | -15.4% | -14.3% | -18.1% | -18.5% |  |  |  |

## Compared with RCE4 Test 2 Anchor

Table 1 Summary of AHG8 Lossy Coding Results (Proposed v.s. RCE4 Test 2)

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.9% | -1.8% | -1.7% | -0.6% | -1.0% | -1.1% | -0.5% | -0.5% | -0.5% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -3.6% | -2.6% | -2.7% | -3.2% | -2.7% | -2.6% | -2.7% | -2.3% | -2.4% |
| RGB 4:4:4 Animation | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| YCbCr 4:4:4 SC | -2.4% | -3.5% | -4.0% | -2.7% | -3.6% | -4.1% | -2.6% | -3.3% | -3.7% |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -3.5% | -3.5% | -3.7% | -4.1% | -4.2% | -4.2% | -4.4% | -4.4% | -4.5% |
| YCbCr 4:4:4 SC (Optional) | -1.6% | -1.0% | -0.8% | -1.2% | -0.7% | -1.0% | -1.1% | -0.9% | -1.2% |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.5% | -1.2% | -1.1% | -0.4% | -0.6% | -0.6% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -2.5% | -1.7% | -1.7% | -1.5% | -1.1% | -1.1% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -1.9% | -2.6% | -3.1% | -1.8% | -2.4% | -2.9% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.5% | -0.4% | -0.8% | -0.6% | -0.6% | -0.6% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | 0.1% | 2.1% | 2.2% | 0.1% | 1.6% | 1.0% |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -1.0% | -0.3% | 0.0% | -0.3% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -0.6% | -0.5% | -0.6% | -0.5% | -0.5% | -0.5% |  |  |  |
| RGB 4:4:4 Animation | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.7% | -0.7% | -1.1% | -0.6% | -0.6% | -1.0% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | 0.0% | -0.1% | -0.2% | -0.4% | -0.4% | -0.3% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -0.6% | 0.1% | 0.0% | -0.4% | 0.2% | -0.2% |  |  |  |

# References

- L. Guo, M. Karczewicz, J. Sole, and R. Joshi, “Evaluation of palette mode coding on HM-12.0+RExt-4.1,” JCTVC-O0218, Geneva, Switzerland, Oct. 2013.

- L. Guo, X. Guo, and A. Saxena, “HEVC Range Extensions Core Experiment 4 (RCE 4): Palette Coding For Screen Content,” JCTVC-O1124, Geneva, Switzerland, Oct. 2013.

- L. Guo, W. Pu, M. Karczewicz, J. Sole, R. Joshi, and F. Zou, “RCE4: Results of test 2 on palette mode for screen content coding,” JCTVC-P0198, San José, US, Jan. 2014.

# Patent rights declaration(s)

Qualcomm Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).