| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0232 |
| --- | --- |

| Title: | Cross-check report of 'RCE3: Subtest C.3 - Intra block copy with CTU-based padding' (JCTVC-P0147) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides a cross-check of RCE3 test C3 by Qualcomm.  In test C3, samples in the current CU are initialised using samples from the left neighbouring CTU or a DC predictor.  The cross check result confirms the simulation result reported by Qualcomm.

# Cross check procedure

Range extensions core experiment 3 (RCE3) [1] studies further refinements of the intra block copy mode.  This is a cross-check of RCE3 test C3, as described in proposal JCTVC-P0147 [2].

The source code for JCTVC-P0147 was provided by Qualcomm. Canon compiled the source code using GCC 4.7.1 and the simulation was performed on a Linux cluster under the AHG8 conditions, as defined in [3]. Note that timing results are not comparable due to differences in the simulation environment.

# Analysis

The source code was studied and the following observations were made:

- PCTUs along the left edge of the frame are initialised using a ‘half-tone’.

- CTUs not along the left edge of the frame are initialised;

- When populating the predictor, if any overlap happens between the reference block and the CU for CUs in the left column of CTUs, a DC predictor is used for the overlapped region.

- Note: the ‘left column’ in the above points includes cases on tile and slice boundaries.

These observations are consistent with the description in [2].

# Coding performance measurement

Table 1 and Table 2 provides the cross-check results for JCTVC-P0147 under Screen content coding conditions (i.e. AHG8 conditions), as defined in [3].

Table 1. JCTVC-P0147 AHG8 lossy simulation result.

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.3% | -0.3% | -0.4% | -0.2% | -0.2% | -0.3% | -0.2% | -0.2% | -0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.4% | -0.4% | -0.4% | -0.3% | -0.4% | -0.4% | -0.3% | -0.4% | -0.3% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.4% | -0.5% | -0.5% | -0.4% | -0.4% | -0.4% | -0.4% | -0.4% | -0.4% |
| YCbCr 4:4:4 Animation | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -0.8% | -0.7% | -0.8% | -0.9% | -0.9% | -0.9% | -0.9% | -0.8% | -0.9% |
| YCbCr 4:4:4 SC (Optional) | -1.3% | -1.1% | -1.2% | -1.4% | -1.2% | -1.4% | -1.4% | -1.3% | -1.4% |
| Enc Time[%] | 96% |  |  | 97% |  |  | 98% |  |  |
| Dec Time[%] | 91% |  |  | 93% |  |  | 95% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.4% | -0.3% | -0.2% | -0.3% | -0.4% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -0.2% | -0.1% | -0.2% | -0.2% | -0.2% | -0.2% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.3% | -0.3% | -0.4% | -0.4% | -0.4% | -0.5% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | #VALUE! | #VALUE! | #VALUE! |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.8% | -0.7% | -0.8% | -0.8% | -0.8% | -0.9% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -1.2% | -1.1% | -1.3% | -1.3% | -1.2% | -1.4% |  |  |  |
| Enc Time[%] | 99% |  |  | #VALUE! |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | #VALUE! |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.7% | -0.3% | -0.1% | -0.3% | -0.1% |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC | -0.4% | -0.4% | -0.4% | -0.5% | -0.4% | -0.4% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.3% | -0.2% | -0.3% | -0.2% | -0.2% | -0.3% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.8% | -0.8% | -0.6% | -0.1% | 0.1% | 0.2% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -1.1% | -1.0% | -1.0% | -1.5% | -1.7% | -1.6% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |

Table 2. JCTVC-P0147 AHG8 lossless simulation result.

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | -0.4% | -0.3% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.2% | -0.2% | -0.1% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.2% | 0.0% | -0.2% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -0.7% | -0.7% | -0.7% |
| YCbCr 4:4:4 SC (Optional) | -1.1% | -1.4% | -1.3% |
| Enc Time[%] | 102% | 101% | 101% |
| Dec Time[%] | 101% | 101% | 100% |

# Conclusion

This cross-check includes results for RCE3 test C3 provided by Qualcomm. The provided software matches the description in the contribution. The cross-check result confirms the simulation result reported by the proponent.

# References

- J. Sole, E. Alshina, D.-K. Kwon, W.-H. Peng, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra block copy refinement,” JCTVC-O1123, 15th JCT-VC meeting, Geneva, CH, Oct. 2013.

- C. Pang, J. Sole, L. Guo, M. Karczewicz, “RCE3: Subtest C.3 - Intra block copy with CTU-based padding,” JCTVC-P0147, 16th JCT-VC Meeting: San Jose, US, Jan 2014.

- D. Flynn, C. Rosewarne, K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-O1006, 15th JCT-VC Meeting: Geneva, CH, Oct. 2013.