| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0233 |
| --- | --- |

| Title: | Cross-check report of 'RCE3: intra block copy search range (tests A)’ test A5 (JCTVC-P0211) by Qualcomm and Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides a cross-check of RCE3 test A5 by Samsung.  In test A5, only the rightmost four columns of the left CTU and the lowermost four rows of the above CTU are used as reference for intra block coy.  The cross check result confirms the simulation result reported by Samsung.

# Cross check procedure

Range extensions core experiment 3 (RCE3) [1] studies further refinements of the intra block copy mode.  This is a cross-check of RCE3 test A5, as described in proposal JCTVC-P0211 [2].

The source code for RCE3 test A5 was provided by Samsung. Canon compiled the source code using GCC 4.7.1 and the simulation was performed on a Linux cluster under the AHG8 conditions, as defined in [3]. Note that timing results are not comparable due to differences in the simulation environment.

# Analysis

The source code was studied and the following observations were made:

- The search range left of the current CTU is set to 4 samples.

- When the above CTU is available and of the same slice and tile as the current CTU, the search range above the current CTU is set to 4 samples, otherwise the search range is not extended above the current CTU.

These observations are consistent with the description in [2].

# Coding performance measurement

Table 1 and Table 2 provides the cross-check results for JCTVC-P0211 test A5 under Screen content coding conditions (i.e. AHG8 conditions), as defined in [3].

Table 1. JCTVC-P0211 test A5 AHG8 lossy simulation result.

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 4.5% | 4.4% | 4.4% | 3.8% | 3.9% | 3.8% | 3.3% | 3.3% | 3.3% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| RGB 4:4:4 SC | 12.4% | 12.2% | 12.2% | 11.6% | 11.5% | 11.5% | 10.9% | 10.8% | 10.9% |
| RGB 4:4:4 Animation | 0.3% | 0.3% | 0.3% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| YCbCr 4:4:4 SC | 13.0% | 12.6% | 12.7% | 12.3% | 12.0% | 12.2% | 11.5% | 11.2% | 11.3% |
| YCbCr 4:4:4 Animation | 0.3% | 0.4% | 0.4% | 0.2% | 0.3% | 0.3% | 0.1% | 0.2% | 0.2% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 17.9% | 17.8% | 17.9% | 17.8% | 17.8% | 18.1% | 17.9% | 17.6% | 17.9% |
| YCbCr 4:4:4 SC (Optional) | 24.3% | 24.1% | 24.1% | 24.5% | 24.2% | 24.3% | 24.4% | 24.2% | 24.4% |
| Enc Time[%] | 93% |  |  | 94% |  |  | 94% |  |  |
| Dec Time[%] | 95% |  |  | 97% |  |  | 99% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 3.2% | 3.2% | 3.3% | 2.8% | 2.8% | 2.8% |  |  |  |
| Class B | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% |  |  |  |
| RGB 4:4:4 SC | 9.3% | 9.3% | 9.3% | 8.7% | 8.7% | 8.8% |  |  |  |
| RGB 4:4:4 Animation | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |  |  |  |
| YCbCr 4:4:4 SC | 9.9% | 9.8% | 9.7% | 9.4% | 9.4% | 9.4% |  |  |  |
| YCbCr 4:4:4 Animation | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| RangeExt | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | 14.3% | 14.4% | 14.4% | 14.2% | 14.2% | 14.4% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | 20.1% | 20.0% | 20.3% | 20.5% | 20.4% | 20.6% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 1.9% | 1.6% | 2.0% | 1.8% | 1.6% | 1.8% |  |  |  |
| Class B | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| RGB 4:4:4 SC | 6.8% | 6.5% | 6.5% | 6.6% | 6.4% | 6.5% |  |  |  |
| RGB 4:4:4 Animation | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| YCbCr 4:4:4 SC | 6.8% | 6.7% | 6.5% | 6.9% | 6.8% | 6.6% |  |  |  |
| YCbCr 4:4:4 Animation | 0.1% | 0.0% | 0.2% | 0.0% | 0.1% | 0.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | 10.9% | 10.8% | 10.6% | 12.3% | 12.1% | 12.1% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | 15.5% | 15.3% | 15.3% | 17.5% | 17.2% | 17.2% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 2. JCTVC-P0211 test A5 AHG8 lossless simulation result.

|  | AI |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression ratio |  |  |  |  |  |  |  | Bit-rate increase |  |  |  |
|  | Total |  | Average |  | Min |  | Max |  | Total | Avg | Min | Max |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 4.58 | 4.53 | 5.59 | 5.50 | 2.27 | 2.27 | 11.20 | 11.10 | 1.1% | 1.6% | 0.1% | 5.1% |
| Class B | 2.25 | 2.25 | 2.26 | 2.26 | 2.08 | 2.08 | 2.44 | 2.44 | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 9.40 | 8.95 | 11.7 | 10.86 | 6.50 | 6.44 | 19.62 | 17.78 | 5.1% | 7.2% | 0.3% | 19.1% |
| RGB 4:4:4 Animation | 2.83 | 2.83 | 2.9 | 2.86 | 2.41 | 2.41 | 3.14 | 3.14 | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 11.17 | 10.54 | 13.5 | 12.50 | 7.96 | 7.85 | 21.63 | 19.57 | 6.0% | 7.9% | 0.7% | 20.5% |
| YCbCr 4:4:4 Animation | 3.05 | 3.05 | 3.2 | 3.22 | 2.62 | 2.62 | 3.98 | 3.98 | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 1.98 | 1.98 | 2.44 | 2.44 | 1.52 | 1.52 | 4.43 | 4.43 | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 30.91 | 27.50 | 34.30 | 29.08 | 17.27 | 17.02 | 48.88 | 39.33 | 12.4% | 14.9% | 1.4% | 24.3% |
| YCbCr 4:4:4 SC (Optional) | 35.53 | 30.11 | 38.63 | 31.16 | 24.30 | 23.66 | 58.96 | 42.84 | 18.0% | 20.4% | 2.7% | 37.6% |
| Enc Time[%] | 93% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 104% |  |  |  |  |  |  |  |  |  |  |  |
|  | RA |  |  |  |  |  |  |  |  |  |  |  |
|  | Compression ratio |  |  |  |  |  |  |  | Bit-rate increase |  |  |  |
|  | Total |  | Average |  | Min |  | Max |  | Total | Avg | Min | Max |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 8.64 | 8.63 | 35.85 | 35.11 | 3.01 | 3.01 | 88.86 | 86.08 | 0.1% | 0.9% | 0.0% | 3.2% |
| Class B | 2.60 | 2.60 | 2.60 | 2.60 | 2.57 | 2.57 | 2.64 | 2.64 | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 56.83 | 55.84 | 140.5 | 127.68 | 20.03 | 19.91 | 290.96 | 263.93 | 1.8% | 6.1% | 0.1% | 18.1% |
| RGB 4:4:4 Animation | 3.67 | 3.67 | 3.7 | 3.68 | 3.47 | 3.47 | 3.86 | 3.86 | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 71.01 | 69.57 | 161.3 | 146.50 | 24.73 | 24.59 | 321.23 | 290.78 | 2.1% | 6.5% | 0.3% | 19.2% |
| YCbCr 4:4:4 Animation | 3.87 | 3.87 | 4.5 | 4.49 | 2.79 | 2.79 | 5.70 | 5.70 | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 2.14 | 2.14 | 2.57 | 2.57 | 1.59 | 1.59 | 4.50 | 4.50 | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 70.95 | 69.15 | 329.65 | 278.94 | 19.93 | 19.81 | 498.73 | 426.30 | 2.6% | 12.7% | 0.6% | 20.4% |
| YCbCr 4:4:4 SC (Optional) | 109.02 | 104.05 | 356.47 | 285.72 | 32.52 | 32.18 | 601.48 | 458.62 | 4.8% | 17.0% | 1.1% | 31.2% |
| Enc Time[%] | 99% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 97% |  |  |  |  |  |  |  |  |  |  |  |
|  | LB |  |  |  |  |  |  |  |  |  |  |  |
|  | Compression ratio |  |  |  |  |  |  |  | Bit-rate increase |  |  |  |
|  | Total |  | Average |  | Min |  | Max |  | Total | Avg | Min | Max |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 8.84 | 8.84 | 56.21 | 55.63 | 3.00 | 3.00 | 163.08 | 160.90 | 0.0% | 0.4% | 0.0% | 1.4% |
| Class B | 2.61 | 2.61 | 2.61 | 2.61 | 2.57 | 2.57 | 2.64 | 2.64 | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 63.65 | 62.82 | 485.60 | 443.27 | 21.03 | 20.92 | 2111.85 | 1928.44 | 1.3% | 5.5% | 0.1% | 14.3% |
| RGB 4:4:4 Animation | 3.65 | 3.65 | 3.67 | 3.67 | 3.45 | 3.45 | 3.83 | 3.83 | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 80.58 | 79.50 | 544.80 | 497.08 | 25.92 | 25.80 | 2333.65 | 2128.17 | 1.4% | 5.6% | 0.2% | 15.3% |
| YCbCr 4:4:4 Animation | 3.88 | 3.88 | 4.53 | 4.53 | 2.78 | 2.78 | 5.75 | 5.75 | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 2.13 | 2.13 | 2.57 | 2.57 | 1.59 | 1.59 | 4.50 | 4.50 | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 77.21 | 76.32 | 1334.89 | 1102.69 | 20.09 | 19.98 | 3012.92 | 2430.29 | 1.2% | 12.6% | 0.6% | 24.0% |
| YCbCr 4:4:4 SC (Optional) | 125.19 | 122.46 | 1294.26 | 1033.05 | 33.08 | 32.76 | 2516.65 | 1986.40 | 2.2% | 17.0% | 1.0% | 26.7% |
| Enc Time[%] | 98% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 97% |  |  |  |  |  |  |  |  |  |  |  |

# Conclusion

This cross-check includes results for RCE3 test A5 provided by Samsung. The provided software matches the description in the contribution. The cross-check result confirms the simulation result reported by the proponent.

# References

- J. Sole, E. Alshina, D.-K. Kwon, W.-H. Peng, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra block copy refinement,” JCTVC-O1123, 15th JCT-VC meeting, Geneva, CH, Oct. 2013.

- E.Alshina, A.Alshin, C. Pang, J. Sole, M. Karczewicz “RCE3: intra block copy search range (tests A),” JCTVC-P0211, 16th JCT-VC Meeting: San Jose, US, Jan 2014.

- D. Flynn, C. Rosewarne, K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-O1006, 15th JCT-VC Meeting: Geneva, CH, Oct. 2013.