| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0235r2 |
| --- | --- |

| Title: | Non-SCE1: Trade-off between coding efficiency and buffer size with the 3D-LUT-based method for Color Gamut Scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

Under SCE1 2 methods are now being studied as predictors for color gamut scalability. One is weighted-prediction-based approach and the other is 3D-LUT-based approach. As of 3D LUT-based approach originally table size is defined as 17x17x17, and under SCE1 it is defined as 9x9x9.

In this contribution trade-off of coding efficiency and implementation cost of different 3D LUT size is studied. It is recommended that trade-off between coding efficiency and implementation cost of the 3D-LUT-based method as well as weighted-prediction-based method be studied under SCE or AHG.

# Introduction / Problem Statement / …

As predictors for color gamut scalability, 2 methods are now being studied: weighted-prediction-based approach and 3D-LUT-based approach. The table size originally proposed by JCTVC-O0159 is 17x17x17. It provides better coding efficiency than the weighted-prediction based method, but it requires buffer with the size around 150kbits.

This document reports trade-off of the 3D-LUT-based approach between coding efficiency and implementation cost (buffer size).

# Simulation Condition

SHM-4.0, including the bit-depth scalability feature and the option to set the weighted prediction restricted to inter-layer as adopted in JCTVC-O0194, is used as anchor.

Software for SCE1: Use Case 1: Test 1.A & Test 1.B is used for the test. That is, the following setting is applied with the configuration file:

SpsPpsPeriod : 0

With this software, sizes of 3D LUT are defined with the configuration file as follow

Test 1: 5x5x5

ColorGamutScalabilityMode : 3

Test 2: 9x9x9

ColorGamutScalabilityMode : 4

Test 3: 17x17x17

ColorGamutScalabilityMode : 5

{AI, RA}_{8, 10}-bit conditions are tested with the sequences described by [1].

# Simulation Result

Summary of the results with Test {1, 2, 3} is shown in Table 3.1, 3.2 and 3.3 respectively. Complete results are shown in JCTVC-P0235_test1.xls, JCTVC-P0235_test2.xls and JCTVC-P0235_test3.xls respectively.

Table 3.1

Table 3.2

Table 3.3

As stated in JCTVC-M0211, buffer size required for storing scaling list in HEVC is 8000 bits. In SCE proposals LUT size is 8bit, so with each of the tests required buffer size is:

Test 1: 5x5x5x3x8 	= 3000 [bits] 	= 0.375 [times compared w/ SL buffer size]

Test 2: 9x9x9x3x8 	= 17496 [bits] 	= 2.187 [times compared w/ SL buffer size]

Test 3: 17x17x17x3x8 = 117912 [bits]	= 14.739 [times compared w/ SL buffer size]

From Fig 3.1 to 3.4 trade-offs between gain and buffer size relative to the one for scaling list are shown.

- PSNRY

- PSNRU

- PSNRV

Fig 3.1 AI_10bit

- PSNRY

- PSNRU

- PSNRV

Fig 3.2 AI_8bit

- PSNRY

- PSNRU

- PSNRV

Fig 3.3 RA_10bit

- PSNRY

- PSNRU

- PSNRV

Fig 3.4 RA_8bit

# Discussions

This document reports trade-off of the 3D LUT-based approach between coding efficiency and implementation cost (buffer size).

It is recommended that trade-off between coding efficiency and implementation cost of the 3D-LUT-based method as well as weighted-prediction-based method be studied under SCE or AHG.

# References

[1] “Description of HEVC Scalable Extensions Core Experiment SCE1: Color Gamut and Bit-Depth Scalability”, JCTVC-O1011, November 2013

[2] “SCE4: Result on 5.3-test1 and 5.3-test2”, JCTVC-O0159, November 2013

[3] “SCE4: Test 5.1 results on bit-depth and color-gamut scalability”, JCTVC-O0194, November 2013

[4] “AHG9: On Scaling List Signalling for SHVC”, JCTVC-M0211, April 2013

[5]“SCE1: Results on Core Experiment on Color Gamut and Bit-Depth Scalability, tests 1A & 1B”, JCTVC-P0128, January 2014

[6] “SCE1: Crosscheck Result of Use Case 1: Test 1.A & Test 1.B”, JCTVC-P0234, January 2013

# Patent rights declaration(s)

Sony Corp. does not have any current or pending patent rights relating to the technology described in this contribution.