| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0236 |
| --- | --- |

| Title: | RCE2: Verification for Test 1 on Rice Parameter Initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E.Alshina | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Ltd.. |  |  |

_____________________________

# Abstract

This contribution verifies the results of RCE2 test 1 on rice parameter initialization.

# Introduction

In HEVC, the binarization of the syntax element coeff_abs_level_remaining depends on the Rice parameter. In the 15-th JCTVC meeting modificaiton of Rice parameter initialization which was only one line chnage (both in code and specicication) was adopted [1]. In verified contribution further improvement of rRice parameter initialization was tested. Test based on [2], which was already analysed in cross-check report [3]. Main concern was additional counters which need to be up-dated separately according to then block type (Luma or Chroma, Transform or Transform Skip is used).

# Experimental results

The proposed method was implemented on HM-12.1+RExt-5.1 and tested under AhG 8 conditions. In addition we report performance drop in adoption of previous meeting [1] is reverted.

Table 1. Comparison between gain of Rice parameter initialization modification adopted in JCTVC-15th (JCTVC-O0206) and content adaptive  Rice parameter initialization verified in this report (JCTVC-P0199).

|  | Loss-less SC | Loss-less non-SC | Lossy SC | Lossy non-SC |
| --- | --- | --- | --- | --- |
| JCTVC-O0206 | 1,4% | 1,2% | 0,1% | 0,0% |
| JCTVC-P0199 | 1,4% | 0,4% | 0,2% | 0,0% |

In Table 1 data reported separately for screen content (“SC”=”Class F& YCbCr 4:4:4 SC”) and camera captured content (“non-SC” =”Class B & RangeExt”). Both methods modify Rice parameter initialization, both efficient for mainly screen content. The one adopted at the previous meeting (JCTVC-O0206) is simpler since doesn’t require statistics collection, but provides even higher gain in loss-less non-SC test compare to content adaptive Rice parameter initialization verified here. More details about performance of verified tool can be found in Tables 2 and 3. Impressive gain ws demonstrated for optional screen content classes only.

| Table 2. Lossy AhG8 test for to content adaptive Rice parameter initialization. |  |  | All Intra Main-tier |  |  |  |  |  |  | All Intra High-tier |  |  |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y |  | U |  |  | V |  | Y |  | U |  | V | Y | U | V |
| Class F |  |  | -0,4% |  | -0,5% |  |  | -0,5% |  | -0,7% |  | -0,8% |  | -0,8% | -1,1% | -1,2% | -1,1% |
| Class B |  |  | 0,0% |  | 0,0% |  |  | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% | 0,0% | 0,0% | 0,0% |
| RGB 4:4:4 SC |  |  | -0,5% |  | -0,6% |  |  | -0,6% |  | -1,2% |  | -1,2% |  | -1,1% | -2,0% | -2,3% | -2,2% |
| RGB 4:4:4 Animation |  |  | 0,0% |  | 0,0% |  |  | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% | 0,0% | 0,0% | 0,0% |
| YCbCr 4:4:4 SC |  |  | -0,4% |  | -0,5% |  |  | -0,4% |  | -0,9% |  | -0,9% |  | -0,8% | -1,6% | -1,7% | -1,7% |
| YCbCr 4:4:4 Animation |  |  | 0,0% |  | 0,0% |  |  | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% | 0,0% | 0,0% | 0,0% |
| RangeExt |  |  | 0,0% |  | 0,0% |  |  | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% | 0,0% | 0,0% | 0,0% |
| RGB 4:4:4 SC (Optional) |  |  | -3,7% |  | -3,5% |  |  | -3,5% |  | -5,8% |  | -5,8% |  | -5,9% | -7,9% | -8,2% | -8,2% |
| YCbCr 4:4:4 SC (Optional) |  |  | -1,4% |  | -1,5% |  |  | -1,5% |  | -2,7% |  | -2,8% |  | -2,7% | -4,5% | -4,6% | -4,8% |
| Enc Time[%] |  |  | 100% |  |  |  |  |  |  | 100% |  |  |  |  | 100% |  |  |
| Dec Time[%] |  |  | 100% |  |  |  |  |  |  | 100% |  |  |  |  | 100% |  |  |
|  |  |  | Random Access Main-tier |  |  |  |  |  |  | Random Access High-tier |  |  |  |  |  |  |  |
|  |  |  | Y |  | U |  |  | V |  | Y |  | U |  | V |  |  |  |
| Class F |  |  | -0,3% |  | -0,3% |  |  | -0,4% |  | -0,5% |  | -0,5% |  | -0,6% |  |  |  |
| Class B |  |  | 0,0% |  | 0,0% |  |  | 0,2% |  | 0,0% |  | 0,0% |  | 0,1% |  |  |  |
| RGB 4:4:4 SC |  |  | -0,5% |  | -0,5% |  |  | -0,4% |  | -1,0% |  | -0,9% |  | -0,9% |  |  |  |
| RGB 4:4:4 Animation |  |  | 0,0% |  | 0,0% |  |  | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |  |  |  |
| YCbCr 4:4:4 SC |  |  | 0,2% |  | 0,2% |  |  | 0,1% |  | -0,3% |  | -0,3% |  | -0,3% |  |  |  |
| YCbCr 4:4:4 Animation |  |  | 0,0% |  | -0,1% |  |  | -0,1% |  | 0,0% |  | 0,0% |  | 0,1% |  |  |  |
| RangeExt |  |  | 0,0% |  | 0,0% |  |  | -0,1% |  | 0,0% |  | 0,0% |  | -0,1% |  |  |  |
| RGB 4:4:4 SC (Optional) |  |  | -3,9% |  | -3,7% |  |  | -3,7% |  | -6,1% |  | -6,0% |  | -6,2% |  |  |  |
| YCbCr 4:4:4 SC (Optional) |  |  | -1,2% |  | -1,2% |  |  | -1,2% |  | -2,4% |  | -2,3% |  | -2,4% |  |  |  |
| Enc Time[%] |  |  | 100% |  |  |  |  |  |  | 100% |  |  |  |  |  |  |  |
| Dec Time[%] |  |  | 99% |  |  |  |  |  |  | 99% |  |  |  |  |  |  |  |
|  |  |  | Low delay B Main-tier |  |  |  |  |  |  | Low delay B High-tier |  |  |  |  |  |  |  |
|  |  |  | Y |  | U |  |  | V |  | Y |  | U |  | V |  |  |  |
| Class F |  |  | 0,0% |  | -0,5% |  |  | 0,2% |  | -0,2% |  | -0,4% |  | -0,1% |  |  |  |
| Class B |  |  | 0,0% |  | 0,0% |  |  | 0,1% |  | 0,0% |  | 0,0% |  | 0,0% |  |  |  |
| RGB 4:4:4 SC |  |  | -0,5% |  | -0,5% |  |  | -0,5% |  | -0,9% |  | -0,8% |  | -0,8% |  |  |  |
| RGB 4:4:4 Animation |  |  | 0,0% |  | 0,0% |  |  | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |  |  |  |
| YCbCr 4:4:4 SC |  |  | -0,1% |  | -0,1% |  |  | -0,2% |  | -0,3% |  | -0,3% |  | -0,3% |  |  |  |
| YCbCr 4:4:4 Animation |  |  | 0,0% |  | 0,0% |  |  | -0,1% |  | 0,0% |  | 0,0% |  | 0,0% |  |  |  |
| RangeExt |  |  | 0,0% |  | 0,0% |  |  | 0,0% |  | 0,0% |  | 0,0% |  | -0,1% |  |  |  |
| RGB 4:4:4 SC (Optional) |  |  | -3,2% |  | -3,4% |  |  | -3,2% |  | -5,3% |  | -5,3% |  | -5,5% |  |  |  |
| YCbCr 4:4:4 SC (Optional) |  |  | -1,5% |  | -1,6% |  |  | -1,4% |  | -2,2% |  | -2,6% |  | -2,3% |  |  |  |
| Enc Time[%] |  |  | 100% |  |  |  |  |  |  | 100% |  |  |  |  |  |  |  |
| Dec Time[%] |  |  | 98% |  |  |  |  |  |  | 98% |  |  |  |  |  |  |  |

Table 3. Loss-less AhG8 test for to content adaptive Rice parameter initialization.

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | -0,7% | -0,5% | -0,3% |
| Class B | 0,0% | 0,0% | 0,0% |
| RGB 4:4:4 SC | -3,0% | -2,8% | -2,7% |
| RGB 4:4:4 Animation | -0,3% | -0,3% | -0,3% |
| YCbCr 4:4:4 SC | -2,6% | -2,0% | -2,4% |
| YCbCr 4:4:4 Animation | -0,2% | -0,1% | -0,2% |
| RangeExt | -1,3% | -0,4% | -0,4% |
| RGB 4:4:4 SC (Optional) | -8,7% | -8,7% | -9,2% |
| YCbCr 4:4:4 SC (Optional) | -6,9% | -6,9% | -7,1% |
| Enc Time[%] | 99% | 98% | 98% |
| Dec Time[%] | 102% | 96% | 97% |
| Number of differences from anchor | 34 | 34 | 34 |
| Total differences | 102 |  |  |
| Avg SC | -1,4% | nonSC | -0,4% |

# Conclusion

RCE2 test 1 which is content adaptive Rice parameter initialization is verified in this report.BD-rate gain 1,4% (screen content) and 0,4% (camera captured content) is confirmed in loss-less AhG8 test. In lossy AhG 8 test the gain is 0,2% for screen content and 0,0% for camera captured content.

# References

[1] , R. Joshi, M. Karczewicz (Qualcomm), “RCE2 Test A1: Simplified update of the coefficient level Rice parameter”, JCTVC-O0206, Geneva, Switzerland, Oct. 2013.

[2] M. Karczewicz, L. Guo, J. Sole, R. Joshi (Qualcomm), “RCE2: Results of Test C1 on Rice Parameter Initialization”, JCTVC-O0219, Geneva, Switzerland, Oct. 2013.

[3] E.Alshina, “RCE2: Verification of test C1 on Rice parameter initialization”, JCTVC-O0283, Geneva, Switzerland, Oct. 2013.