| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0237 |
| --- | --- |

| Title: | Non-RCE3: Major Color Padding Algorithm for Intra Block Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Chao-Hsiung Hung 2Erh-Chung Ke 3Hsin-Ta Chiao 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan 4Wen-Hsiao Peng 5Hsueh-Ming Hang 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | 1+886-3-5917964 1chhung@itri.org.tw 2+886-3-5914724 2JohnKe@itri.org.tw 3+886-3-5914757 3JosephChiao@itri.org.tw 4+886-3-5712121 # 56625 4wpeng@cs.nctu.edu.tw 5+886-3-5731861 5hmhang@mail.nctu.edu.tw |
| Source: | 1 2 3Industrial Technology Research Institute (ITRI) ## 4 5National Chiao Tung University (NCTU) |  |  |

_____________________________

# Abstract

Intra Block Copy (IBC) [1] was adopted in the 14th JCT-VC meeting. It provides good compression performance for screen content test sequences. In IBC, the current processing CU can not overlap with reference CU. Some padding algorithms [2][3] pad the overlapping region by neighboring available reconstructed pixels. We propose a padding algorithm based on major color in this document.

# Introduction

IBC [1] was adopted in HEVC Range Extensions draft specification in the 14th JCT-VC meeting. It predicts screen content well due to the repetitive pattern. However, current CU and reference CU are located on the same frame. Overlapping between these two blocks is inadmissible. Some padding [2][3] algorithms try to handle the overlapping to extend the search range of IBC. They pad the overlapping by the surrounding reconstructed pixels horizontally or vertically. In this document, we first analyze the surrounding reconstructed pixels of current CU. Then we pick a pixel as padding pixel. When performing IBC for this CU, we pad the overlapping region by this padding pixel only.

# Proposed Algorithm

When processing current CU, its left and top reconstructed pixels are already available. We calculate the histogram of reconstructed pixels located at the left one column, up one row, and left-up 1×1 square for simplicity. The total pixel number is N. These pixels can classified into M colors, P1, P2, …, PM. P1, P2, …, PM are the color indexes and the order is P1 < P2 < … < PM. Each color index Pi has its corresponding accumulated number Hi, 1 ≤ i ≤ N. The sum of these accumulated number Hi is N. Hmax = max(H1, H2, …, HM) and the corresponding color index is Pmax. The center color index of the histogram is PC. If M ≤ 4 or Hmax ≥ N/4, we choose Pmax as padding pixel. Otherwise, we choose PC instead. When the current CU and reference CU overlap, we pad the overlapping region by the selected pixel above.

# Experiment results

This proposed method is experimented based on HM-12.1+RExt-5.1 under conduction of the common test condition [4] and [5]. Table 1 and Table 2 show the lossless and lossy results.

Table 1. Lossless Result

Table 2. Lossy Results

# Conclusion

This document proposes a major color padding algorithm to extend the search range of IBC. For current processing CU, we calculate the histogram of around reconstructed pixels. Then we select a pixel by analyzing the histogram. We pad the overlapping region by this pixel when applying IBC on this current block. It saves about 0.6% ~ 3% on BD rate for many test sequences.

# References

- C. Pang, J. Sole, L. Guo, M. Karczewicz, and R. Joshi, “Non-RCE3: Intra Motion Compensation with 2-D MVs”, JCTVC-N0256, Vienna, AT, August 2013.

- S. Lee, E. Alshina, and C. Kim, “AHG5: Extension of intra block copy”, 15th JCT-VC meeting, JCTVC-O0112, Geneva, CH, Oct. 2013.

- C. Pang, J. Sole, L. Guo, R. Joshi, M. Karczewicz, T. Lin, and S. Wang, “AhG5: Intra block copying with padding”, 15th JCT-VC meeting , JCTVC-O157, Geneva, CH, Oct. 2013.

- David Flynn, Karl Sharman, Chris Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions”, 15th JCT-VC meeting, JCTVC-O1006, Geneva, CH, Oct. 2013.

- J. Sole, E. Alshina, D. -K. Kwon, and W. –H. Peng, “HEVC Range Extensions Core Experiment 3 (RCE 3): Intra block copy refinement”, 15th JCT-VC meeting, JCTVC-O1123, Geneva, CH, Oct. 2013.

# Patent rights declaration(s)

ITRI and NCTU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).