| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0238 |
| --- | --- |

| Title: | Non-RCE3: Block-based padding scheme for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, Hua-Lung Tsai, 2Chun-Lung Lin, 3Fan-Di Jou 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan 4Chun-Chi Chen, Ru-Ling Liao, 5Wen-Hsiao Peng, Hsueh-Ming Hang 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | +886-3-591-7269 1britpablo@itri.org.tw 2chunlung@itri.org.tw 3fandi@itri.org.tw +886-955-028-902 4cheerchen.cs98g@g2.nctu.edu.tw 5wpeng@cs.nctu.edu.tw |
| Source: | ITRI International/ National Chiao Tung University (NCTU), ITRI International |  |  |

_____________________________

# Abstract

This contribution proposes a block-based padding scheme for intra block copy to generate unavailable samples in overlapped region. The block-based padding scheme pads the unavailable region with the block region from the top-left position of reference CU. The experiment results show that the coding gain in MT/lossy coding for SC YUV 444 is -0.9% in AI, -0.7% in RA, and -0.7% in LD, respectively.

# Introduction

At the last meeting in Geneva, padding schemes for intra block copy was proposed [1-2] to generate unavailable samples in overlapped region. This contribution proposes the block-based padding scheme on this subject.

# Proposed schemes

Fig. 1 shows an example of overlapped region (blue region) in intra block copy. The samples of overlapped region are unavailable for intra block copy. Because the intra block copy can predict the overlapped region of current CU by the top-left region of reference CU, the overlapped region of reference CU can be directly padded by copying the top-left region of reference CU as shown in Fig. 2. Therefore, the proposed block-based padding scheme pads the unavailable region with the block region from the top-left position of reference CU. The block-based padding scheme is specifically depicted in Fig. 3. Based on the block vector (BVx, BVy), there are two cases:

If |BVx| < |BVy|: the overlapped region is recursively padded by the horizontal block region from the top-left position of reference CU as shown in Fig. 3(a).

Otherwise: the overlapped region is recursively padded by the vertical block region from the top-left position of reference CU as shown in Fig. 3(b).

Fig. 1. Overlapped region (blue region) in intra block copy

Fig. 2. Padding unavailable region by copying the top-left block region of reference CU

- (b)

Fig. 3. The proposed block-based padding scheme (a) |BVx| < |BVy| (b) |BVx| ≧ |BVy|

# Test results

The coding gain of the proposed method is evaluated on HM12.1+RExt5.1 anchor. The experimental conditions are subject to the common test condition of RCE3 [3]. The following Tables show the coding gain of the proposed schemes in lossy-AI, lossy-RA, lossy-LD and lossless coding configurations, respectively. The coding gain of the proposed scheme for SC YUV 444 is -0.9% in AI-MT, -0.7% in RA-MT, and -0.7% in LB-MT, respectively.

Table 1. BD-rate of the proposed method in lossy-AI coding configuration

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.3% | -0.2% | -0.3% | -0.3% | -0.2% | -0.3% | -0.2% | -0.2% | -0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.9% | -1.0% | -1.0% | -1.0% | -1.1% | -1.0% | -1.0% | -1.1% | -1.0% |
| Animation RGB 444 | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| SC YUV 444 | -0.9% | -0.7% | -0.6% | -0.8% | -0.8% | -0.6% | -0.8% | -0.7% | -0.7% |
| Animation YUV 444 | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -2.6% | -2.7% | -2.6% | -2.8% | -3.1% | -3.2% | -2.8% | -3.0% | -3.1% |
| SC(444) YUV Optional | -1.5% | -1.1% | -1.3% | -1.5% | -1.3% | -1.3% | -1.6% | -1.4% | -1.4% |
| Enc Time[%] | 98% |  |  | 98% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |

Table 2. BD-rate of the proposed method in lossy-RA coding configuration

|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.4% | -0.1% | -0.2% | -0.3% | -0.2% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.8% | -0.8% | -0.7% | -0.8% | -0.8% | -0.8% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.7% | -0.7% | -0.6% | -0.7% | -0.6% | -0.6% |
| Animation YUV 444 | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| RangeExt | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -2.1% | -2.1% | -2.1% | -2.2% | -2.3% | -2.4% |
| SC(444) YUV Optional | -1.5% | -1.4% | -1.4% | -1.7% | -1.4% | -1.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

Table 3. BD-rate of the proposed method in lossy-LD coding configuration

|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | -0.1% | -0.6% | -0.1% | 0.0% | -0.2% | 0.0% |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.7% | -0.6% | -0.7% | -0.8% | -0.8% | -0.8% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| SC YUV 444 | -0.7% | -0.6% | -0.5% | -0.7% | -0.5% | -0.5% |
| Animation YUV 444 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -2.1% | -2.2% | -2.3% | -2.1% | -2.1% | -2.2% |
| SC(444) YUV Optional | -1.8% | -1.6% | -1.6% | -1.3% | -1.3% | -1.3% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 4. Average bit-rate increase of the proposed method in lossless coding configuration

|  | AI | RA | LD |
| --- | --- | --- | --- |
| Class F | -0.1% | -0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.2% | -0.4% | -0.2% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.3% | -0.5% | -0.3% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -1.0% | -1.4% | -0.9% |
| SC(444) YUV Optional | -1.3% | -1.3% | -1.4% |
| Enc Time[%] | 100% | 99% | 100% |
| Dec Time[%] | 100% | 99% | 101% |

# Conclusion

In this contribution, we propose the block-based padding scheme for unavailable samples of intra block copy. The experiment results show that the coding gain for SC YUV 444 is -0.9% in AI-MT, -0.7% in RA-MT, and -0.7% in LB-MT, respectively. It is suggested that the proposed scheme is adopted into the next version of the HEVC Range Extensions text and reference software.

# References

- S. Lee, E. Alshina, C. Kim, “AHG5: Extension of intra block copy,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0112, Oct. 2013.

- C. Pang, J. Sole, L. Guo, R. Joshi, M. Karczewicz, “AHG5: Intra block copying with padding,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0157, Oct. 2013.

- J. Sole, E. Alshina, D.-K. Kwon, W.-H. Peng, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra block copy refinement,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1123, Oct. 2013.

# Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NCTU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).