| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0239 |
| --- | --- |

| Title: | Non-RCE4: Joint proposal of JCTVC-P0231 and JCTVC-P0119: Palette with limited run and lasted coded palette prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou Wei Pu Joel Sole Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA Guillaume Laroche Tangi Poirier Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +1-858-845-1115 fzou@qti.qualcomm.com +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Qualcomm Incorporated & Canon Research Centre France |  |  |

_____________________________

# Abstract

RCE4 Test 2 evaluated a palette coding tool. This proposal combines Non-CE4 Test 2 related proposals, including JCTVC-P0231 and JCTVC-P0119. The joint proposal evaluates the joint performance of these two. Simulation results demonstrate that the coding gain is additive and thus 16.9% for Intra RGB SC 4:4:4, YCbCr 4:4:4 SC 12.2% are achieved.

# Introduction

A palette-based coding method [1] was presented in 15-th JCT-VC meeting and was included as test 2 in RCE4 for further discussion in San Jose meeting [2, 3].

The palette desgin is divided in two stages. In the first one, for a CU, the palette is derived and signalled. In the second stage, pixels in the CU are coded using one of the three candidate modes:

- “Run mode”

- “Copy above mode”

- “Pixel mode” (escape mode)

# Method Description (JCTVC-P0231)

## Skip the Index Coding when Palette has One Entry

When the palette has only one item, i.e. there is only one color in the CU, there is no necessity to code further syntax elements during the second stage. The information can be implicitly derived by the decoder. Specifically, when the decoder sees a palette coded CU with only one entry in the palette, all the pixels in the CU have the same value as specified in the palette.

## Skip the Coding of Escape Flag

In [3], during the second stage, the encoding logic can be expressed using the following pseudo code:

for (each pixel x in the CU)

{

Code a flag indicating whether x is escape;

if (x is escape)

{

code x with ‘pixel mode’;

}

else

{

code x with ‘run mode’ or ‘copy above mode’;

}

}

However, if the palette size is less than the predefined maximum palette size (default=32 in RCE4 Test 2), then no escape pixel can be present, since if an escape pixel still exists, it should have already been added into the palette.

Based on this fact, if the palette size is less than 32, coding the escape indication flag is avoided.

## Escape Pixel Coding in ‘Pixel Mode’

### Increase CABAC Throughput

In [3], to code ‘pixel mode’, fixed length coding is used for each color component with one CABAC context for each bit-plane. To increase the entropy coding throughput, only the MSB is context-coded using a single context while the other bits are coded using bypass CABAC coding. This refinement also reduces the number of contexts for the palette method.

### Removal of the Escape Pixel Prediction

To reduce the coding complexity and to make the syntax cleaner, escape pixel prediction is disabled in ‘pixel mode’. The quantized escape pixel value is directly coded using the fixed length coding described above.

### Reducing the Fixed Length Coding Bit-Depth

In [3], to code an escape pixel, each component is fixed length coded using input bit-depth bits. For lossy coding, due to quantization and depending on the quantization step size, the necessary bit-depth to represent a quantized pixel value might be less than the input bit-depth. This fact is used to reduce the number of signalled bits.

# Method Description (JCTVC-P0119)

## The limit_run method is enabled

The method consists in indicating the maximum palette index value (Limit_run) for which the run is coded. Thus, for all palette index values superior to this Limit_run, the run is not coded.

The syntax element Limit_run is decoded just after the palette_size_minus1 for all Palette CU. It is coded with a unary max code (CABAC bypass) with the maximum value equal to the Palette_size.

On encoder side, the Limit_run is determined a posteriori. It means that, during the selection between the Run mode and the Copy above mode, the rates of using or not the proposed method are estimated. At the end of this process, the best Limit_run value is determined and the syntax of the current palette CU is updated.

## Palette prediction with lasted coded palette

In order to limit the impact of the palette prediction on the memory needed to store the palette predictor, only the palette of the last decoded Palette CU is used as the predictor. This does not modify the worst case as only the maximum number of elements in a palette need to be stored with the size of this last palette. In addition, to offer palette estimation in parallel at CTB line level at encoder side, the palette is only reset at the beginning of each CTB line.

# Experimental results

The proposed method is implemented on HM-12.1+RExt-5.1. RCE4 common test conditions specified in [4] are used. Encoding time and decoding time are measured on a homogenous cluster.

Table 1 Summary of AHG 8 Lossless Coding Results

[will be updated soon]

Table 2 Summary of AHG8 Lossy Coding Results

[Partial results, will be updated soon]

# References

- L. Guo, M. Karczewicz, J. Sole, and R. Joshi, “Evaluation of palette mode coding on HM-12.0+RExt-4.1,” JCTVC-O0218, Geneva, Switzerland, Oct. 2013.

- L. Guo, X. Guo, and A. Saxena, “HEVC Range Extensions Core Experiment 4 (RCE 4): Palette Coding For Screen Content,” JCTVC-O1124, Geneva, Switzerland, Oct. 2013.

- L. Guo, W. Pu, M. Karczewicz, J. Sole, R. Joshi, and F. Zou, “RCE4: Results of test 2 on palette mode for screen content coding,” JCTVC-P0198, San José, US, Jan. 2014.

- D. Flynn, C. Rosewarne, K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-O106, Oct. 2013

# Patent rights declaration(s)

Qualcomm Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).