| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0245 |
| --- | --- |

| Title: | RCE1: Cross-verification of subtest B.7 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim Andrew Segall | Email: | kimse@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP. |  |  |

_____________________________

# Abstract

This contribution provides a cross-check of RCE1 test B7 by Qualcomm.  In test B7, The coding of coded_sub_block_flag, significance_coeff_flag, coeff_abs_level_greater1 and coeff_abs_level_greater2 flags is skipped for the current transform block based on the given threshold (4), and coeff_sign_flag and  coeff_abs_level_remaining syntax element was coded after the last significant coefficient position coding.  The cross check result confirms the simulation result reported by Qualcomm.

# Cross check procedure

This is a cross-check of RCE1 test B7 in proposal JCTVC-P0074 [1]. The source code was provided by Qualcomm. Sharp compiled the source code and the simulation was performed under the AHG18 conditions, as defined in the RCE description [2]. Note that timing results are not comparable due to differences in the simulation environment but leave the timing result as observed in [1].

# Analysis

A combination of subtest A3 with the alignment of ivlCurrRange to 256 is represented based on the condition that the Golomb Rice parameter at the end of the previous transform block is greater than or equal to a threshold (4).

- The coding of coded_sub_block_flag, significance_coeff_flag, coeff_abs_level_greater1 and coeff_abs_level_greater2 flags is skipped for the current transform block.

- The last significant coefficient position is coded in bypass mode as (blockSizeX – LastSignificantCoeffX) and (blockSizeY – LastSignificantCoeffY).

- For the current transform block, ivlCurrRange is aligned to 256 before the coding of the last significant coefficient position. In this case the coding pass for coeff_sign_flag comes after the coding pass for the coeff_abs_level_remaining syntax element. The coeff_abs_level_remaining syntax element is adjusted to account for the fact that significance_coeff_flag, coeff_abs_level_greater1 and coeff_abs_level_greater2 flags are not coded.

# Coding performance measurement

Table 1 provides the cross-check results for JCTVC-P0074 test B5 under AHG18 conditions.

Table 1. JCTVC-P0074 AHG18 simulation result.

# Conclusion

This cross-check includes results for RCE1 Test B7 provided by Qualcomm. The provided software matches the description in the contribution. The cross-check result confirms the simulation result reported by the proponent.

# References

- R. Joshi, K. Sharman “HEVC Range Extensions Core Experiment 1 (RCE1): High bit rate coding at high bit depths,” JCTVC-O1121, 15th JCT-VC meeting, Geneva, CH, Oct. 2013.

- R. Joshi, J. Sole, M. Karczewicz, “RCE1: Results for subtests B5, B6, B7,” JCTVC-P0074, 16th JCT-VC Meeting: San Jose, US, Jan 2014.