| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0247 |
| --- | --- |

| Title: | RCE1: Cross-Check of subtest A.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jing Ye Shan Liu | Email: | jing.ye@mediatek.com shan.liu@mediatek.com |
| Source: | Mediatek. Inc |  |  |

_____________________________

# Abstract

This contribution reports the cross-verification results of the subtest A.1(JCTVC P0073) from RCE1. In this method, all the coding of of coeff_abs_level_greater1 and coeff_abs_level_greater2 flags are skipped for a 4x4 subblock if the Golomb Rice parameter at the end of the previous 4x4 subblock is greater than or equal to a threshold (4). Also the coeff_abs_level_remaining syntax element is adjusted to account for the fact that coeff_abs_level_greater1 and coeff_abs_level_greater2 flags are skipped. The software implementation of this method is the same to the document description. AHG 18 test results are matched that provided by the proponent.

# Introduction

This contribution reports the cross-verification results of the subtest A.1 from RCE1. In this method , all the coding of of coeff_abs_level_greater1 and coeff_abs_level_greater2 flags are skipped for a 4x4 subblock if the Rice parameter value at the end of the previous 4×4 subblock is equal or greater than a threshold. Mediatek compiled the source code and did the simulation on a linux cluster under the common test condition defined for RCE1 [2].

# Test results

The cross-verification results of the subtest A.1 from RCE1 are summarized in Table 1, 2, and 3. It is confirmed that AHG18 test results match those provided by the proponent. AHG5 and AHG8 lossless test results were not available when this report was uploaded due to the late release of the test results from the proponent. They will be updated as soon as the results are available.

Table 1. Cross-check test results of RCE1 subtest A.2 in AHG5 CTC

Table 2. Cross-check test results of RCE1 subtest A.2 in AHG8 CTC (lossless)

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | 0.3% | 0.2% | 0.2% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 0.2% | 0.2% | 0.1% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | 0.1% | 0.1% | 0.0% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | 0.4% | 0.4% | 0.4% |
| YCbCr 4:4:4 SC (Optional) | 0.6% | 0.3% | 0.4% |
| Enc Time[%] | 117% | 1154% | 105% |
| Dec Time[%] | 115% | 108% | 110% |

Table 3. Cross-check test results of RCE1 subtest A.2 in AHG18 CTC

# Conclusion

This contribution reports the cross-verification results of the subtest A.1 from RCE1. The coding performance is independently evaluated by Mediatek and it is confirmed that AHG18, AHG5 and AHG8 test results match those provided by the proponent.

# References

- R. Joshi, K. Sharman, “HEVC Range Extensions Core Experiment 1 (RCE1): High bit rate coding at high bit depths,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1121, Oct. 2013

- R. Joshi, J. Sole, M. Karczewicz, “AHG5 and AHG18: Bypass of the significance and coefficient level flags for higher throughput,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0209, Oct. 2013.

- R. Joshi, J. Sole, M. Karczewicz, “AhG5 and AhG18: Bypass of the coefficient level flags for higher throughput,” Document of Joint Collaborative Team on Video Coding, JCTVC-O0208, Oct. 2013.