| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0249 |
| --- | --- |

| Title: | Non-RCE4: A combination of the four-neighbor major color index prediction in JCTVC-P0098 and a simplified transition copy mode from JCTVC-P0115 on top of RCE4 Test1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun, Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yc.sun, peter.chuang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this proposal, a combination of the four-neighbor major color index prediction in JCTVC-P0098 and the transition copy mode in JCTVC-P0115 is shown on top of RCE4 Test1 (JCTVC-P0108). The transition copy mode is simplified and added into the major color index candidate list. Simulation results show that the proposed method achieves 4.1%, x.x%, and x.x% BD-rate savings compared with RCE4 Test1 for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

# Introduction

In RCE4 Test1 (JCTVC-P0108) [1], major color indices of the left sample and the above sample can be used to predict that of the current sample. In JCTVC-P0098 [2], four neighboring samples including the left, above, above-left, and above-right samples are included into a candidate list of major color indices for the current sample. In JCTVC-P0115 [3], a transition copy mode is proposed. A causal area is searched to find a matching sample with the left sample of the current sample, and the right sample of the matching sample is used to predict the current sample. Multiple matching sample candidates in the causal area can be considered.

# Proposed method

In this contribution, we attempt to combine JCTVC-P0098 and JCTVC-P0115. In the candidate list of major color indices as used in JCTVC-P0098, the transition copy candidate as used in JCTVC-P0115 is added after the above candidate and the left candidate and followed by the above-left candidate, the above-right candidate, and no prediction. Next, pruning is performed to remove redundancy. Finally, a candidate index is signaled. Although the adaptive candidate list size (method-1) in JCTVC-P0098 is used in the experiments of this proposal, the fixed candidate list size (method-2) can be easily used as well.

Complexity reduction of the transition copy candidate is proposed as follows. For each coded major color index of a sample, its coded major color index of the next sample is recorded in a table and treated as the transition copy candidate. When coding the current sample, the major color index of the left sample is used as the table input to find the transition copy candidate quickly. This design is equivalent to always selecting the latest matching sample in the causal area as the transition copy candidate, where the causal area covers all coded samples in the CU. It is not required to search the matching sample or access the buffer of major color indices of coded samples.

# Experimental results

The proposed method is implemented on top of RCE4 Test1. The tests are conducted under the common test conditions in RCE4 [4]. Table 1 shows the results of the proposed method using HM-12.1+RExt-5.1 [5] as the anchor. Table 2 shows the results of RCE4 Test1 using the same anchor as Table 1.

For comparison, we change the anchor of Table 1 to RCE4 Test1, and the results are shown in Table 3. Compared with RCE4 Test1, 4.1%, x.x%, and x.x% BD-rate savings are achieved for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

# Conclusion

In this proposal, a combination of JCTVC-P0098 and JCTVC-P0115 is shown. It is reported that the proposed method achieves 4.1%, x.x%, and x.x% BD-rate savings compared with RCE4 Test1 for SC YUV 444 sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- X. Guo, Y. Lu, S. Li, “RCE4: Test1. Major-color-based screen content coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0108, Jan. 2014.

- T.-D. Chuang, Y.-C. Sun, Y.-W. Chen, Y.-W. Huang, S. Lei, “Non-RCE4: Four-neighbor major color index prediction,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0098, Jan. 2014.

- C. Gisquet, G. Laroche, P. Onno, “Non-RCE4: Transition copy mode for palette mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-P0115, Jan. 2014.

- L. Guo, X. Guo, A. Saxena, “HEVC Range Extensions Core Experiment 4 (RCE 4): Palette coding for screen content,” Document of Joint Collaborative Team on Video Coding, JCTVC-O1124, Oct. 2013.

- HM-12.1+RExt-5.1, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/

Table 1.  Results of the proposed method; anchor: HM-12.1+RExt-5.1

Table 2.  Results of RCE4 Test1; anchor: HM-12.1+RExt-5.1

Table 3.  Results of the proposed method; anchor: RCE4 Test1